3GPP TSG-RAN WG4 Meeting # 114bis	                                              R4-2503606

Wuhan, CN, April 7 – 11, 2025

Agenda item:	7.19.3

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In this contribution, we provide our views on core requirements of AI/ML BM use case.

- Discussion

## 2.1 Whether Rx beam information is known for predicted beams

In the past few meetings, RAN4 discussed whether Rx beam information is known for predicted beams. We would like to provide our view here.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

As captured in TR38.843 [1], in R18 SI, both DL Tx beam prediction and beam pair prediction are studied.

TR38.843 For both sub-use cases, the following alternatives are studied for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction (deprioritized) - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) Using BM-case1 as an example:

In R19 WI, as pointed out in WID, only DL Tx beam prediction is in scope and beam pair prediction is not in the scope.

RP-243244 - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

In the next, we will focus on DL Tx beam prediction at first.

Take BM-case 1 as an example. Two alternatives had been studied:

- Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

- Alt. ii): Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

In Alt. i), if CSI-RS to be predicted is QCL Type-D to SSB in Set B, then Rx beam of the associated SSB can be applied to CSI-RS Tx beam. In this scenario, Rx beam of the predicted Tx beam can be known.

In R18 SI, the following options were evaluated for DL Tx beam prediction. It is possible to use a fixed Rx beam for all the beams at inference and Rx beam of the predicted beams can be assumed to be the fixed Rx beam. Also as argued last meeting, it is UE implementation whether to use a fixed Rx beam for inference or use Rx beam related information as model input.

TR38.843 For the purpose of DL Tx beam prediction evaluations, consider the following options for Rx beam as AI/ML model input for training and/or inference if applicable: - Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample. - Companies expected to report how to select the "best" Rx beam(s). - Option 2: Measurements of specific Rx beam(s). - Companies expected to report how to select specific Rx beam(s). - Option 3: Measurements of random Rx beam(s) per model input sample. - Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. Companies expected to report the measurement/RS overhead together with the beam prediction accuracy, as well as, how to find the quasi-optimal Rx beam with "previous measurement". - Other options are not precluded and can be reported by companies.

Proposal 1: Rx beam of the predicted Tx beam which is not measured can be known if the beam is QCL Type-D to a Tx beam in Set B.

Proposal 2: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

      - 2.2 Scenarios to define requirements for?

We had done some simulations with different Set B patterns when Set B is a subset of Set A. In the simulation, there are 32 Tx beams with 2 elevation angles and 16 azimuth angles in Set A and 8 Tx beams in Set B. We tried different set B patterns (e.g., in pattern 0, Set B = the first 8 beams in Set A) and got the Top 5/1 prediction accuracy requirements as in Fig. 1. It can be observed that the top 5/1 prediction accuracy of different patterns are varied. The gap is more than 10% between the best and worst.

Fig. 1

Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Usually, when we define accuracy requirements in RAN4, the requirements defined is scenario agnostic. Take L1-RSRP measurement accuracy as an example. The requirements are applicable to all the channel models, all UE speeds, all BS radiation patterns and etc. But for AI/ML BM models, it seems not possible to define a scenario-agnostic performance requirements based on the above observations. In R18 MIMO, we faced similar issue when discussing how to define requirements for TDCP. At that time, due to the requirements vary with different UE speeds, different serving frequency as well as different channels, RAN4 decided to define the test cases for limited cases (i.e., under certain doppler shift and channel model) instead of defining scenario-agnostic accuracy requirements. So we propose to discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 3: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

As mentioned above, for each BM sub-case, there are different scenarios. If it is not possible to define scenario-agnostic performance requirements, we shall discuss which scenarios to define requirements and test cases for. Take BM case 1 as an example. There are several candidate scenarios:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: RSRP with specific Rx beam

- Scenario 2: Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

  - Scenario 2a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2b: RSRP with specific Rx beam

As shown by the evaluation results in TR38.843, there is some performance loss compared to best Rx beams obtained from one shot measurements using a specific or sub-optimal Rx beam for DL Tx beam prediction. In addition, there are several ways to choose the specific Rx beam. We prefer to define requirements only for Scenario 2a which is the most basic one.

Evaluation results in TR38.843: For BM-Case 1 DL Tx beam prediction without UE rotation, for Top-1 beam prediction accuracy, compared to the best Rx beams obtained from one shot measurements, with quasi-optimal Rx beam performance degradation is observed: - evaluation results from 1 source show 2% beam prediction accuracy degradation when Set B = 1/2 Set A and 7% beam prediction accuracy improvement when Set B = 1/4 or 1/8 Set A, when using the best Rx beams obtained from previous exhaustive sweeping (20ms ago) of all beams in Set A, comparing with using the best Rx beam for each Tx beams in Set B obtained from current exhaustive sweeping, without considering UE rotation for 3km/h UE speed. Such beam prediction accuracy improvement may not exist when considering UE rotation and higher UE speed. - evaluation results from 1 source show 2.5% beam prediction accuracy degradation using the best Rx of each Tx beams obtained from previous exhaustive sweeping (20ms ago) than using the best Rx of each Tx beams obtained from current exhaustive sweeping, without considering UE rotation for 3km/h UE speed. - evaluation results from 1 source shows 6.6%/6.9%/32.1%/45% degradation using a stochastic model in which the UE Rx beam is randomly selected with average probability that the best Rx beam is selected equal to 87.1%/75.1%/34.3%/10.9% compared to using the best Rx of each Tx beams obtained from current exhaustive sweeping, without considering UE rotation - evaluation results from 1 source show 13% beam prediction accuracy degradation, with the assumption of the best Rx beam for each Tx beam obtained from previous exhaustive sweeping over all beams in Set A in a SSB-like structure (in the past 160ms for each Rx beam with every 20ms a burst of Set A of beams) without considering UE rotation for 3km/h UE speed. - evaluation results from 1 source show 3%~11% beam prediction accuracy degradation, with the assumption of the best Rx beam obtained from one specific Tx beam which is 1st Tx beam in Set B. - evaluation results from 1 source show 12% beam prediction accuracy degradation with the assumption of the best Rx beams obtained from one specific Rx beam which is the best between the same Rx beam for different panels. - In addition, evaluation results from 3 sources show 1%~4% and 6%~12% beam prediction accuracy degradation, with the assumption of the best Rx beam is used for 90% and 80% of the model input samples and random Rx beam for the remaining samples respectively. - Even though, AI/ML can still provide better performance than non-AI baseline option 2 (exhaustive beam sweeping in Set B of beams), e.g., 50%~60% beam prediction accuracy difference in terms of Top-1 beam prediction accuracy based on the evaluation results from 2 sources, where non-AI baseline option 1 (exhaustive beam sweeping in Set A of beams) provides 100% prediction accuracy.

Also it is up to UE implementation whether to use beam pair prediction models. However, as beam pair prediction is not in the WI scope, we propose not to define separate requirements or test cases for beam pair prediction.

Proposal 4: Further discuss which scenario among the following to define requirements and test cases for:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: RSRP with specific Rx beam

- Scenario 2: Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

  - Scenario 2a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2b: RSRP with specific Rx beam

Proposal 5: Not to define separate requirements or test cases for beam pair prediction in R19 WI.

      - 2.3 Impact of measurement error

2.3.1 General analysis

Last meeting, the simulation settings were further discussed to evaluate the impact of measurement error in R4-2503021. In our simulations, Option 2 in R4-2503021 is adopted to get the distribution of BB error under different SNRs, i.e., TDL-C channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After tens of simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the mean and variance of BB measurement error under SNR= [-24, 10] dB with step 3dB at first (as in Fig. 2). In our previous simulation, due to lack of enough simulations under low SNR, the distribution of BB measurement error in our last meeting’s Tdoc R4-2417893 is not correct.

Fig.2

During inference, the way to add measurement error is shown in Fig. 3:

- In SLS simulation, SINR of each Tx-Rx beam pair is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 10dB which is as guided in TR38.843 Table 6.3.1-1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam pair, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

Fig. 3 Method of adding measurement error

We simulated the performance without RF error of BM-case 1 with classification model at first. The simulation results are provided as in Table 1. We simulated several cases to study the impact of measurement error. Considering that it is possible to train a model without measurement in the simulation but not possible in the real field, we simulated both case 2/3 (train without measurement error) and case 4/5 (train with measurement error). Also considering it is possible to get the genie aided ground truth without measurement error in simulation but not possible in the real field, we simulated both case 2/4 (genie aided ground truth without measurement error) and case 3/5 (ground truth with measurement error). It can be observed that the performance degraded the worst in case 3, train without error and inference with measurement error in both set B and set A. Although Case 5 (train with measurement error) is more aligned with how to train a model in the real field, it is ok to define requirements based on Case 3 considering how to train a model is totally up to UE implementation and RAN4 defines the minimum requirements.

Table 1 Simulation results for BM-case 1 with classification model – impact of measurement error

|  | Train with Error | Inference with Error | Input with error | Ground truth with error | BB error assumption | top-1/1 beam prediction accuracy (%) | top-3/1 beam prediction accuracy (%) | top-5/1 beam prediction accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | No | No | No | No | N.A. | 72.28 | 94.76 | 98.15 |
| Case 2 | No | Yes | Yes | No | Gaussian distribution with mean and variance corresponding to the calculated SINR | 52.75 | 81.11 | 90.11 |
| Case 3 | No | Yes | Yes | Yes |  | 45.56 | 71.8 | 81.27 |
| Case 4 | Yes | Yes | Yes | No |  | 55.19 | 83.12 | 92.7 |
| Case 5 | Yes | Yes | Yes | Yes |  | 50.58 | 76.83 | 84.87 |
| Note: In this set of simulations, there are 80% indoor UEs and 20% outdoor UEs. |  |  |  |  |  |  |  |  |

As shown in 2.5.2.3, the upper bound of RSRP during the test is about -83.5dB. In the test, the impact of the upper bound should be considered. We have done some simulation and the results are shown in Table 2. It can be observed that filtering out the data exceeds TE’s maximum Tx power has little impact on the prediction accuracy no matter considering measurement error or not. However, the performance degrades little when all the UEs are outdoor UEs if measurement error is considered while the performance degrades significantly when most are indoor UEs. After checking RSRP distribution as shown in Fig. 4 and Fig. 5, it can be observed that when most of the UEs are indoor UEs, the percentage of RSRP lower than -120dB is much higher than 100% outdoor UEs. Low RSRP kind of leads to larger measurement error.

Table 2 Simulation results for BM-case 1 with classification model – impact of TE dynamic range and measurement error

|  | Train with Error | Inference with Error | Train: upper bounded by -83.5dBm | Inference: upper bounded by -83.5dBm | top-1/1 beam prediction accuracy (%) | top-3/1 beam prediction accuracy (%) | top-5/1 beam prediction accuracy (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80% indoor, 20% outdoor | No | No | No | No | 72.28 | 94.76 | 98.15 |
|  | No | No | No | Yes | 72.46 | 94.87 | 98.25 |
|  | No | No | Yes | Yes | 72.1 | 94.81 | 97.95 |
|  | Yes | Yes | No | No | 50.58 | 76.83 | 84.87 |
|  | Yes | Yes | Yes | Yes | 47.77 | 74.34 | 82.79 |
| 100% outdoor | No | No | No | No | 76.56 | 95.82 | 98.15 |
|  | No | No | No | Yes | 72.93 | 94.55 | 97.76 |
|  | No | No | Yes | Yes | 71.67 | 93.38 | 97.08 |
|  | Yes | Yes | No | No | 71.85 | 93.92 | 97.2 |
|  | Yes | Yes | Yes | Yes | 65.53 | 90.07 | 96.79 |
| Upper bounded by -83.5dBm: The group of data is filtered out if RSRP of any Tx beam in set B is above -83.5dB. |  |  |  |  |  |  |  |

Fig. 4 RSRP distribution with 80% indoor+20% outdoor UEs

Fig. 5 RSRP distribution with 100% outdoor UEs

Observation 3: The performance degrades a bit when all the UEs are outdoor UEs if measurement error is considered while the performance degrades significantly when most are indoor UEs.

Observation 4: Filtering out the data exceeds TE’s maximum Tx power has little impact on the prediction accuracy.

Proposal 6: Discuss which UE distribution to be used in the test and to be defined requirements for, e.g., 80% indoor+20% outdoor UEs or 100% outdoor UEs.

Proposal 7: When defining the accuracy requirements for AI/ML BM models, the impact of measurement error should be considered.

Proposal 8: When defining the accuracy requirements for AI/ML BM models, the impact of the dynamic range of TE should be considered.

2.3.2 Impact on different Metrics

As guided in R4-2503021, we focus on the scenario with 100% outdoor UEs to check the performance of different metrics. The simulation results are summarized as in Table 3. To check the predicted RSRP accuracy, regression model is adopted in the simulations.

Table 3

| KPI |  |  |  | ideal | BB error only | BB error + RF error (Option 1) |
| --- | --- | --- | --- | --- | --- | --- |
| parameter | K | N | X |  |  |  |
| baseline: top-K/1 accuracy | 1 |  |  | 67.72 | 64.81 | 64.07 |
|  | 2 |  |  | 83.76 | 81.75 | 80.21 |
|  | 3 |  |  | 90.26 | 88.99 | 88.1 |
|  | 4 |  |  | 93.65 | 92.54 | 91.43 |
|  | 5 |  |  | 95.5 | 94.39 | 93.92 |
| top-K/N, Top-N genie-aided beam is one of the Top-K predicted beams (%) | 2 | 2 |  | 92.8 | 91.48 | 90.32 |
|  | 2 | 4 |  | 97.41 | 96.72 | 95.93 |
|  | 3 | 4 |  | 98.68 | 98.36 | 98.04 |
|  | 4 | 4 |  | 99.15 | 98.84 | 98.84 |
|  | 5 | 2 |  | 98.2 | 97.88 | 97.25 |
| top K/1 with XdB margin (KPI 1-2 Alt2) (%) | 1 |  | 1 | 74.44 | 71.8 | 69.52 |
|  | 1 |  | 2 | 79.84 | 76.83 | 75.56 |
|  | 1 |  | 3 | 83.81 | 81.32 | 79.79 |
|  | 2 |  | 1 | 87.46 | 85.77 | 83.92 |
|  | 3 |  | 1 | 93.02 | 91.43 | 90.48 |
| 90%-tile RSRP difference (between maximum RSRP of the Top-K predicted beam(s) and the ideal RSRP of the genie aided strongest beam) (dB) | 1 |  |  | 5.92 | 6.72 | 7.83 |
|  | 3 |  |  | 0 | 0 | 0 |
|  | 5 |  |  | 0 | 0 | 0 |
| 90%-tile predicted RSRP accuracy (predicted RSRP of beam i - ideal RSRP of beam i) (dB) | 1 |  |  | 6.69 | 7.54 | 7.87 |
|  | 3 |  |  | 6.83 | 8.17 | 8.42 |
|  | 5 |  |  | 6.57 | 8.25 | 8.42 |
| 90%-tile RSRP difference (between maximum RSRP of the Top-K predicted beam(s) and the measured RSRP of the strongest beam) (dB) | 1 |  |  |  | 6.95 | 7.78 |
|  | 3 |  |  |  | 0.33 | 0.78 |
|  | 5 |  |  |  | 0 | 0 |
| 90%-tile predicted RSRP accuracy (predicted RSRP of beam i - measured RSRP of beam i) (dB) | 1 |  |  |  | 7.46 | 7.51 |
|  | 3 |  |  |  | 7.58 | 7.88 |
|  | 5 |  |  |  | 7.46 | 7.84 |

In TR38.843, we have three candidate metrics for evaluating the performance of AI/ML BM model. After several meetings discussion, it is still difficult to converge. The progress in last meeting is also copied here for information.

In TR38.843 - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Progress in RAN4#114 Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

In our view, one of the principles to choose the metrics is that UE will report corresponding results or the metric can be derived through the reported results. Another principle is that the metric is meaningful for NW to make the decision on next behaviors.

Predicted RSRP can be reported according to RAN1 agreement. We think it is valuable for NW to make a decision on switching the TCI state or not. Maybe the serving beam is not the best, but it is only 1dB or 2dB lower than the best beam. In this case, it is reasonable not to switch the TCI state to the best beam and RSRP value is needed for NW. According to the simulation results shown in Table 3, taken measurement error into consideration, 90%-tile of predicted RSRP accuracy (predicted RSRP of beam i - ideal RSRP of beam i) is less than 8.5dB which is about 2dB worse than legacy measurement RSRP accuracy requirements.

In our understanding, the reason to consider Top K/1 (K≠1), Top K/N and Top K/1 with margin is that the prediction accuracy of Top 1 is too low. NW may not switch the beam to the predicted strongest one just based on the predicted results. Then the metric Top K/1 seems meaningful. After UE reports the predicted Top K beam indexes, NW may let UE measure these beams and then decide the beam to switch to based on the measurement results.

The reason behind to propose the metrics Top K/1 with margin is the difference among the ground truth of the genie-aided Top K beams may be marginal. Even the predicted Top 1 is not the genie-aided best beam, while it is only X dB lower than the best beam. The predicted Top 1 is still good enough. It can be observed in Table 3 that the prediction accuracy of Top 1 with 3dB margin is around 80%. To be honest, we are not sure whether it is accurate enough for NW to make the decision solely on the reported predicted Top 1.  If it is not accurate enough, we may just use the metric Top K/1.

For the metric Top K/N, we are not quite clear about the expected NW behavior after receiving the predicted results.

In our view, if there is a metric that is accurate enough and can avoid further measurement on the predicted Top K beams, we will be supportive of such a metric. But it seems either Top K/N or Top 1 with margin can achieve this goal. Given the status, we prefer to use the simplest metric Top K/1 and RSRP accuracy as the metric to evaluate the model performance.

Proposal 9: If RSRP and beam ID are reported, use RSRP accuracy and Top-K/1 (%) “the percentage of the Top-1 strongest beam is one of the Top-K predicted beams" as the KPI. If only beam ID is reported, use Top-K/1 (%) “the percentage of the Top-1 strongest beam is one of the Top-K predicted beams" as the KPI.

      - 2.4 LCM of AI/ML BM models

RAN1 had some discussion on the signalling procedure on applicable functionality reporting for beam management UE-sided model based on the RAN2 input in LS R2-2407848. RAN2 discussed the procedure further based on RAN1 reply LS. The progress is copied here for information.

Agreement in RAN1#119 - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement in RAN2#129 - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - The provided periodic CSI configuration should be consistent with reported UE capabilities - FFS option B - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

According to the above RAN1/2 agreements, for different reporting types, the occasions that UE activates the corresponding AI/ML BM model are different, i.e.,

- Periodic report: UE activates the model before step 4.

- Semi-persistent reporting: UE activates the model after receiving the MAC CE that activates semi-persistent reporting.

- Aperiodic CSI reporting: UE activates the model after receiving the DCI that activates aperiodic reporting.

RAN4#113 discussed how to define the prediction delay and reached the following agreement. According to the above  RAN1/2 agreements, the activation delay of AI/ML BM model shall also be counted into the prediction delay for aperiodic reporting and the first report of semi-persistent reporting.

RAN4#113 Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

Proposal 10: Activation delay of AI/ML BM model shall also be counted into the prediction delay for aperiodic reporting and the first report of semi-persistent reporting, FFS the value.

      - 2.5 Test setup for AI/ML BM

2.5.1 Testing goal

In R18 SI, the following principle was identified: The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field (in TR38.843).

In General part, RAN4 discussed the testing goals of this WI and reached the following agreement in RAN4#112.

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

As observed in the simulation results in Fig.1, in terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference. RAN4 once agreed to use synthetic channels in R19 AI/ML test. Now we wonder whether the simulation results under synthetic channels can reflect the performance of AI/ML models in the real field. If not, we think it may be necessary to design a test system that is possible to use field data in the test. The key issue to use field data in the test is how to get the test dataset. We understand that it is not easy to figure this out.

Considering the potential fact that the performance under synthetic channels may not reflect the performance of AI/ML models in the real field and the difficulty to use field data in the test, we think at least for AI/ML BM, RAN4 needs to discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 11: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 12: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 13: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

2.5.2 Test set up

2.5.2.1 Single AoA test system

For DL Tx beam prediction, the model inputs are L1-RSRP of Tx beams in Set B with Tx beam ID information as input or implicitly indicated, no matter RSRP of Tx beams in Set B is the one obtained from exhaustive sweeping or a specific fixed Rx beam. It is only the final L1-RSRP value that matters. All the spatial characteristics will be reflected on RSRP. Instead of emulating CDL channels and [32-128] Tx beams through multiple AoAs and AoDs, the spatial-selective and time-varying propagation conditions can be finally reflected on received RSRP at the test UE through adjusting Tx power at TE on each beam at different time. This is also the method which is widely used in legacy RAN4 RRM tests.

Proposal 14: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. In the next, we would like to explain that using real field data is also possible with single AoA test system.

As shown in Fig.6, in the data collected from real field, the measured RSRP (at baseband, including Rx beamforming gain) of Tx#1 and Tx#2 are P1 and P2, respectively. Using single AoA test system, with Rx beam lock, the Rx beamforming gain during the test can be fixed, assuming it is G. If the transmit RSRP at TE for Tx#1 and Tx#2 is P1-G and P2-G, then the received RSRP at UE baseband will be P1 for Tx#1 and P2 for Tx#2 which are exactly the same as the measured RSRP in real field. The Rx beamforming gain G can be derived by letting TE transmits a fixed high power P0, G is equal to P0’-P0 where P0’ is the RSRP measured at UE. This can be done multiple times to derive an average G to reduce the impact of noise.

Fig. 6

If to use field data, TE transmission power for each Tx beam in set B can be determined as shown in Fig.7.

Fig. 7

If to use synthetic channels defined in 38.901, BS/UE antenna configuration & radiation pattern defined in 38.901 can be used to determine TE transmission power for each Tx beam in set B (as shown in Fig.8).

Fig. 8

In computer simulations, it is possible to get RSRP of each Tx beam under a specific gNB settings (Tx spatial radiation pattern, BS height, downtilt angle and etc.), a specific channel (e.g., UMi) and a specific UE radiation pattern (defined in 38.901) at a random UE location with random UE rotation. Though computer simulation, it is possible to get a set of RSRP values, as shown in Table 4. Take Data 1 as an example, if the transmission power at the probe is RSRP#B1-1 for Tx beam B1, the channel between the probe and UE is purely AWGN channel, and UE beamforming gain is 1, then the measured RSRP of Tx beam B1 at UE is RSRP#B1-1. As the channel between the probe and UE may be not purely AWGN channel and UE beamforming gain is not 1, the real transmission power at the TE shall be calibrated.

Table 4

| Data set | Set B | Set A |
| --- | --- | --- |
| Data 1 | RSRP#B1-1, …,RSRP#BM-1 | RSRP#A1-1, …,RSRP#AN-1 |
| Data 2 | RSRP#B1-2, …,RSRP#BM-2 | RSRP#A1-2, …,RSRP#AN-2 |
| …… |  |  |
| Data L | RSRP#B1-L, …,RSRP#BM-L | RSRP#A1-2, …,RSRP#AN-L |

As the data set can be easily obtained through simulation for varied scenarios and settings, we think single AoA test system can be easily extended to different scenarios and settings.

It is observed that one of the concerns on single AoA test system is that using UE’s radiation pattern to get the inference dataset at TE will lead to high complexity. We don’t think it is necessary to use the test UE’s radiation pattern. We evaluated the performance of models trained with a UE antenna radiation pattern (case 2) and inference on UEs with varied antenna radiation patterns (case 1, 3, 4) as shown in Fig. 9. It can be observed that AI/ML BM Tx beam prediction model generalize well for different antenna radiation patterns. Therefore, we think it is enough to use a specified UE radiation pattern during the test.

Fig. 9

Observation 5: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

For set A, TE can get the generated RSRP value for each beam as shown in Table 4. Then after some calibration, we think TE can also know the expected RSRP of Tx beams at UE baseband, i.e., roughly transmitted RSRP at TE + fixed Rx beamforming gain.

Observation 6: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

With single AoA, existing IFF and enhance IFF test systems can be re-used for AI/ML BM test.

Proposal 15: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models.

Proposal 16: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system instead of using test UE’s radiation pattern.

The procedures of AI/ML BM test with single AoA can be as following. We are open to do necessary adjustments.

Proposal 17: The procedures of AI/ML BM test with single AoA:

Beam lock at test UE during the test.

- Step 1: Calibration

  - TE transmit the signal with a fixed power, UE receive signal and report RSRP to TE. TE can get the relationship between TE’s transmit power and the reported RSRP. This can be done multiple times to reduce the impact of noise.

- Step 2: Main test

  - TE get the test dataset. The dataset can be early prepared through computer-like simulation using standardized UE radiation pattern in 38.901 and is the same for all the test UEs.

  - TE transmit the calibrated power on each Tx beam in set B based on the test dataset. UE measures set B and predicts Top K or RSRP of set A.

2.5.2.2 Multiple AoAs test system

In the past few meetings, there were some proposals to further enhance multiple AoAs test system for AI/ML BM test. After studying the Tdocs, we still have some concerns.

We also think UE rotation is necessary if to use multiple AoA test systems. Without UE rotation during the test, Rx beamforming gain for each AoA is basically fixed. For BM-Case2, to evaluate temporal DL Tx beam prediction, there should be some variation in beam direction over time, including UE rotation. But according to the information provided by TE vendors [2-5], it may be difficult and costly to support frequent and flexible UE rotation, i.e., rotation 2o, 5o. Now, the time needed to rotate UE is about 1s, which is not acceptable.

Proposal 18: If to use multiple AoAs test system for AI/ML BM test, UE rotation should be considered.

Observation 7: The time needed to rotate UE in FR2 MIMO OTA test systems, i.e., about 1s, is not acceptable for AI/ML.

To reduce the complexity, how to generate a simplified CDL channel was studied. It seems the analysis focused on the similarity of the simplified CDL channel and CDL channel. As which is also pointed out by several companies that it is the impact on model performance that really matters. Proponents should analyse the impact on AI/ML BM models by using multiple AoA test system.

Proposal 19: The impact on performance of AI/ML BM models by using multiple AoA test systems should be studied at first before concluding to use multiple AoAs test system.

To use multiple AoAs test system, the test will include two steps. In the first step, TE generates the channel and transmits Tx beams in Set B only. UE reports prediction results. In the next step, TE tries to generate the same channel as in the first step and transmits all the Set A Tx beams. The order of the above steps can also be reversed. In the test, as the measurement results may be 10dB lower or 20dB higher the transmitted RSRP at TE, it is difficult to check whether UE reports the real measurement results of Set A or reports fake values according to the previous predicted results to pass the test. Another issue is how to ensure the channels in these two steps are the same. We observed there were proposals to make the mapping relation of RS index and beam index different in the two steps. We admit it is workable to avoid UE cheating in the test. But this may also make model monitoring fail and is not preferred.

Proposal 20: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

2.5.2.3 Dynamic range of Single AoA and Multiple AoA test systems

In early releases, dynamic range of IFF and DFF test systems had been analysed. Since several years have passed, there may be some new progress. But we are not experts, so here we just copy the previous results in TR38.810 here for information.

Table 2

| Test methods | Wanted signal (dBm/SCS) at reference point: upper bound | SNR at baseband (dB): upper bound | SNR at baseband (dB): upper bound – 30dB |
| --- | --- | --- | --- |
| IFF, peak fine beam | -83.5 | 25.1 | -4.9 |
| IFF, peak rough beam | -83.5 | 18.1 | -11.9 |
| IFF, non-peak fine beam | -83 | 13 | -17 |
| IFF, non-peak rough beam | -83 | 6 | -24 |
| DFF, peak fine beam | -83.8 | 24.8 | -5.2 |
| DFF, peak rough beam | -83.8 | 17.8 | -12.2 |
| DFF, non-peak fine beam | -83.3 | 12.7 | -17.3 |
| DFF, non-peak rough beam | -83.3 | 5.7 | -24.3 |

Considering the upper bound of SNR with rough beam are much lower than that with fine beam as shown in Table 1, fine beam should be used in AI/ML BM test to get a larger dynamic power range. Next we will only focus testing using fine beam.

Observation 8: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 21: Fine beam should be used in AI/ML test.

It can be observed that the power level at reference point (before UE Rx beam forming) for different cases are basically the same as it is determined by maximum Tx power at TE.

The upper bound of SNR at UE baseband is determined by several factors. One of the factors is Tx power at TE. If TE can do better now, the upper bound of SNR can be higher.

Regarding the lower bound, in our understanding, it is not quite a limitation. As observed through the simulation results in 2.2, the week beams should be excluded otherwise the predict accuracy of AI/ML BM model will be very poor after considering the impact of measurement error.  In our simulation, we excluded the beams which is 30dB weaker than the best beam, and the Top 5/1 prediction accuracy is above 90%. Therefore 30dB dynamic range may be enough for the test based on our initial simulations. We can get the corresponding SNR at baseband in the last column. It can be observed that SNR in the last column is above -6dB only when using peak fine beam. It is important to have a large SNR dynamic range, otherwise the measurement error will be large for many beams. If to use multiple AoA test system, it is not possible to use peak fine beam only. With single AoA, it is possible to use peak fine beam. And we propose to use peak fine beam in AI/ML BM test. It can be observed that the upper bound of SNR with non-peak fine beam is 12.1dB lower than that of peak fine beam, which means SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

Observation 9: Among all the test methods, only using peak fine beam can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 10: Multiple AoA test systems cannot use peak fine beam only.

Proposal 22: Using peak fine beam in AI/ML BM test with single AoA test system.

Observation 11: SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

- Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Observation 1: Both DL Tx beam prediction and beam pair prediction are studied in R18 AI/ML SI, but only DL Tx beam prediction is in the scope of R19 AI/ML WI.

Proposal 1: Rx beam of the predicted Tx beam which is not measured can be known if the beam is QCL Type-D to a Tx beam in Set B.

Proposal 2: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

Observation 2: In terms of Top K/1 prediction accuracy, the performance of AI/ML BM case 1 models for different scenarios may have much difference.

Proposal 3: Further study and discuss whether it is possible to define scenario-agnostic performance requirements for AI/ML BM. If not, maybe we can seek for similar solutions as TDCP, i.e., define the test cases for limited scenarios instead of defining scenario-agnostic accuracy requirements.

Proposal 4: Further discuss which scenario among the following to define requirements and test cases for:

- Scenario 1: Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use SSB beams to predict CSI-RS beams.

  - Scenario 1a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 1b: RSRP with specific Rx beam

- Scenario 2: Set B is a subset of Set A. E.g., use part of SSB beams to predict all the SSB beams.

  - Scenario 2a: RSRP with best Rx beam for each Tx beam in set B

  - Scenario 2b: RSRP with specific Rx beam

Proposal 5: Not to define separate requirements or test cases for beam pair prediction in R19 WI.

Observation 3: The performance degrades a bit when all the UEs are outdoor UEs if measurement error is considered while the performance degrades significantly when most are indoor UEs.

Observation 4: Filtering out the data exceeds TE’s maximum Tx power has little impact on the prediction accuracy.

Proposal 6: Discuss which UE distribution to be used in the test and to be defined requirements for, e.g., 80% indoor+20% outdoor UEs or 100% outdoor UEs.

Proposal 7: When defining the accuracy requirements for AI/ML BM models, the impact of measurement error should be considered.

Proposal 8: When defining the accuracy requirements for AI/ML BM models, the impact of the dynamic range of TE should be considered.

Proposal 9: If RSRP and beam ID are reported, use RSRP accuracy and Top-K/1 (%) “the percentage of the Top-1 strongest beam is one of the Top-K predicted beams" as the KPI. If only beam ID is reported, use Top-K/1 (%) “the percentage of the Top-1 strongest beam is one of the Top-K predicted beams" as the KPI.

Proposal 10: Activation delay of AI/ML BM model shall also be counted into the prediction delay for aperiodic reporting and the first report of semi-persistent reporting, FFS the value.

Proposal 11: For AI/ML BM, further discuss whether the performance under synthetic channels can reflect the performance of AI/ML models in the real field.

Proposal 12: For AI/ML BM, discuss what the testing goal is, to verify the model can work well in real field or just to test UE can train a model that works well under a specific scenario.

Proposal 13: If the testing goal is to verify the model can work well in real field, and the performance under a specific channel/scenario cannot reflect the performance in the real field, it is necessary to design a test system that is possible to use field data in the test.

Proposal 14: In AI/ML BM test, emulate the spatial-selective and time-varying in propagation conditions through adjusting Tx power at TE on each beam at different time.

Observation 5: Single AoA test system can be applicable to all kinds of synthetic channels as well as using real field data. It can be easily extended to different scenarios and settings.

Observation 6: With single AoA, TE can know the expected RSRP of Tx beams at UE baseband.

Proposal 15: Use existing IFF or enhance IFF test systems with single AoA to test R19 DL Tx beam prediction models.

Proposal 16: Standardized UE radiation pattern in 38.901 can be assumed in R19 AI/ML BM test with single AoA test system instead of using test UE’s radiation pattern.

Proposal 17: The procedures of AI/ML BM test with single AoA:

Beam lock at test UE during the test.

- Step 1: Calibration

  - TE transmit the signal with a fixed power, UE receive signal and report RSRP to TE. TE can get the relationship between TE’s transmit power and the reported RSRP. This can be done multiple times to reduce the impact of noise.

- Step 2: Main test

  - TE get the test dataset. The dataset can be early prepared through computer-like simulation using standardized UE radiation pattern in 38.901 and is the same for all the test UEs.

  - TE transmit the calibrated power on each Tx beam in set B based on the test dataset. UE measures set B and predicts Top K or RSRP of set A.

Proposal 18: If to use multiple AoAs test system for AI/ML BM test, UE rotation should be considered.

Observation 7: The time needed to rotate UE in FR2 MIMO OTA test systems, i.e., about 1s, is not acceptable for AI/ML.

Proposal 19: The impact on performance of AI/ML BM models by using multiple AoA test systems should be studied at first before concluding to use multiple AoAs test system.

Proposal 20: How to avoid UE cheating in the test, such as reports fake values according to the previous predicted results, shall be further discussed to use the reported RSRP measurement as ground truth.

Observation 8: The upper bound of SNR with rough beam are much lower than that with fine beam.

Proposal 21: Fine beam should be used in AI/ML test.

Observation 9: Among all the test methods, only using peak fine beam can make sure the lowest SNR above -6dB with 30dB dynamic power range.

Observation 10: Multiple AoA test systems cannot use peak fine beam only.

Proposal 22: Using peak fine beam in AI/ML BM test with single AoA test system.

Observation 11: SNR dynamic range of multiple AoA test system would be at least 12dB smaller than single AoA test system.

- Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface