3GPP TSG-RAN WG4 Meeting # 114bis	                                            R4-2503609

Wuhan, CN, April 7 – 11, 2025

Agenda item:	7.21.3

Source:	MediaTek Inc.

Title:	Discussion on testability and interoperability of AI mobility

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

According to the agreed work plan last meeting, this meeting RAN4 will focus on RRM measurement prediction use-case. In this contribution, we would like to discuss the related testability issues.

- Discussion

RAN4#112bis agreed to consider “Conductive testing” as a baseline option for testing in FR1. There were some proposals to study the testability regarding with more cells and more than one frequency layers in the test. In our understanding, there is no testability issue with conductive testing in FR1.

Take scenario#3 FR1 to FR1 inter-frequency prediction as an example. There may be several cells on different frequency layers. There will be no issue to simulate multiple frequency layers which is widely used in some non-AI test cases, e.g., inter-frequency measurement/HO test cases. In our understanding, as UE is connected to the channel simulator through cable, the test would be similar as computer simulations. If the model is to predict measurement results of Cell 2 and Cell 3 on different frequencies from serving cell (Cell 1), a test setup as shown in Fig.1 can be used. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. SSB is transmitted on cell 1 at all the SMTC occasions.  For cell 2 and cell 3, SSB will not be transmitted on the SMTC/MG occasions to be predicted. We think it is sufficient to change TE Tx power every symbol to avoid any impact on UE’s algorithms. TE vendors may need to further check whether it is possible to change Tx power every symbol.

To avoid missing something, we are open to take a scenario as an example to further discuss the test set up details.

Fig. 1

Proposal 1: Further check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility.

Proposal 2: In FR1, computer simulations alike test setup can be adopted for AI mobility test. The following figure can be used to determine the power transmitted at TE on each cell, i.e., TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model.

- Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: Further check whether it is possible to change Tx power every symbol at TE using conductive test in FR1 for AI/ML mobility.

Proposal 2: In FR1, computer simulations alike test setup can be adopted for AI mobility test. The following figure can be used to determine the power transmitted at TE on each cell, i.e., TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.