# 3GPP TSG-RAN WG4 Meeting # 114bis		  R4-2503643

# Wuhan, Hubei, China, 07th – 11th April, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on BS RF requirements for supporting 7 MHz channel bandwidth

# Agenda Item:	7.8.3

# Document for:	Approval

- Introduction

During RAN#106 meeting, a new Work Item [1] has been approved on a new added 7 MHz Channel Bandwidth for n26 and n5. And after the RAN4#114 meeting, some agreements were made to promote further research on the RF requirements. In RAN#107 a revised WID was approved for the same issue[2].In this contribution, we continue to study the BS RF requirements and an overview of expected impacts based on the progress. The agreements related to BS made in last RAN4# 114 meeting are shown as below:

Scope:

Agreement:

- The 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth.

- In-band NB-IoT can be supported in 7MHz channel BW

System parameters – Release independence

Agreement:

- 35 RBs will be supported.

- Support of 7 MHz channel BW could be Release independent from Rel-15.

  - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz.

  - Support of 7MHz is optional in Rel-19 and in the previous releases.

- Introducing 7 MHz channel BW has no impact on channel spacing, channel raster nor sync raster.

BS RF

Agreement:

- Conducted requirements shall be covered within this WI.

- Muti-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID, but no new BCS is specified within this WI.

- 7 MHz should not be supported by IAB

- 7 MHz should not be supported by NCR

- Discussion

2.1 General aspect

To address the release dependence, majority companies agreed to support that 7 MHz channel BW could be release independent from Rel-15. To avoid the NBC issue when 7 MHz channel BW is introduced, it was agreed that 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth. It means that the minimum BW in the band supporting 7 MHz is at least including 5 MHz, and there seems no longer any problems when 7MHz is neither the minimum nor the maximum BW in band.

For initial BWP including CORESET0 for UE initial access, legacy initial BWP size for FR1 for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz or with minimum channel bandwidth 3 MHz and channel bandwidth larger than 3 MHz is usually 5 MHz or 10 MHz or 20 MHz, as defined in table 13-1 in TS 38.213, this table can naturally be applied to 7 MHz CBW, it can be seen only 5 MHz CBW can be used for 7 MHz CBW initial access, and no spec impact is found. From this point of view, UE can work well under current spec, and RAN4 need not to add any additional restriction.

Proposal1: No need to add any additional restriction for initial BWP size to support of 7 MHz channel BW release independent from Rel-15 in RAN4.

It was also discussed that whether BS OTA requirements should also be covered in last meeting. Since AAS has been supported for sub-1G, we do not see any problems to support AAS for 7 MHz CBW in band n5 and band n26. According the note in WID, the 7 MHz bandwidth can be introduced also for other bands when this WI is completed, and specifying the OTA requirements is more benefit for commercial deployment with 7MHz CBW, so it is suggested to support AAS for 7 MHz CBW in Rel 19.

Proposal2: Supporting to specify BS OTA requirements in Rel 19.

2.2 FRCs for new added 7 MHz CBW

Regarding the BS RF requirement, a number of Fixed Reference Channels (FRCs) have been defined for almost all Rx requirements such as reference sensitivity level, dynamic range, ACS, etc. In addition to that, both wideband operation and NB-IoT operation are supported, the FRCs are defined in TS 36.104, and for wideband operation, the FRCs are specified in annex A.1 and annex A.2 in TS 38.104.

Observation1：The FRCs for NB-IoT operation are defined in TS 36.104, and FRCs for wideband operation are specified in annex A.1 and annex A.2 in TS 38.104.

For 7 MHz CBW, RAN4 need to determine the FRCs for the new added 7 MHz. For NB-IoT operation in NR in band, it is suggested to reused the FRCs defined in TS 36.104, while for wideband operation in NR in band, we need to consider whether to define new FRCs for 7 MHz or to reuse the legacy FRCs defined for 5 MHz/15 kHz SCS CBW carefully. If legacy FRCs defined for 5 MHz/15 kHz SCS CBW are reused for 7 MHz CBW, then some overlapping of FRC occurrences are needed for conformance testing to cover the whole 7MHz CBW, which may increase the test burden. Take reference sensitivity level for an example, when the 5 MHz CBW G-FR1-A1-1 is used for 7 MHz CBW reference sensitivity level conformance testing, the remaining 2 MHz CBW can not be covered, then some overlapping might be needed, as shown in figure 1:

Figure 1 An illustration of overlapping FRC occurrences

However, such overlapping and test burden issues can be avoided by defining new FRCs with a proper CBW for 7 MHz. With consideration to that, we prefer to define new FRCs.

Proposal3: For NB-IoT operation in NR in band, the legacy FRCs defined in TS 36.104 can be reused.

Proposal4: For wideband operation in NR in band, to define new FRCs for new added 7MHz/15 k SCS CBW.

To define new FRCs, table A.1-1 and table A2.1-1 need to be updated for 7 MHz CBW. According to our analysis, the updates are added in new row(s) as below:

Table A.1-1: FRC parameters for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR1-A1-1 | G-FR1-A1-2 | ... | - G-F - R1-A - 1-[X] | G-F R1-A1-[X’] | - G-F - R1-A 1-[X’’] |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | ... | 15 | 15 | 15 |
| Allocated resource blocks | 25 | 11 | ... | 35 | 34 | 20 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | ... | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | ... | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | ... | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 2152 | 984 | ... | 3104 | 2976 | 1864 |
| Transport block CRC (bits) | 16 | 16 | ... | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | ... | - | - | - |
| Number of code blocks - C | 1 | 1 | ... | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 2168 | 1000 | ... | 3120 | 2992 | 1880 |
| Total number of bits per slot | 7200 | 3168 | ... | 10080 | 9792 | 6048 |
| Total symbols per slot | 3600 | 1584 | ... | 5040 | 4896 | 3024 |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with = 2, = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity NOTE 3: Code block size including CRC (bits) equals to in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |

Table A.2-1: FRC parameters for FR1 dynamic range and OTA dynamic range

| Reference channel | G-FR1-A2-1 | G-FR1-A2-2 | G-FR1-A2-3 | G-FR1-A2-4 | G-FR1-A2-5 | G-FR1-A2-6 | G-FR1-A2-15 | G-FR1-A2-[Y] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 15 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 | 15 | 35 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate (Note 2) | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 | 2/3 |
| Payload size (bits) | 9224 | 4032 | 4032 | 38936 | 18960 | 8968 | 5504 | 13064 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | - | - | 24 | 24 | 24 | - | 24 |
| Number of code blocks - C | 2 | 1 | 1 | 5 | 3 | 2 | 1 | 2 |
| Code block size including CRC (bits) (Note 3) | 4648 | 4056 | 4056 | 7816 | 6352 | 4520 | 5528 | 6568 |
| Total number of bits per slot | 14400 | 6336 | 6336 | 61056 | 29376 | 13824 | 8640 | 20160 |
| Total symbols per slot | 3600 | 1584 | 1584 | 15264 | 7344 | 3456 | 2160 | 5040 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0= 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: MCS index 16 and target coding rate = 658/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. |  |  |  |  |  |  |  |  |

Proposal5: To define the FRC parameters marked in blue in table A.1-1 for 7 MHz CBW FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity.

Proposal6: To define the FRC parameters marked in blue in table A.2-1 for 7 MHz CBW FR1 dynamic range and OTA dynamic range.

2.3 BS RF requirement for 7 MHz channel bandwidths

The BS RF requirements can be divided into two categories: the first category is band/channel bandwidth agnostic requirements, and the second category is band/channel bandwidth specified requirements. For the former one, the requirements such as base station output power, transmit ON/OFF power, transmitted signal quality, occupied bandwidth, etc, are defined as channel bandwidth agnostic way, which means these requirements are not impacted with introduction of the 7 MHz channel bandwidth. While for the latter one, almost all of the Rx requirements are defined based on channel bandwidth. Thus for these requirement, the newly 7MHz channel bandwidth shall be included.

The radiated Tx/Rx requirements are usually determined according to the conducted corresponding Tx/Rx requirements by just reusing or adding an offset, so we propose to prioritize the conducted requirements discussion, and the radiated requirements can be captured later.

Proposal7: Supporting to prioritize the conducted Tx/Rx requirements discussion.

It is noted that conducted requirements which are impacted are listed firstly in the following parts, the radiated requirements can be determined later after the agreement of the corresponding conducted requirements.

2.3.1 Total power dynamic range

As agreed in last meeting, the maximum transmission bandwidth configuration NRB =35, then we can determine that total power dynamic range for 7 MHz/15k SCS is 15.4 dB.

Table 6.3.3.2-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 3 | 11.7 | N/A | N/A |
| 5 | 13.9 | 10.4 | N/A |
| 7 | 15.4 | N/A | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |
| 25 | 21.2 | 18.1 | 14.9 |
| 30 | 22 | 18.9 | 15.7 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 23.3 | 20.2 | 17 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 24.3 | 21.2 | 18.1 |
| 60 | N/A | 22 | 18.9 |
| 70 | N/A | 22.7 | 19.6 |
| 80 | N/A | 23.3 | 20.2 |
| 90 | N/A | 23.8 | 20.8 |
| 100 | N/A | 24.3 | 21.3 |

For NB-IoT RB power dynamic range for NB-IoT operation in NR in-band, 7 MHz CBW should be captured in Table 6.3.4.2-1：

Table 6.3.4.2-1: NB-IoT RB power dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 7, 10 | Any | +6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 20 | Within center 102*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Within center 90% of BS channel bandwidth | +6 |
|  | Other | +3 |

2.3.2 ACLR

For ACLR/CACLR, the existing requirements can be reused, and we need to add 7 MHz CBW to Table 6.6.3.2-1 and Table 6.6.3.2-2a for ACLR limit, and add 7 MHz CBW to Table 6.6.3.2-3 for CACLR limit.

Proposal8: For ACLR/CACLR, the existing requirements are reused for 7 MHz CBW.

2.3.3 Reference sensitivity level

According to the formula of reference sensitivity:

Where:

-	BW is the maximum transmission bandwidth,e.g. 35PRB for 7 MHz SCS:15 kHz;

-	NF is BS noise figure, equal to 5 dB for Wide Area BS, 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for which we reach 95% throughput according to the G-FR1-A1-[X]/G-FR1-A1-[X’] proposed in section 2.2.

The SNRs based on the link level simulations of the new defined FRCs as well as the corresponding reference sensitivity are obtained in Table2.3.3 as below:

Table2.3.3 SNR and corresponding reference sensitivity

| FRC for 7 MHz CBW | SNR | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | For WA | For MR | For LA |
| G-FR1-A1-[X] | -1.23 dB | -100.2 | -95.2 | -92.2 |
| G-FR1-A1-[X’] | -1.27dB | -100.2 | -95.2 | -92.2 |

The corresponding LLS results are shown in figure 2.3.3-1 and 2.3.3-2 respectively below:

Figure 2.3.3-1 SNR-throughput results for G-FR1-A1-[X]

Figure 2.3.3-2 SNR-throughput results for G-FR1-A1-[X’]

2.3.4 Dynamic range

For dynamic range requirement, the mean power of the wanted signal is defined in the following method:

Pwanted = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20+SNR+IM

Where

-	NF is noise figure of NR BS receiver in dB, where 5dB is assumed for WA, 10dB for MR ,13dB for LA;

-	IM is implementation margin reserved for BS manufacturer in dB; for high MCS level, IM is assumed to be 2.5dB;

-	NRB is 35 depends on the FRC G-FR1-A2-[Y] defined for dynamic range requirement;

-	SCS is 15k Hz;

-	SNR is the value to satisfy the 95% throughput of measurement channel defined by FRC G-FR1-A2-[Y] proposed in section 2.2;

The SNR based on the link level simulations of the new defined FRCs as well as the corresponding Pwanted is obtained in Table2.3.4-1 as below:

Table2.3.4-1 SNR and corresponding wanted signal mean power

| FRC for 7 MHz CBW | SNR | Pwanted(dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | For WA | For MR | For LA |
| G-FR1-A2-[Y] | 8.977 | -69.5 | -64.5 | -61.5 |

The corresponding LLS result is shown in figure 2.3.4 below:

Figure 2.3.4 SNR-throughput results for G-FR1-A2-[Y]

For the mean power of interfering signal, it is defined in the following method:

PIntf = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20

Where:

-	NRB is 35;

-	SCS is 15k Hz;

-	NF is noise figure of NR BS receiver in dB, where 5dB is assumed for WA, 10dB for MR ,13dB for LA;

Then the mean power of interfering signal PIntf is calculated as in Table2.3.4-2 as below:

Table2.3.4-1 The interfering signal PIntf

| FRC for 7 MHz CBW | PIntf for WA | PIntf for MR | PIntf for LA |
| --- | --- | --- | --- |
| G-FR1-A2-[Y] | -81.0 dBm | -76.0 dBm | -73.0 dBm |

2.3.5 ACS

The exiting ACS requirements except for the interferer frequency offset value can be reused for 7 MHz, it is proposed to add 7MHz CBW for base station ACS requirement in table 7.4.1.2-1.

The Base Station ACS interferer frequency offset values should be recalculated, and we propose to use the same type of interfering signal of 5 MHz CBW(5MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs) as the interfering signal of 7 MHz CBW. According to the formula defined in TR 38.817-02 section 7.4.1, the ACS interferer frequency offset value can be obtained as ±2.5075 MHz.

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±1.5075 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 5 | ±2.5025 |  |
| 7 | ±2.507 |  |
| 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal |
| 15 | ±2.5125 | 15 kHz SCS, 25 RBs |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 |  |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 | 20 MHz DFT-s-OFDM NR signal |
| 60 | ±9.4725 | 15 kHz SCS, 100 RBs |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

2.3.6 In-band blocking

The exiting In-band blocking requirements except for the narrowband blocking interferer frequency offsets can be reused for 7 MHz, it is proposed to add  In-band blocking requirement for 7MHz CBW in table 7.4.2.2-1, table 7.4.2.2-2, table 7.4.2.2-2a and table 7.4.2.2-3.

For Base Station narrowband blocking interferer frequency offsets, the same type of interfering signal of 5 MHz CBW(5MHz DFT-s-OFDM NR signal 15 kHz SCS, 1 RBs) is proposed as the interfering signal of 7 MHz CBW,

and ±(355+m*180) kHz, m=0, 1, 2, 3, 4, 9, 14, 19, 24 is proposed for 7MHz/15k SCS CBW.

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±(255+m*180), m=0, 1, 2, 3, 4, 7, 10, 13 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 5 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 7 | ±(355+m*180) kHz, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | ±(360+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | ±(350+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |
| 30 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 35 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 40 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 45 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 50 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 60 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 70 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 80 | ±(560+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 90 | ±(570+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| 100 | ±(565+m*180), m=0, 1, 2, 3, 4, 29, 54, 79, 99 |  |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

2.3.7 RX IMD

For interfering signals for intermodulation requirement, CW and 5 MHz DFT-s-OFDM NR signal are proposed for 7 MHz CBW interfering signals type, and then interfering signal centre frequency offset values of ±7.45 MHz@CW, ±17.5 MHz@5 MHz DFT-s-OFDM NR signal are considered for the interfering signal centre frequency offset.

Table 7.7.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 3 | ±4.5 | CW |
|  | ±10.5 | 3 MHz DFT-s-OFDM NR signal |
| 5 | ±7.5 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 10 | ±7.465 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 15 | ±7.43 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 20 | ±7.395 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |
| 25 | ±7.465 | CW |
|  | ±25 | 20MHz DFT-s-OFDM NR signal (Note 2) |
| 30 | ±7.43 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 35 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 40 | ±7.45 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 45 | ±7.37 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 50 | ±7.35 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 60 | ±7.49 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 70 | ±7.42 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 80 | ±7.44 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 90 | ±7.46 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| 100 | ±7.48 | CW |
|  | ±25 | 20 MHz DFT-s-OFDM NR signal (Note 2) |
| NOTE 1: Number of RBs is 25 for 15 kHz subcarrier spacing and 10 for 30 kHz subcarrier spacing. NOTE 2: Number of RBs is 100 for 15 kHz subcarrier spacing, 50 for 30 kHz subcarrier spacing and 24 for 60 kHz subcarrier spacing. NOTE 3: The RBs shall be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. |  |  |

For interfering signals for narrowband intermodulation requirement, CW and 5 MHz DFT-s-OFDM NR signal 1 RB are proposed for 7 MHz CBW interfering signals type, and then interfering signal centre frequency offset values of ±400 kHz@CW, ±1240 kHz@5 MHz DFT-s-OFDM NR signal 1 RB are considered for the interfering signal centre frequency offset.

Table 7.7.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±360 | CW |
|  | ±960 | 3 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 5 | ±360 | CW |
|  | ±1420 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 7 | ±400 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 10 | ±370 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 15 (Note 2) | ±380 | CW |
|  | ±1960 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 20 (Note 2) | ±390 | CW |
|  | ±2320 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 25 (Note 2) | ±325 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 30 (Note 2) | ±335 | CW |
|  | ±2350 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 35 (Note 2) | ±345 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 40 (Note 2) | ±355 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 45 (Note 2) | ±365 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 50 (Note 2) | ±375 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 60 (Note 2) | ±395 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 70 (Note 2) | ±415 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 80 (Note 2) | ±435 | CW |
|  | ±2710 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 90 (Note 2) | ±365 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| 100 (Note 2) | ±385 | CW |
|  | ±2530 | 20 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. NOTE 3: The centre of the interfering RB refers to the frequency location between the two central subcarriers. |  |  |

2.3.8 ICS

For ICS requirement for 7MHz CBW, new defined FRC G-FR1-A1-[X’’] is used for wanted signal power level and interfering signal mean power calculation.

The formula to calculate the wanted signal power level is defined as following:

Wanted signal power level = -174 dBm/Hz+10*log10(BW)+NF+SNR+IM+3;

Where:

-	BW is wanted signal bandwidth in Hz, (20PRB for 7MHz SCS:15 kHz);

-	NF is agreed as 5dB for WA, 10dB for MR, 13dB for LA BS;

-	SNR is dependent on the link level simulation results according to FRC G-FR1-A1-[X’’] ;

-	IM is implementation margin which is assumed as 2dB;

-	3dB is reference sensitivity degradation which is reused from legacy E-UTRA requirement;

The SNR based on the link level simulations of the new defined FRC as well as the corresponding Pwanted is obtained in Table2.3.8-1 as below:

Table2.3.8-1 SNR and corresponding wanted signal power level

| FRC for 7 MHz CBW | SNR | Pwanted(dBm) |  |  |
| --- | --- | --- | --- | --- |
|  |  | for WA | for MR | for LA |
| G-FR1-A1-[X’’] | -1.209 dB | -99.6 | -94.6 | -91.6 |

The corresponding LLS result is shown in figure 2.3.8 below:

Figure 2.3.8 SNR-throughput results for G-FR1-A1-[X’’]

According to the formula defined in TR38.817-02 section 7.8:

Interfering signal power level = -174dBm/Hz+10*log10(BW)+NF+ICS;

Where:

-	BW is interfering signal bandwidth in Hz, (15PRB for 7MHz SCS:15 kHz);

-	NF is agreed as 5dB for WA, 10db for MR, 13dB for LA BS;

-	ICS is agreed as 25dBc for all BS type which is the same as legacy E-UTRA ICS;

Then the Interfering signal power level is calculated in Table 2.3.8-2:

Table2.3.8-2 The interfering signal PIntf

| FRC for 7 MHz CBW | PIntf for WA | PIntf for MR | PIntf for LA |
| --- | --- | --- | --- |
| G-FR1-A1-[X’’] | -79.6 dBm | -74.6 dBm | -71.6 dBm |

2.4 BS RF requirement spec impact for 7 MHz channel bandwidths

According to the analysis in section 2.3, we give a briefly summarize on the spec impact of conducted BS RF requirement to consider 7 MHz channel bandwidths in table 2.4 as below:

Table 2.4: Conducted BS RF requirement for 7 MHz channel bandwidths

| Tx RF requirements part |  |
| --- | --- |
| Tx requirements to update | Proposals |
| 6.2 Base station output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Specification impact: -To add Total power dynamic range requirement for 7 MHz CBW in table Table 6.3.3.2-1: -To add NB-IoT RB power dynamic range for NB-IoT operation in NR in-band requirement for 7 MHz CBW in Table 6.3.4.2-1 |
| 6.3.4 NB-IoT RB power dynamic range for NB-IoT operation in NR in-band | No specification impact. |
| 6.4.1 Transmitter OFF power | No specification impact. |
| 6.4.2 Transmitter transient period | No specification impact. |
| 6.5.1 Frequency error | No specification impact. |
| 6.5.2 Modulation quality | No specification impact. |
| 6.5.3 Time alignment error | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Specification impact: -To add ACLR requirement for 7MHz CBW in Table 6.6.3.2-1, Table 6.6.3.2-2a and Table 6.6.3.2-3: |
| 6.6.4 Operating band unwanted emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | No specification impact. |
| Rx RF requirements part |  |
| Rx requirements to update | Proposals |
| 7.2 Reference sensitivity level | Specification impact: -To add REFSENS requirement for 7MHz CBW |
| 7.3 Dynamic range | Specification impact: -To add dynamic range requirement for 7MHz CBW |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Specification impact: -To add 7MHz CBW for base station ACS requirement in table 7.4.1.2-1 -To add 7MHz CBW for Base Station ACS interferer frequency offset values in table 7.4.1.2-2 |
| 7.4.2 In-band blocking | Specification impact: -To add In-band blocking requirement for 7MHz CBW in table 7.4.2.2-1, table 7.4.2.2-2, table 7.4.2.2-2a, table 7.4.2.2-3, |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | Specification impact: -To add RX IMD requirement for 7MHz CBW: |
| 7.8 In-channel selectivity | Specification impact: -To add ICS requirement for 7MHz CBW: |

Proposal9:To consider the proposals in Table 1 for BS RF requirement for 7 MHz channel bandwidths.

- Conclusion

In this contribution, some initial views on BS RF requirements are provided and the proposals ares made as following:

Proposal1: No need to add any additional restriction for initial BWP size to support of 7 MHz channel BW release independent from Rel-15 in RAN4.

Proposal2: Supporting to specify BS OTA requirements in Rel 19.

Observation1：The FRCs for NB-IoT operation are defined in TS 36.104, and FRCs for wideband operation are specified in annex A.1 and annex A.2 in TS 38.104.

Proposal3: For NB-IoT operation in NR in band, the legacy FRCs defined in TS 36.104 can be reused.

Proposal4: For wideband operation in NR in band, to define new FRCs for new added 7MHz/15 k SCS CBW.

Proposal5: To define the FRC parameters marked in blue in table A.1-1 for 7 MHz CBW FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity.

Proposal6: To define the FRC parameters marked in blue in table A.2-1 for 7 MHz CBW FR1 dynamic range and OTA dynamic range.

Proposal7: Supporting to prioritize the conducted Tx/Rx requirements discussion.

Proposal8: For ACLR/CACLR, the existing requirements are reused for 7 MHz CBW.

Proposal9:To consider the proposals in Table 1 for BS RF requirement for 7 MHz channel bandwidths.

- Reference

RP-243323, 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA.

RP-243323, NR FR1 7 MHz Channel Bandwidth , China Telecom, T-Mobile USA.

3GPP TS38.104 NR Base Station (BS) radio transmission and reception.

3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio                           transmission and reception"