# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503645

# Wuhan, China, 7th – 11st April, 2025

Agenda Item:	7.21.1

Source: 	OPPO

Title: 	TP on RAN4 aspects for TR 38.744

Document for:	Approval

- Introduction

In this contribution, we provide the text proposal on the framework of RAN4 part in TR 38.744 to capture the RAN4 agreements.

- Text proposal

< Start of text proposal >

6.2	Interoperability, testability, and RRM requirements

6.2.1 Scenarios and use cases

RAN4 to define requirements for scenario 2, 3, 4 in WI stage.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For Scenario 2, 3 and 4, the following 3 cases can be discussed:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

6.2.2 RRM requirements impact for measurement prediction

6.2.2.1 General

- RAN4 to study the impacts on requirements for the following case:

  - L3 cell level RSRP (Point C) prediction

  - FFS L1 beam-level (Point A1)

  - FFS L3 beam-level (Point E)

6.2.2.2 Core requirements

6.2.2.3 Performance requirements

Use the following definition for absolute and relative accuracy:

  - Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

  - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1– reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1– ground truth of RSRP of cell 2)

Down-select from the following alternatives for the definition of ground truth:

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions.

  - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

6.2.3 RRM requirements impact for measurement event prediction

6.2.3.1 Core requirements

6.2.3.2 Performance requirements

6.2.4 Testability and Interoperability

6.2.4.1 Testing goal

As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

6.2.4.2 Testing setup

6.2.4.3 Testing principle

6.2.4.4 Generalization

<End of text proposal>