3GPP TSG-RAN WG4 Meeting #114bis	R4-2503652

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	On performance metric for FR1 dynamic MIMO

Source:	Samsung

Agenda item:	7.14.2.3

Document for:	Discussion

1.	Introduction

About the performance metric of dynamic MIMO OTA, it is agreed that different device orientations and rotations shall be combined into a single CDF curve. The open issues include the required percentile value and how to handle Uma and Umi channel models in the final requirement. [1]

Issue 1-2: Dynamic MIMO OTA test and UE performance metric Agreement: - All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test. FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF. - Use the following as a starting point: - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

In this contribution, we share our views on above highlighted item – the required percentile value in final requirement derivation.

2. 	Discussion

Given the group has agreed that all measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test, the corresponding FFS part in TR [2] can be removed.

Proposal 1:	Remove the FFS part regarding different each orientation and rotation in TR 38.762 as shown in Annex TP.

Now 3 percentile points are under consideration, i.e., the 10%, 50%, and 90%-ile CDF. It is possible that a UE showing best performance at 90%-ile may not show best performance at 50% and 10%-ile, and vice versa. In usual OTA metric, average performance is adopted as performance metric when handling low/middle/high channels, or consider both average and minimum performance. To some extent, the 10%-tile CDF reflects the minimum value and the 50%-tile CDF reflects the average value. The 90%-tile reflects the best cases which is usually not considered in previous OTA requirements.

Observation 1:	To some extent, the 10%-tile CDF reflects the minimum value and the 50%-tile CDF reflects the average value. The 90%-tile reflects the best cases which is usually not considered in previous OTA requirements.

Based on above observations, requirement at 90%-tile can be excluded. 50%-tile can be adopted for sure while 10%-tile can be further discussed.

If there will be requirements at both 50%-tile and 10%-tile, a joint pass rate should be considered in the measurement campaign which may bring complex to spec derivation.

Observation 2:	if requirements are to be specified at both 50%-tile and 10%-tile, a joint pass rate should be considered in the measurement campaign which may bring complexity to spec derivation

Proposal 2:	For requirement definition, 50%-tile is adopted; 90%-tile is excluded; and 10%-tile can be further discussed.

3. 	Conclusion

Proposal 1:	Remove the FFS part regarding different each orientation and rotation in TR 38.762 as shown in Annex TP.

Observation 1:	To some extent, the 10%-tile CDF reflects the minimum value and the 50%-tile CDF reflects the average value. The 90%-tile reflects the best cases which is usually not considered in previous OTA requirements.

Observation 2:	if requirements are to be specified at both 50%-tile and 10%-tile, a joint pass rate should be considered in the measurement campaign which may bring complexity to spec derivation

Proposal 2:	For requirement definition, 50%-tile is adopted; 90%-tile is excluded; and 10%-tile can be further discussed.

4. 	References

- R4-2502296    “WF for [114][328] MIMO_OTA_Ph3”, CAICT

- R4-2501749    3GPP TR 38.762 v0.2.0 (2024-12)

- CTIA 01.42

5. 	Annex: TP to TR 38.762

Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e.,

TMTUMi=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMi

and

TMTUMa=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMa

Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF

CTMTUMi,10%=CDF10%TMTUMi

CTMTUMi,50%=CDF50%TMTUMi

CTMTUMi,90%=CDF90%(TMTUMi)

CTMTUMa,10%=CDF10%TMTUMa

CTMTUMa,50%=CDF50%TMTUMa

CTMTUMa,90%=CDF90%(TMTUMa)

Whether and how to combine the CTMT metrics for different routes/channel models is FFS.