3GPP TSG-RAN WG4 Meeting #114bis	R4-2503653

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	On TRP TRS spec for narrow width UE

Source:	Samsung

Agenda item:	7.14.3.1

Document for:	Discussion

1.	Introduction

It is one of the WID objectives to specify TRP TRS requirements for Size 2 (narrow) handheld UE [1, RP-250197]:

- Specify TRP and TRS requirements: - Specify FR1 TRP and TRS requirements for NR bands for UE based on operators demand - …… - Size 2 Handheld UEs with 1Tx at legacy bands n1, n28, n41, n78 are first priority Note: Handheld UEs includes Size 1 (wide, width >72mm and ≤92mm) and Size 2 (narrow, width ≥56mm and ≤72mm);

After several meeting cycles, there is no progress on this issue yet. In this contribution we share our views on this aspect and propose to go with offset approach.

2. 	Discussion

Basically there are two ways, one is offset based approach, the other is measurement based approach. For measurement approach it is consensus that the measurement campaign for narrow UE would be a small scale one.

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis) Agreements: - RAN4 can initiate a preliminary (not large scale) measurement campaign for size 2 UEs, FFS how to down select between measurement-based approach and the offset-based approach.

A small scale measurement campaign could not derive TRP TRS spec reliably. Given there is already TRP TRS spec for wide UE in the same band, if the TRP TRS spec for narrow UE is much different from the wide UE’s, it is also misleading to the industry as there is no much difference expected only due to UE width.

Observation 1:	The benefits of a new measurement campaign for Size 2 (narrow) UE is rather limited.

ETSI has already a precedent to specify TRP TRS spec with offset approach for different UE width for LTE. For each band, in many cases the same spec is adopted and in worst case up to 0.5dB deviation is adopted as difference between narrow UE and wide UE, refer to Table 1.

Table 1. EN301908-13 LTE OTA limits for >72mm: offset from narrow device

| Operating band | Unit | Difference between Size 1 and Size 2 for TRP | Difference between Size 1 and Size 2 for TRS |
| --- | --- | --- | --- |
| 1 | dB/10 MHz | -0.5 | 0.5 |
| 3 | dB/10 MHz | -0.5 | 0.5 |
| 7 | dB/10 MHz | -0.25 | 0 |
| 8 | dB/10 MHz | -0.5 | 0.5 |
| 20 | dB/10 MHz | 0 | 0 |
| 28 | dB/10 MHz | 0 | 0 |
| 38 | dB/20 MHz | -0.5 | 0.5 |
| 40 | dB/20 MHz | -0.5 | 0.5 |

The ETSI practice can be taken as a reference. Considering that a new measurement campaign with Size 2 (narrow) UE in a small scale may not benefit much to the final spec determination, the offset-based approach seems the best way to go.

Proposal 1:	RAN4 to adopt offset based approach to specify TRP TRS requirements of Size 2 (narrow) UEs based on existing TRP TRS requirements of Size 1 (wide) UEs.

3. 	Conclusion

Observation 1:	The benefits of a new measurement campaign for Size 2 (narrow) UE is rather limited.

Proposal 1:	RAN4 to adopt offset based approach to specify TRP TRS requirements of Size 2 (narrow) UEs based on existing TRP TRS requirements of Size 1 (wide) UEs.

4. 	References

- RP-250197    “Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3”, vivo, CAICT

- R4-2416590    “Way Forward for [112bis][323] TRP_TRS_Ph3”, vivo