3GPP TSG-RAN WG4 Meeting #114bis	R4-2503654

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	On remaining issues for STxMP test method

Source:	Samsung

Agenda item:	7.15.2

Document for:	Discussion

1.	Introduction

The only remaining issue for STxMP test method lies in MU aspect, especially system related MU which is not quite aligned with core requirement [1, R4-2502294]:

Whether to consider additional MU for distinguishing power per beam - Agreements - No need to introduce additional MU term for distinguishing power per beam - FFS on the impact of the system related MU MU assessment for Minimum peak EIPR and EIRPmax and TRPmax testing with STxMP - Agreements - RAN4 to consider the following MU assessment for minimum peak EIRP, EIRPmax and TRPmax testing with STxMP as the starting point. - FFS on how to accommodate the difference from the core requirement derivation assuming perfect direction compared to the practical test where there is finite resolution.

In this contribution we share our views on the MU highlighted as above.

2. 	Discussion

The systematic uncertainties include the following items and the systematic error related to beam peak search is TBD.

| Systematic uncertainties (NOTE 6) | Value |
| --- | --- |
| Systematic error due to TRP calculation/quadrature (NOTE 4) | 0.00 |
| Influence of noise (23.45GHz <= f <= 32.125GHz) | 0.13 |
| Influence of noise (32.125GHz < f <= 40.8GHz) | [0.27] (NOTE 4) [0.20] (NOTE 5) |
| [Systematic error related to beam peak search] (NOTE 5) | [TBD] |

The systematic error related to beam peak search actually is the MU or TT factor arising from finite measurement grid. This contributing factor always exists as long as the measurement grid is not infinite. Though in STxMP testing there is no beam peak search procedure, finite measurement grid still contributes a deviation from ideal directions which is assumed in core requirements.

Regarding the mismatch between core requirements and test method for STxMP, the finite measurement grid actually only contributes a tiny level MU/TT but the test system limitation itself (test probes restricted to 2D xz plane, few AoA choices) contributes much more. However, such misalignment has not been taken into account yet.

Observation 1:	The mismatch between core requirements and test method for STxMP mainly arises from the test system limitation itself (test probes restricted to 2D xz plane, few AoA choices, etc.)

A quite flexible UE declaration can compensate such test system restrictions in some levels but could not eliminate thoroughly.  In practical products the two peak directions from two different panels may not fit into xz plane of test system no matter UE vendor declares any one Orientation from all 12 Orientation choices and declares any one AoA offset from all 5 AoA offset choices.

So it is more important to study the system uncertainties arising from the mismatch between core requirements and the test system restrictions.

Proposal 1:	Further study the system uncertainties arising from the mismatch between core requirements and the test system restrictions (test probes restricted to 2D xz plane, few AoA choices, etc.).

The system uncertainties arising from the mismatch between core requirements and the test system restrictions are expected much larger than the contribution from finite measurement grids. If this system uncertainties are to be included, the MU/TT from finite measurement grid is also partially addressed.

Observation 2:	The MU/TT from finite measurement grid can be merged into the system uncertainties arising from the mismatch between core requirements and the test system restrictions

3. 	Conclusion

Observation 1:	The mismatch between core requirements and test method for STxMP mainly arises from the test system limitation itself (test probes restricted to 2D xz plane, few AoA choices, etc.)

Proposal 1:	Further study the system uncertainties arising from the mismatch between core requirements and the test system restrictions (test probes restricted to 2D xz plane, few AoA choices, etc.).

Observation 2:	The MU/TT from finite measurement grid can be merged into the system uncertainties arising from the mismatch between core requirements and the test system restrictions

4. 	References

- R4-2502294    “Way Forward for [113][327] NR_FR2_OTA”, Qualcomm