3GPP TSG-RAN WG4 Meeting #114bis		R4-2503663

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.10.3.4

Source:		Samsung

Title:	Discussion on RRM core requirements of Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

In last RAN4#114 meeting, for RRM core requirements of Rel-19 NR-NTN in less than 5MHz, there are only two issues left with the values in square brackets in [1]. In this contribution, we provide our view on the two issues.

- Discussion

In RAN4#114 meeting, RAN4 achieved agreements in [1]. There are two issues with square brackets which are shown as below:

#### Issue 1-1-1: Measurement(SSB index detection requirements) Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR. #### Issue 1-1-2: CHO Agreements: - For CHO with L3 measurement, when Tmeasure is extended with [3] for intra-frequency case, T∆ in Tinterrupt can be extended to [3] Trs.

In current TN less than 5MHz scenario, for SSB based L3 measurement, time period for time index detection of both intra-frequency measurement without MG or with MG, 4 additional samples are needed. For inter-frequency measurement without MG or with MG, 3 additional samples are added.

In last meeting, some companies proposed to change the samples for intra-frequency measurement from 4 to 3 to align with the samples for inter-frequency due to the side condition of intra-frequency and inter-frequency are different. As we checked in existing spec, the side condition for intra-frequency and inter-frequency are the same. It is preferred to reuse the same values in TN less than 5MHz.

Proposal 1: For SSB index detection requirements, prefer to reuse the same values in TN less than 5MHz. The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-frequency and inter-frequency cases respectively.

For the second issue, some companies suggested to short the values for CHO with L3 measurement due to the long time. We understand the motivation of whole CHO with L3 measurement delay is long. The reason from proponent of decreasing T∆ from 3Trs to 2Trs is explained: UE can obtain some information when the cell was detected/measured during Tmeasure in coarse synchronization. We cannot fully agree about this explanation. It can be observed in HO, no matter the cell is known or unknown which means the cell has been identified well or not, the fine timing/frequency tracking T∆ are with the same extension. If review the history of the reason for extension in TN less than 5MHz, it is just due to the punctured SSB in frequency in simulation summary.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 2: For CHO with L3 measurement, keep T∆ in Tinterrupt to 3Trs and remove [].

- Conclusion

In this contribution, we provide our proposals for RRM core requirements for less than 5MHz for NR-NTN:

Proposal 1: For SSB index detection requirements, prefer to reuse the same values in TN less than 5MHz. The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-frequency and inter-frequency cases respectively.

Proposal 2: For CHO with L3 measurement, keep T∆ in Tinterrupt to 3Trs and remove [].

- Reference

[1] R4-2502589, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi