3GPP TSG-RAN WG4 Meeting #114bis		R4-2503666

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.22.3.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

In last RAN4#114 meetings, the WF was approved in [1]. There are still open issues in RRM requirements for CJTC reporting and asymmetric DL sTRP/UL mTRP scenarios. In this contribution, we continue to discuss the RRM requirements for asymmetric DL sTRP/UL mTRP scenarios in Rel-19 MIMO phase 5.

- Discussion

Asymmetric DL sTRP/UL mTRP

In RAN4#114 meeting, the agreement of RRM requirement for 2TA enhancement for asymmetric DL sTRP/UL mTRP is captured in [1] as:

Issue 2-1: Whether RAN4 need to consider the scenario when UE is/isn’t configured with pathloss offset? Agreement: For Scenario#1: RAN4 define the timing requirements for FR1, FFS on FR2. For Scenario#2, FFS on whether to reuse the legacy timing requirements (Rel-18 m-DCI two TA with updated or not). RAN4 common understanding: the RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs.” are currently only applied for Scenario #1. - Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s) - Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE expect to receive SSB from UL TRP(s). Note 1: the scenario 1 and 2 are based on RAN1 LS. Note 2: The above agreements apply when 2TA are configured.

There are two FFS bullets as above.

In last RAN4#114 meeting, RAN4 achieved the timing requirements for FR1 of Scenario#1. Although we proposed it should use the same timing requirements for FR2 of Scenario#2 as well, some companies want to further check whether there are some impacts due to RF discussion.

By our information, there is no agreement in RF session in last meeting.

R4-2503037 <Way forward>: RAN4 will decide between following options in the next meeting - Option 1: No UE RF impact of asymmetric DL sTRP/UL mTRP - Option 2: Adopt following dCR, where the companies are encouraged to propose wording improvements in the next meeting

Even the discussion is not related to Scenario#1, all the FFS parts or proposed modifications are for the case the UE is expect to receive SSB from UL TRPs.

Proposal 1: For Scenario#1, FR2, reuse the same timing requirements as that of FR1. For Scenario#2, since RAN4 sent LS to RAN1 in last meeting, wait for further progress. RAN4 cannot define anything new if no further RAN1 design.

For the TCI state switching delay requitements of Rel-19 asymmetric DL sTRP/UL mTRP scenarios, it is still open and the options are listed in [1] as below:

Issue 2-3: Whether to define UL TCI state activation requirement based on SRS? Way forward: Further discuss the two proposals and down-select one proposal in the next meeting: - Proposal 1 (Qualcomm, Huawei, MediaTek, Samsung, Apple, QC): No RRM impacts on TCI state switching requirement. - Proposal 3 (Xiaomi, Nokia): - Legacy DCI based UL TCI state activation requirement can be re-used for UL TRP. - RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline. - For DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. no need to define SRS based ULTCI state activation delay. - Further refine the wording for P3 in the WF, if needed

In last two meetings, we analyzed the UL TCI state switching based on SRS. Let’s emphasize our analysis again.

Firstly, let’s aligned with the assumption. 
In RAN1 agreement:

RAN1#117 agreement For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

According to RAN1 agreement, joint TCI states is not supported in FR2. Only separate TCI UL state can be supported in FR2.

For FR2, even if there can be expected SSB in Scenario#2, if the UL TCI state can be associated to SSB when there is on PL-offset is configured, it is just the legacy case and the requirements associated with DL RS have been specified in existing requirements in Rel-17/Rel-18.

In Proposal 3 in the WF of last meeting, it is proposed to “RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline”.

The first question is what is known TCI?

In uplink TCI, there are two types of RS. One if referenceSignal which we called source RS (SRS in FR2 above). The other is pathlossReferenceRS. In UL-TRP, the power control is using PL-RS offset + PL-RS in DL/UL TRP if PL offset is configured.

If the known condition of target TCI is defined, it can contain: known condition of source RS; known condition of PL-RS in another TRP and relationship with PL-RS offset.

The first challenge how to define known condition for SRS. For uplink TCI state, the only thing is the spatial info of the beams.

In legacy Rel-17 feMIMO discussion, the similar discussion on the known condition of source RS as SRS were discussed and finally, the conclusion is it is hard to define the known condition of SRS. From the SRS spatial, it depends on the Tx switching. For the known condition of PL-RS in another TRP, the legacy condition can be reused. Thus, the difficulty is still the known condition of SRS.

The second point is even no matter what is the known condition of SRS, “The legacy MAC CE based delay requirement can be used as baseline”. The legacy MAC CE based delay depends on the DL RS measurement if condition is met. It is totally related to DL RS (PL-RS) measurement. If the UL TCI state can be associated to SRS, there is no time for measurement and UL beam refinement. In other word, if there is the delay, the delay is corresponding to the MAC-CE decoding time.

Compare the difference between Rel-17 feMIMO such discussion and current Rel-19 discussion, there is no difference. By our understanding, it is the reason that why current requirements “There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS” and why we proposed to follow no requirements when UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS in this asymmetric DL sTRP/UL mTRP scenarios.

Proposal 2: RAN4 to decide:

- If UL TCI states is associated to SRS: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for for “CJTC reporting” and “asymmetric DL sTRP/UL mTRP scenarios” in Rel-19 MIMO phase 5:

Proposal 1: For Scenario#1, FR2, reuse the same timing requirements as that of FR1. For Scenario#2, since RAN4 sent LS to RAN1 in last meeting, wait for further progress. RAN4 cannot define anything new if no further RAN1 design.

Proposal 2: RAN4 to decide:

- If UL TCI states is associated to SRS: Follow same principle in Rel-17 which is no requirement for UE is requested to a separate UL TCI state.

- Reference

[1] R4-2502596, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek