3GPP TSG-RAN WG4 Meeting #114bis		R4-2503669

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:		7.29.4.1

Source:		Samsung

Title:	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In RAN4#114 meeting, we have already discussed all the RRM requirements of RedCap in NTN and achieved some agreements in [1]. There are still some open issues in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands.

- Discussion

##### Issue 1-2-3-1: Feature 25-1(parallelSMTC) Online discussion <Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

In order to achieve the better performance of power assumption of RedCap UE and the complexity of implementation, we prefer option 1 for 1 SMTC or 2 SMTCs not in parallel. The parallel multiple SMTCs parallel are for the parallel measurement of multiple LEO satellites. From the mobility, the RedCap UE can be required to measure two LEO satellites but the measurement time is not so tight, TDM is a good candidate solution to balance the measurement and the cost.

Proposal 1: For RedCap UE in NTN, support 2 SMTC not in parallel.

##### Issue 1-2-3-1a: The proximity distance between two SMTCs Ad-hoc agreement Agreement: - RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. - For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, - Option 1: No. RAN4 to reuse the legacy collision condition. - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e., - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

In existing spec for handheld NTN UE,

-	the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms.

From the RedCap UE perspective, the relaxed proximity can get better performance for complexity and processing.

Proposal 2: For RedCap UE in NTN, the proximity distance can be relaxed compared to the legacy NTN UE.

#### Issue 1-2-6: NCD-SSB <Way Forward>: - Proposal 1: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB. - Proposal 2: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

In previous meeting, RAN4 agreed: For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

Therefore, there is needed to add the clarification that the requirements are only applicable to CD-SSB.

Proposal 3: Add a clarification that the requirements are only applicable to CD-SSB.

#### Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN. - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused. - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE, - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples - Same delay requirements for 1Rx and 2Rx UEs. - Proposal 1c: - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements - For 1 Rx (e)RedCap NTN UE, RAN4 to - Option 1: Reuse the existing enhanced cell reselection measurement requirements - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined. - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Firstly, in legacy NTN requirements, enhanced RRM requirements for measurements is optional capability to perform fast detection/measure/evaluate. For RedCap UEs, we also think this enhanced RRM requirements are based on optional UE to support it.

The differences between those proposals are:

- For 1Rx and 2Rx RedCap UE, whether to use the same requirements.

- Whether to reuse the legacy value or extend by additional samples.

By our preference, this enhanced RRM requirements capability is optional for UE, we support to reuse the existing enhanced RRM requirements for both 1Rx and 2Rx RedCap UEs for simplicity.

Proposal 4: Reuse the existing enhanced RRM requirements for both 1Rx and 2Rx RedCap UEs.

#### Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline. - Proposal 2: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle. - Proposal 3: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, - For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands. - For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

For cell re-selection measurement requirements of inter-RAT E-UTRAN cells for RedCap in TN system, both 2Rx and 1Rx UEs have the same margin.

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS 38.304 [1] within (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

Proposal 5: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, for both 2Rx UE and 1Rx UE, reuse the existing requirements for measurements for inter-RAT E-UTRAN cells with TN carrier.

#### Issue 2-4-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1: RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

For accuracy requirements for L1/L3 measurement, compared to legacy requirement, there is no difference and no reason to change.

Proposal 6: For accuracy requirements:

- For 2Rx (e)RedCap UEs, reuse Rel-17 NTN requirements.

- For 1Rx (e)RedCap UEs, reuse the same relaxation for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP in TN:

  - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

  - L1-RSRP: relax by 3dB.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands:

Proposal 1: For RedCap UE in NTN, support 2 SMTC not in parallel.

Proposal 2: For RedCap UE in NTN, the proximity distance can be relaxed compared to the legacy NTN UE.

Proposal 3: Add a clarification that the requirements are only applicable to CD-SSB.

Proposal 4: Reuse the existing enhanced RRM requirements for both 1Rx and 2Rx RedCap UEs.

Proposal 5: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, for both 2Rx UE and 1Rx UE, reuse the existing requirements for measurements for inter-RAT E-UTRAN cells with TN carrier.

Proposal 6: For accuracy requirements:

- For 2Rx (e)RedCap UEs, reuse Rel-17 NTN requirements.

- For 1Rx (e)RedCap UEs, reuse the same relaxation for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP in TN:

  - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

  - L1-RSRP: relax by 3dB.

- Reference

[1] R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT