3GPP TSG-RAN WG4 Meeting#114bis		R4-2503673

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.24.1

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on system parameters for ambient IoT

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was agreed. This contribution continues to discuss the remaining open issues as follows..

Issue 2-1-4: Values of R2D bandwidth Agreement: - Define R2D transmission bandwidth configuration: - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS - FFS on other RB numbers - Further discuss the R2D channel bandwidth considering additional guard band. - If RAN1 has the agreement different than above, RAN4 will revisit the agreements Issue 2-1-5: R2D channel raster and channel spacing Agreement: - Do not define channel spacing for R2D transmission - Define channel raster for R2D transmission. - Details need further discussion. Issue 2-1-6: D2R bandwidth Agreement: - D2R channel bandwidth for A-IoT device - Define D2R channel bandwidth for A-IoT device considering the following aspects: - [1SB or] 2SB - D2R maximum data rate (depending on RAN1 conclusion) - SFO - FDMA, i.e., considering the different device 1 has different frequency shift - Reference Filter performance - Other aspects are not precluded

Discussion

Values of R2D bandwidth

According to RAN1’s agreements, whether to specify the maximum Btx,2D# of PRBs is up to RAN4. According to simulation results before RAN1, the more RBs, and the better its performance. In order to improve the performance better, RAN4 needs to specify the maximum Btx,2D#of PRBs. The transmission bandwidth could be 1RB, 2RB, 3RB, 4RB, 8RB and 16RB.

Agreement RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D. - For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value. - For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4 - Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Proposal 1: It is needed to specify the maximum Btx,R2D# of PRBs.

Proposal 2: Define R2D transmission bandwidth configuration: 1RB, 2RB, 3RB, 4RB, 8RB and 16RB with 15 kHz SCS.

In RAN4, channel bandwidth includes transmission bandwidth and guard band for co-existence study. In addition, guard band for SFO (i.e., 104 or 105ppm) needs to be considered. Guard band for SFO should be equal to 1/10 transmission bandwidth, guard band depends on co-existence study results. According to the WID, only D1T1-B needs to be considered, and NR UE is only outdoor in R19 WI stage, there is no co-existence problem. Hence, there is no need to define guard band for co-existence between NR and A-IoT system. Based on the above analysis, channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO. GBchannel=2*(1/10) transmission bandwidth. Channel bandwidth could be equal to GBchannel plus transmission bandwidth.

Proposal 3: Channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO. There is no need to consider guard band for co-existence study for R2D.

R2D channel raster and channel spacing

Similar to LP-WUR, regarding in-band deployment for A-IoT system, R2D can reuse channel raster as legacy NR channel raster. In addition, for standalone deployment of A-IoT system, R2D can also reuse channel raster as legacy NR channel raster.

Proposal 4: R2D channel raster can reuse legacy NR channel raster.

D2R bandwidth

In the last meeting, an agreement was agreed to define D2R channel bandwidth for A-IoT device considering the following aspects: 1SB or 2SB, maximum data rate, SFO, FDMA, reference filter performance and other options. For 2SB, according to RAN1#120 meeting’s agreement, for D2R, only 2SB (Double sideband) modulation is supported. Hence, only 2SB needs to be considered. For D2R maximum data rate, which is strongly with D2R transmission bandwidth, it depends on RAN1’s conclusion. For SFO, if the device does not support small frequency shift, guard band for SFO is equal to 2*(1/10) transmission bandwidth. If the device supports small frequency shift, guard band for SFO is equal to 2*(1/10) transmission bandwidth plus 2*(1/10) the largest small frequency shift. For FDMA, no matter the values of small frequency shift, the transmission bandwidth for D2R is still unchanged. Therefore, we don’t need to consider the factor like that FDMA, i.e., considering the different device 1 has different frequency shift. For reference filter performance, based on the architecture of device1 in TR for SI stage, there is no any reference filter in transmission related blocks in figure 1.Therefore, we don’t need to consider reference filter performance. Since there is no filter, we cannot suppress the power of 3rd harmonics and 5th harmonics interference. Based on [2], the power of 3rd and 5th harmonic reduces 9.5dB and 14dB compared to the power of main load. We cannot ignore it, especially for the 3rd harmonics interference. In our understanding, we need to consider the 3rd harmonics interference when defining D2R channel bandwidth for A-IoT devices.

Figure1: Architecture of device1

Proposal 5:  Define D2R channel bandwidth for A-IoT device considering the following aspects:

- 2SB

- D2R maximum data rate (depending on RAN1’s conclusion)

- SFO

- 3rd harmonics interference

Proposal 6: SFO for guard band for D2R is equal to 2*(1/10)*transmission bandwidth plus 2*(1/10)* the largest small frequency shift.

Conclusions

In this paper, we provide our views on system parameters for ambient IoT, we have made the following proposals.

Proposal 1: It is needed to specify the maximum Btx,R2D# of PRBs.

Proposal 2: Define R2D transmission bandwidth configuration: 1RB, 2RB, 3RB, 4RB, 8RB and 16RB with 15 kHz SCS.

Proposal 3: Channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO. There is no need to consider guard band for co-existence study for R2D.

Proposal 4: R2D channel raster can reuse legacy NR channel raster.

Proposal 5:  Define D2R channel bandwidth for A-IoT device considering the following aspects:

- 2SB

- D2R maximum data rate (depending on RAN1’s conclusion)

- SFO

- 3rd harmonics interference

Proposal 6: SFO for guard band for D2R is equal to 2*(1/10)*transmission bandwidth plus 2*(1/10)* the largest small frequency shift.

Reference

- RP-2502858, WF on requirements for system parameters and A-IoT device, CMCC, TSG RAN4 meeting #114