3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503687

Wuhan, China, 7th – 11st April, 2025

Source:	Huawei, HiSilicon

Title:	TP for TR 38.719-02-01 on introduction of CA_n28A-n71A

Agenda item:	6.3.3

Document for:	Approval

1 Background

This contribution provides text proposal on the NR band combination CA_n28A-n71A.

2 Text Proposal

---Start of changes---

5.x	CA_n28-n71

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n71A | CA_n28A-n71A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 CA_n28A-n71A UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n28 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 703 | 1406 | 2109 | 2812 | 3515 | MSD type |
| n71 | fLow | fHigh | 748 | 1496 | 2244 | 2992 | 3740 |  |
| DL1 | 617 | 652 | N/A | No | No | No | No | UL harmonic |
| DL23 | 1234 | 1304 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | No | No | N/A | No | N/A |  |
| DL4 | 2468 | 2608 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no MSD issue due to harmonic / harmonic mixing |  |  |  |  |  |
| UL/DL |  | n71 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 663 | 1326 | 1989 | 2652 | 3315 |  |
| n28 | fLow | fHigh | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 758 | 803 | N/A | No | No | No | No | UL harmonic |
| DL23 | 1516 | 1606 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 2274 | 2409 | No | No | N/A | No | N/A |  |
| DL4 | 3032 | 3212 | Yes | N/A | N/A | N/A | N/A |  |
| DL54 | 3790 | 4015 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no MSD issue due to harmonic / harmonic mixing |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range |  |  |  |  |  |  |  |  |
| Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. |  |  |  |  |  |  |  |  |
| When the gap between ULx and DLy frequency range is from 0Hz to n*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. |  |  |  |  |  |  |  |  |
| Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz |  |  |  |  |  |  |  |  |
| Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 CA_n28A-n71A UL/DL cross band isolation analysis

| Bands3 | n28 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 703 | 748 | 663 | 698 |
| fDL (MHz) | 758 | 803 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 30 | 5 | 20 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 673 | 778 | 643 | 718 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 643 | 808 | 623 | 738 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 613 | 838 | 603 | 758 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 583 | 868 | 583 | 778 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 553 | 898 | 563 | 798 |
| Analysis | The 2nd adjacent channel interference for UL band n28 may fall into the Rx band n71 |  | The 4th adjacent channel interference for UL band n71 may fall into the Rx band n28 |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: |  |  |  |  |
| - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. |  |  |  |  |
| - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency |  |  |  |  |
| - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible |  |  |  |  |
| Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges |  |  |  |  |
| while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n28-n71, the TIB,c and RIB,c values are given in the tables below, which has been specified in the spec.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n28-n71 | 1.1 | 1.1 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n28-n71 | 0.7 | 0.7 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

The MSD due to cross band isolation has been specified in the spec for this combo.

5.x.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements for CA_n28-n71.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n71A | 23 | +2/-3 |

5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n28 + Band n71 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n28 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 85 | 5 | 1366 | 1446 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 708 | 833 | 578 | 693 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2069 | 2194 | 2029 | 2144 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1411 | 1581 | 1241 | 1391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 10 | 170 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2772 | 2942 | 2692 | 2842 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2732 | 2892 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2089 | 1904 | 2329 | 2114 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 688 | 493 | 918 | 713 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3355 | 3540 | 3475 | 3690 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3395 | 3590 | 3435 | 3640 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, IMD3 and IMD5 will fall into the Rx frequency range of band n28 and n71.

5.x.2.3	REFSENS requirements

Three antenna RF architecture is assumed for CA_n28-n71.

Figure 5.x.2.3-1 Three-antenna RF architecture for CA_n28-n71

Since the Tx filter in one band can not provide the sufficient rejection of another Tx band, the PA reverse mixing interference dominate the MSD requirements. For example, the following table represents the different interference component for DL n71 IMD3 MSD when both band n28 and n71 transmit signals.

| No. | Component | IMD3(dBm/5MHz) |  |
| --- | --- | --- | --- |
|  |  | Main_@Antenna | Div_@Antenna |
| 1) | n28 PA Forward mixing interference | -143.6 |  |
| 2) | n71 PA Forward mixing interference | -79.55 |  |
| 3) | n28 PA Reserve mixing interference | -74.6 |  |
| 4) | n71 PA Reserve mixing interference | -47.6 |  |
|  | n71 Duplexer | -93.5 |  |
| 5) | n71 LNA via filters | -57.5 | -88.5 |
|  | n71 LNA via PCB coupling | -92 | -92 |
|  | Diplexer (L-H) | -121.5 | -142 |
|  | Antenna switch | -84 | -108.5 |
|  | From main path |  | -57.12 |
|  | Total | -47.12 | -57.12 |

The MSD value due to IMD3 can be specified below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n71 | n28 | 728 | 5 | 25 | 773 | 38 | FDD | IMD34 |
|  | n71 | 683 | 5 | 25 | 637 | N/A | FDD | N/A |
|  | n28 | 729 | 5 | 25 | 774 | N/A | FDD | N/A |
|  | n71 | 683 | 5 | 25 | 637 | 38 | FDD | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

Reference

[1]	RP-241833, “Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells”, Ericsson, ZTE, Huawei, HiSilicon