# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503692

# Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on UL CA_n5-n8 for non-concurrent of n8 UL and n5 DL

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In this meeting, RAN4 received the RAN2’s LS [1] with the following two questions:

Regarding whether to define UE capability signalling, RAN2 didn't reach consensus due to the lack of details on simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8. Please note that RAN2 only defines UE capability signalling for well-defined behaviour/features. RAN2 therefore would like to ask RAN4 the following questions for better understanding: - Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version? - If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

In this contribution, we’d like to provide the views on these two questions.

Discussion

During Rel-18 discussion for sub-1GHz band combinations captured in TR 38.872, there are three options for CA_n5-n8 implementation.

1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

For question 1, option 3 with dedicated filter in band n8 shown below can enable simultaneous Rx/Tx in partial UL frequency range of band n8.

Figure 1 CA_n5-n8 architecture using dedicated duplexers with partial range in n8 UL only

n5 duplexer	UL: 824 MHz – 849 MHz	DL: 869 MHz – 894 MHz;

n8 duplexer	UL: 904 MHz – 915 MHz	DL: 925 MHz – 960 MHz.

Based on the WF [2], RAN4 may discuss option 3 in Rel-20 or later.

Observation 1: for Q1, based on the outcome of TR 38.872, it’s possible for a UE to support CA_n5-n8 with simultaneous Rx/Tx in partial UL frequency range of band n8 by implementing dedicated filter in band n8. But this option may be discussed in Rel-20 or later.

For question 2, since only the dedicated filter solution (option 3) can enable the simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8, the restricted frequency range in band n8 UL and/or n5 DL is required to be indicated by UE. For example, UL partial frequency range in band n8 should be indicated by UE.

Observation 2: for Q2, In order to enable the simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 in partial frequency range, it requires additional UE capability or clarification to reflect the restricted frequency range in band n8 UL and/or n5 DL.

Additionally, apart from CA_n5-n8, CA_n26-n106 may have the similar issue that band n26 DL and band n106 UL can’t support con-current operation referring to the contribution [3].

Observation 3: apart from CA_n5-n8, RAN4 also identify the other case which has the similar issue, e.g. CA_n26-n106.

Based on the WF [4], it’s agreed that simultaneous Rx-Tx is mandatory without signaling for FDD-FDD band combinations by default, but special case like CA_n5-n8 need to clarify the non-concurrent operation between band n8 UL and n5 DL. From network deployment perspective, it is not expected that the scheduling restriction (non-concurrent operation between band n8 UL and n5 DL) is only captured in the NOTE of RAN4 specification. Thus, from forward compatibility perspective, it’s necessary to specify a UE capability to indicate the non-concurrent operation between band n8 UL and n5 DL.

Proposal 1: from forward compatibility perspective, it’s necessary to specify a UE capability to indicate the non-concurrent operation between band n8 UL and n5 DL.

Summary

Observation 1: for Q1, based on the outcome of TR 38.872, it’s possible for a UE to support CA_n5-n8 with simultaneous Rx/Tx in partial UL frequency range of band n8 by implementing dedicated filter in band n8. But this option may be discussed in Rel-20 or later.

Observation 2: for Q2, in order to enable the simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 in partial frequency range, it requires additional UE capability or clarification to reflect the restricted frequency range in band n8 UL and/or n5 DL.

Observation 3: apart from CA_n5-n8, RAN4 also identify the other case which has the similar issue, e.g. CA_n26-n106.

Proposal 1: from forward compatibility perspective, it’s necessary to specify a UE capability to indicate the non-concurrent operation between band n8 UL and n5 DL.

- References

[1] R2-2501572, “Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx”, RAN2

[2] R4-2417125, “WF on UL CA_n5-n8”, CATT, China Telecom, Huawei, HiSilicon, RAN4#112bis

[3] RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE

[4] R4-2502840, “WF on simultaneous Rx-Tx”, Huawei