# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503696

# Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the Rel-19 PC2 FDD RedCap UE

Agenda Item:	7.6.1

Document for:	Approval

Introduction

In the RAN106 meeting, the WI [1] for NR PC2 RedCap UE was revised to include the FDD PC2 feature. There are two aspects highlighted here. Firstly, RAN4 may need to discuss the band agnostic way for FDD PC2 RedCap UE. Secondly, it’s noted that wearable device doesn’t support HPUE in FDD bands.

- For FDD bands, specify RF requirements for PC2 RedCap UE - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes) - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item

In the RAN4#114 meeting, based on the WF [2], RAN4 reached some consensus on MPR/AMPR/REFSENS for HD-FDD/REFSENS for 2Rx FD-FDD requirements for PC2 RedCap UE in FDD bands. Only one remaining issue, i.e. 1Rx REFSENS in FD-FDD, need to be further discussed.

Issue 1-2-3: REFSENS for 1Rx in FD-FDD Agreement: - Further discuss the following formats for further evaluations - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR - Evaluate the values of ΔR band by band - Note: Other options are not precluded Way forward: It is encouraged for companies to provide analysis for the above options and give the preference in next meeting.

In this paper, we’d like to discuss this remaining issue for FDD PC2 RedCap UE.

Discussion

In last meeting, RAN4 has agreed the REFSENS requirements for 1Tx (26dBm) /2Rx with the following way forward.

Issue 1-2-2: REFSENS for 2Rx in FD-FDD Online agreement: - 2Rx RSD for 1Tx PC2 is provided for non-Redcap UE in Table 7.3.2-1c in TS38.101-1, and this requirement could apply to RedCap UE for channel BWs up to 20MHz.

Thus, it’s natural to consider option 2 as a baseline for further discussion. In theory, the ΔR1R can’t be higher than 3dB. It’s observed that the REFSENS (1Tx (26dBm) /1Rx) should follow the condition below.

REFSENS (1Tx (23dBm) /1Rx) < REFSENS (1Tx (26dBm) /1Rx) ≤ REFSENS (1Tx (26dBm) /2Rx) + ΔR1R (3dB)

Observation 1: the REFSENS (1Tx (26dBm) /1Rx) should follow the condition below.

REFSENS (1Tx (23dBm) /1Rx) < REFSENS (1Tx (26dBm) /1Rx) ≤ REFSENS (1Tx (26dBm) /2Rx) + ΔR1R (3dB)

Considering the worst case, the minimum REFSENS (1Tx (26dBm) /1Rx) can be specified as REFSENS (1Tx (26dBm) /2Rx) + ΔR1R or REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD. In other words, REFSENS (1Tx (26dBm) /2Rx) + ΔR1R is equal to REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD. And the logical diagram for band n3 20MHz REFSENS as an example can be found below.

Figure 1 the logical diagram about how to specify the REFSENS for 1Tx/1Rx architecture of FD-FDD PC2

No matter which option RAN4 choose to specify the final REFSENS (1Tx (26dBm) /1Rx), the same result of REFSENS (1Tx (26dBm) /1Rx) for different options can be expected.

Observation 2: No matter which option RAN4 choose to specify the final REFSENS (1Tx (26dBm) /1Rx), the same result of REFSENS (1Tx (26dBm) /1Rx) for different options can be expected.

Proposal 1: For 1Tx/1Rx architecture of FD-FDD PC2 RedCap, the minimum REFSENS (1Tx (26dBm) /1Rx) can be specified as REFSENS (1Tx (26dBm) /2Rx) + ΔR1R or REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD.

Summary

Observation 1: the REFSENS (1Tx (26dBm) /1Rx) should follow the condition below.

REFSENS (1Tx (23dBm) /1Rx) < REFSENS (1Tx (26dBm) /1Rx) ≤ REFSENS (1Tx (26dBm) /2Rx) + ΔR1R (3dB)

Observation 2: No matter which option RAN4 choose to specify the final REFSENS (1Tx (26dBm) /1Rx), the same result of REFSENS (1Tx (26dBm) /1Rx) for different options can be expected.

Proposal 1: For 1Tx/1Rx architecture of FD-FDD PC2 RedCap, the minimum REFSENS (1Tx (26dBm) /1Rx) can be specified as REFSENS (1Tx (26dBm) /2Rx) + ΔR1R or REFSENS (1Tx (23dBm) /1Rx) + 1Tx RSD.

- References

[1] RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom

[2] R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple

[3] TR 38.861, Study on high power UE (power class 2) for one NR FDD band