# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503698

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on VSAT requirements for NR NTN Ku band

Agenda Item:	7.11.4

Document for:	Approval

Introduction

In the RAN4#114 meeting, we have the following way forward [1] for Ku band VSAT.

- Issue 1-2-3: Minimum EIRP requirements - Companies are encouraged to provide views on the minimum peak EIRP requirement for the NTN Ku band in the next meeting.

In addition, RAN4 never discuss the minimum EIS requirements for Ku band VSAT. In this paper, we’d like to share our views on these RF requirements for Ku band VSAT.

Discussion

1) Minimum EIRP requirements:

Referring to the contribution [2], the minimum EIRP for Ku band VSAT could be derived as following:

EIRPmin=-174dBm/Hz+10log10BW+NF+SNR+IM-AntennaGainRx_SAN+FSPL+Atmosphereloss

Where:

Noise Figure is assumed as 3dB.

SNR: -1dB

IM: 2dB

AntennaGainRx_SAN: 58.5dB for 90 deg elevation and 55.5dB

FSPL: 32.44 + 20log(fc_MHz) + 20log(Distance_km)

Atmosphereloss: 2dB

Currently, in this release RAN4 only agreed to specify the following VSAT types which support both GSO and LEO scenarios.

- VSAT type 1-3 will not be specified for Ku bands based on the revised WID - The mobile VSAT support of the Ku bands will be added based on the existing mobile VSAT type 4 & 5 - The following changes can be the starting point NTN VSAT class; NTN VSAT type; Type description. Mobile VSAT; 43; Mobile VSAT communicating with GSO and LEO with mechanical steering antenna.. 52,3; Mobile VSAT communicating with GSO and LEO with electronic steering antenna.. NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD).

|  | GEO |  | GEO |  |
| --- | --- | --- | --- | --- |
| Elevation angle (Deg) | 30.00 | 90.00 | 30.00 | 90.00 |
| Distance (VSAT-SAN) | 38611.64 | 35786.00 | 38611.64 | 35786.00 |
| UL frequency | 14500.00 | 14500.00 | 14500.00 | 14500.00 |
| FSPL | 207.40 | 206.74 | 207.40 | 206.74 |
| Atmosphere loss (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN NF (dB) | 3.00 | 3.00 | 3.00 | 3.00 |
| UL BW (MHz) | 100.00 | 100.00 | 200.00 | 200.00 |
| Target SNR (dB) | -1.00 | -1.00 | -1.00 | -1.00 |
| Implementation Margin (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN Rx Gain (dBi) | 55.50 | 58.50 | 55.50 | 58.50 |
| VSAT Minimum EIRP (dBm) | 63.90 | 60.24 | 66.91 | 63.25 |

Observation 1: For FR2 numerology Ku band, the minimum EIRP should be larger than 67dBm to support max 200MHz BW.

Observation 2: For FR1 numerology Ku band, the minimum EIRP should be larger than 64dBm to support max 100MHz BW.

Proposal 1: RAN4 can discuss whether to specify different minimum EIRP requirements for FR1 numerology Ku band and FR2 numerology Ku band based on different maximum channel bandwidths.

2) Minimum EIS requirements:

From link budget perspective, the minimum EIS can be derived below.

EISmin= SAN_EIRPdensity + 10log(BW) – FSPL – Atmosphereloss

|  | GEO |  | GEO |  |
| --- | --- | --- | --- | --- |
| Elevation angle (Deg) | 30.00 | 90.00 | 30.00 | 90.00 |
| Distance (VSAT-SAN) | 38611.64 | 35786.00 | 38611.64 | 35786.00 |
| UL frequency | 14500.00 | 14500.00 | 14500.00 | 14500.00 |
| FSPL | 207.40 | 206.74 | 207.40 | 206.74 |
| Atmosphere loss (dB) | 2.00 | 2.00 | 2.00 | 2.00 |
| EIRP density (dBm/MHz) | 70 | 73 | 70 | 73 |
| DL BW (MHz) | 10 | 10 | 50 | 50 |
| VSAT Minimum EIS (dBm) | -127.76 | -124.1 | -120.77 | -117.11 |

From VSAT implementation perspective, the minimum EIS can be derived below.

EISmin = 10*log10(k)+10*log10(BW)+60+IM+Tar_SNR+G/T

Where:

k is Boltzmann constant, 1.38*10-23 J/K

BW is assumed as 50MHz.

IM is implementation margin 2dB

Tar_SNR is -1dB

If G/T for VSAT is larger than 1dB, then the minimum EIS is larger than -121.6dBm.

Proposal 2: RAN4 can consider to specify -121 dBm minimum EIS requirements for 50MHz BW and scale requirements for other channel bandwidths.

Summary

Observation 1: For FR2 numerology Ku band, the minimum EIRP should be larger than 67dBm to support max 200MHz BW.

Observation 2: For FR1 numerology Ku band, the minimum EIRP should be larger than 64dBm to support max 100MHz BW.

Proposal 1: RAN4 can discuss whether to specify different minimum EIRP requirements for FR1 numerology Ku band and FR2 numerology Ku band based on different maximum channel bandwidths.

Proposal 2: RAN4 can consider to specify -121 dBm minimum EIS requirements for 50MHz BW and scale requirements for other channel bandwidths.

- References

[1] R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[2] R4-2402521, Further discussion on UE RF Tx requirements for NTN in Ka-band, ZTE Corporation