# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503699

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirements for NR NTN HPUE

Agenda Item:	7.10.2.2.1

Document for:	Approval

Introduction

In the RAN4#114 meeting, we have the following way forward [1].

Issue 2-1: NR-NTN PC2 Agreement: - Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible - 1Tx handheld - 2Tx handheld - 1Tx non-handheld - 2Tx non-handheld - If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19 Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed Issue 2-3: NR-NTN PC1 Agreement: - RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19. Issue 2-4-1: PC2 ACLR Agreement: - PC2 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-2: PC1.5 ACLR Agreement: - PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Issue 2-4-3: PC1 ACLR Agreement: - PC1 ACLR is [31 or 37] dBc for NR-NTN UE - Further study averaging value, i.e. 34. Issue 2-5: SEM Agreement: - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. - Further discuss whether and how to capture the FCC 47 CFR part 25 specification in clause 202 Issue 2-6: MPR/A-MPR Agreement: - For PC 2 and PC 1.5 MPR: - Option 1: Existing MPR requirements for PC2 and PC1.5 in TS 38.101-1 apply, if PC2/PC1.5 ACLR reused TN PC2/PC1.5 ACLR as 31 dBc. - Option 2: Aim for MPR analysis for PC2 and PC1.5 if PC/PC1.5 ACLR agreed as 30 dBc. - For PC 1 MPR: - Option 1: If PC1 ACLR reused TN PC1 ACLR as 37 dBc: - Option 1-1: Existing MPR requirements for PC1 in TS 38.101-1 applies - Option 1-2: More analysis is needed to revisit the MPR requirements from TN spec. - Option 2: Aim for MPR analysis for PC1 for other ACLR values [31 or 34] dBc - Aim for MPR/A-MPR analysis in #114bis for different power classes - For PA model calibration: - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage 28dBc - IQ image:28 dBc - CIM3: 60 dBc - ACLR: PC2 [31]dB, PC1.5 [31]dB, PC1 [31]dB - SEM: existing PC3 SEM - EVM: 17.5%

Based on the WF in last meeting, the remaining issue is to discuss MPR requirements and decide ACLR value (30 or 31dB). In this paper, we’d like to share our views on these Tx RF requirements.

Consideration about spec architecture

As RAN4 has agreed to consider the following RF architectures for different power classes, RAN4 may need to discuss how to capture the RF requirements for different RF architectures for different power classes.

|  | 1Tx | 2Tx | 4Tx |
| --- | --- | --- | --- |
| PC2 | 1Tx for both handheld and non-handheld UE | 2Tx for both handheld and non-handheld UE | N/A |
| PC1.5 | N/A | 2Tx for non-handheld UE | N/A |
| PC1 | 1Tx for non-handheld UE | N/A | 4Tx for non-handheld UE |

In current TS 38.101-5, only the RF requirements for 1Tx PC3 were defined. In addition, NR NTN UE may not support the UL MIMO. Thus, RAN4 need to create the new suffix for TxD requirements under the assumption of the RF architectures more than 1Tx.

Observation 1: RAN4 need to create the new suffix for TxD requirements under the assumption of more than 1Tx.

SEM requirements

Based on the analysis [2] in last meeting, the requirements specified in FCC 47 CFR part 25 specification in clause 202 are shown below.

| Center Frequency of assigned channel to out-of-band region | Attenuation [dB] | Measurement BW [kHz] | NOTE |
| --- | --- | --- | --- |
| 0.5B - B | 25 | 4 | 1dBm/4kHz for PC2 |
| B – 2.5B | 35 | 4 | -9dBm/4kHz for PC2 |
| >=2.5B | 43+10log10(W) | 4 | -13dBm/4kHz |

It can be observed that the FCC mask requirements are more relaxed than the existing 3GPP SEM requirements. Based on the discussion in last meeting, it’s agreed to apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline.

Observation 2: the FCC mask requirements are more relaxed than the existing 3GPP SEM requirements.

MPR evaluation VS ACLR

In the last meeting, we have two options for ACLR, i.e. 30dB or 31dB. The MPR evaluation can be found below.

figure 1 QPSK DFT-s-OFDM MPR comparison between 30dB and 31dB ACLR

Based on the simulation results, the MPR difference for QPSK DFT-s-OFDM between 30dB ACLR and 31dB ACLR is very small.

RAN4 could consider to reuse the existing TN 1Tx PC2 MPR requirements for 1Tx PC2 NR NTN UE, as there is no big difference for the MPR simulation results between 30dB and 31dB ACLR.

Proposal 1: RAN4 could consider to reuse the existing TN 1Tx/2Tx PC2 MPR requirements for 1Tx/2Tx PC2 NR NTN UE.

From UE implementation perspective, 30dB or 31dB ACLR requirements may not cause big implementation difference. If RAN4 specify the relaxer ACLR requirements, UE may have some margin to adjust the PA efficiency.

Proposal 2: for NR NTN PC2 HPUE, it’s proposed to specify 30dB/31dB ACLR.

Summary

Observation 1: RAN4 need to create the new suffix for TxD requirements under the assumption of more than 1Tx.

Observation 2: the FCC mask requirements are more relaxed than the existing 3GPP SEM requirements.

Proposal 1: RAN4 could consider to reuse the existing TN 1Tx/2Tx PC2 MPR requirements for 1Tx/2Tx PC2 NR NTN UE.

Proposal 2: for NR NTN PC2 HPUE, it’s proposed to specify 30dB ACLR.

- References

[1] R4-2502954, WF on HPUE requirments for NR-NTN, Samsung

[2] R4-2501458, Discussion on Tx requirements for NR NTN HPUE, Huawei, HiSilicon