# 3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503700

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rx requirements for NR NTN HPUE

Agenda Item:	7.10.2.2.2

Document for:	Approval

Introduction

In the RAN4#113 meeting, the following way forwards were captured in the Tdoc [1].

Issue 3-1-2: RSD Requirements for PC2 on n254 Agreement: For PC2 on n254: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx Issue 3-1-3: RSD requirements for PC2 on n255 Way forward: For PC2 on n255: Further discuss the follow assumptions - Option 1: Duplex isolation as 50~52; PA interference as -107 dBm/Hz (P2/R4-2419073) - Option 2: Duplex isolation as 51; PA interference as -117 dBm/Hz (O1/R4-2419540) Issue 3-1-4: RSD requirements for PC2 on n256 Way forward: For PC2 on n256: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx as starting point - To be confirmed by further discuss the following assumptions: - Option 1: Duplex isolation as 50~52; PA interference as -117.8 dBm/Hz - Option 2: Duplex isolation as 51; PA interference as -124 dBm/Hz Issue 3-4: Others general proposals w.r.t RSD Way forward: - Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

In the RAN4#114 meeting, RAN4 conclude that there is no RSD for PC2 NR NTN HPUE on both band n256 and n254 based on the way forward [2]. The open issues are how to specify the RSD requirements for PC2 on band n255 and whether and/or how to specify the RSD for PC1 and PC1.5 non-handheld UE.

In this paper, we’d like to share our views on how to address these open issues.

Discussion

For band n255, if the default Tx-Rx separation is assumed, the -117dBm/Hz PA interference can be assumed. In addition, the 52dB duplexer isolation and 54dB receiver IIP2 are assumed. Based on these assumptions, the RSD evaluations for both 1Tx/2Rx and 2Tx/2Rx RF architectures are provided as Annex A.1.

|  | 1Tx RF architecture | 2Tx RF architecture |
| --- | --- | --- |
| 20MHz RSD | 1.4dB | 2.5dB |

Proposal 1: To specify RSD for band n255 under both 1Tx/2Rx and 2Tx/2Rx RF architectures below if the default Tx-Rx separation is assumed.

|  | 1Tx RF architecture | 2Tx RF architecture |
| --- | --- | --- |
| 20MHz RSD | 1.4dB | 2.5dB |

For the PC1 and PC1.5, only the non-handheld UE is assumed in this Rel-19. For FWA form factor terminals, there is no restriction on the filter implementation, thus good isolation performance can be expected. As the starting point, it’s proposed not to specify RSD requirements for PC1.5 and PC1.

Proposal 2: As the starting point, it’s proposed not to specify RSD requirements for PC1.5 and PC1.

Summary

Proposal 1: To specify RSD for band n255 under both 1Tx/2Rx and 2Tx/2Rx RF architectures below.

|  | 1Tx RF architecture | 2Tx RF architecture |
| --- | --- | --- |
| 20MHz RSD | 1.4dB | 2.5dB |

Proposal 2: As the starting point, it’s proposed not to specify RSD requirements for PC1.5 and PC1.

- References

[1] R4-2420403, WF on NR NTN HPUE, Samsung

[2] R4-2502954, WF on HPUE requirments for NR-NTN, Samsung

- Annex

- A.1 The 20MHz PC2 RSD calculations for 1Tx and 2Tx in band n255 with default Tx-Rx separation

table 1 20MHz RSD calculation for 1Tx in band n255

|  | parameters | n255 main path | n255 diversity path |
| --- | --- | --- | --- |
| transmit power for n255, dBm |  | 26 |  |
| Tx band n255 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n255 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| n255 receival singal at ANT port, dBm |  | -22 | -36 |
| n255 Rx filter rejection at 1626.5~1660.5MHz, dB | 50 |  |  |
| signal After n255 Rx filter, dBm |  | -20 | -38 |
| Typical receiver IIP2, dB | 54 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -94 | -130 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -90 | -126 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n255 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -92.45 | -92.45 |
| n255 PA noise PSD at PA output port, dBm/Hz | -117 |  |  |
| n255 PA noise PSD at PA output port, dBm/RxBW |  | -44.45 |  |
| n255 Tx filter rejection at 1525~1559MHz, dB | 50 |  |  |
| n255 PA noise power at Rx n256 ant port at 1525~1559MHz, dBm |  | -94.45 | -104.45 |
| Total noise level at ANT port |  | -87.15 | -92.18 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -88.15 | -93.18 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -92.37 |  |  |
| MSD | 1.43 |  |  |

table 2 20MHz MSD calculation for 1Tx in band n255

|  | parameters | n255 main path | n255 diversity path |
| --- | --- | --- | --- |
| transmit power for n255, dBm |  | 23 | 23 |
| Tx band n255 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n255 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| n255 receival singal at ANT port, dBm |  | -25 | -25 |
| n255 Rx filter rejection at 1626.5~1660.5MHz, dB | 50 |  |  |
| signal After n255 Rx filter, dBm |  | -23 | -23 |
| Typical receiver IIP2, dB | 54 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -100 | -100 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -96 | -96 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n255 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -92.45 | -92.45 |
| n255 PA noise PSD at PA output port, dBm/Hz | -117 |  |  |
| n255 PA noise PSD at PA output port, dBm/RxBW |  | -44.45 | -44.45 |
| n255 Tx filter rejection at 1525~1559MHz, dB | 50 |  |  |
| n255 PA noise power at Rx n256 ant port at 1525~1559MHz, dBm |  | -94.45 | -94.45 |
| Total noise level at ANT port |  | -89.28 | -89.28 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -90.28 | -90.28 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -91.29 |  |  |
| MSD | 2.51 |  |  |