3GPP TSG-RAN WG4 Meeting #114bis	R4-2503701

Wuhan, China April 07 – April 11, 2025

Source:	Tejas Networks

Title:	RRM core requirement and testing framework for beam management

Agenda item:	7.19.3

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#114 in the scope of Rel-19 WI. Agreements achieved during this meeting are summarized in the WF [1] and the ad-hoc minutes [2].

In this paper, we provide our views on issues related to requirements for beam prediction.

Discussion

Relative RSRP accuracy

In the previous meeting (RAN4 #114) RAN4 discussed about the definition of relative RSRP accuracy and following options were discussed.

Issue 3-1: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others

RAN4 agreed on the definition of absolute RSRP accuracy in RAN4 #112bis meeting as follows:

The absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. Here, the index i may correspond to any beam index within the top-K beams based on the ground truth L1-RSRP.

The absolute RSRP accuracy metric provides insight into how much the predicted RSRP values deviate from the actual ground truth RSRP values. It is a critical parameter for evaluating the performance of AI/ML-based beam prediction.

Regarding the relative RSRP accuracy, several options have been proposed:

- Option 1: The ground truth of L1-RSRP of the predicted beam index i – the ground truth of L1-RSRP of the Top-1 ideal beam index j.

This option compares the ground truth RSRP of the predicted beam with the ground truth RSRP of the best beam. However, this approach does not directly evaluate the predicted results in a relative manner. Additionally, if the gap between the best beam and the second-best beam is large, the UE might fail the test, even if the prediction is reasonably accurate.

- Option 2: Predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where beam index n has the largest RSRP among the predicted beams.

This option compares the predicted RSRP values of different beams, focusing on the deviation from the best-predicted beam. However, the predicted RSRP values can be influenced by factors such as beam direction, which may introduce variability.

- Option 3: The RSRP difference between the predicted L1-RSRP of different beams.

This option is similar to Option 2 but is more general in nature, as it does not explicitly reference the best-predicted beam.

Observation 1: If the gap between the best beam and the second-best beam is large, the UE may fail to meet any of the requirements in the test.

Instead of relying solely on the ground truth or predicted RSRP values, a more comprehensive approach would be to evaluate the absolute RSRP accuracy for each predicted beam and then compare these values across the top-K predicted beams. This approach provides a clearer understanding of how accurately the beams are predicted relative to the best-predicted beam. The definition could be as follows:

Relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

This method ensures that the relative accuracy is evaluated based on the deviation of predicted values from the ground truth, while also considering the performance of the best-predicted beam. It provides a more robust and meaningful metric for assessing AI/ML-based beam prediction performance.

A similar discussion is taking place in RAN4 related to the study item NR AIML Mob, where a consensus was reached on the definition of relative RSRP accuracy as follows:

Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam 1 – reported RSRP of cell 2/beam 2) – (ground truth RSRP of cell 1/beam 1 – ground truth RSRP of cell 2/beam 2).

- With a minor rearrangement of the above expression, we obtain the following expression, which closely resembles the one we proposed above for relative RSRP accuracy for AI/ML-based beam prediction.

- Predicted relative RSRP accuracy = (Ground truth of RSRP of cell 2/beam2 - reported RSRP of cell 2/beam2) – (Ground truth of RSRP of cell 1/beam1- reported predicted RSRP of cell 1/beam1)

Predicted relative RSRP accuracy = Absolute L3 predicted RSRP accuracy of cell 1/beam 1 - Absolute L3 predicted RSRP accuracy of cell 2/beam 2.

Proposal 1: The definition should capture the idea of relative L1-RSRP accuracy, not relative L1-RSRP values.

Proposal 2: The relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

UE Rx beam knowledge

To accurately predict the L1-RSRP of the Tx beams, it would be beneficial for the UE to know the Rx beam paired with each predicted Tx beam. However, if the UE does not have prior knowledge of which Rx beam to use for a new Tx beam direction (due to a change in the TCI state), the UE would need to detect and adapt its Rx beam to align with the new Tx beam. While having knowledge of the Rx beam paired with the predicted Tx beam would help improve prediction accuracy, this knowledge should be internal to the UE and its implementation.

Observation 2: An unknown TCI state change can result in increased delay during the state transition.

Proposal 3: TCI state associated with the predicted beam should be known.

Beam prediction KPIs

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered Option 1: RSRP accuracy Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x Option 4: combinations of above options Agreement: If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

The agreed definition for absolute RSRP accuracy is given section 2.1 above.

The above definition of RSRP accuracy, ensure the deviation of all the top-K predicted beam is within a limit. Which is a necessary test metric to ensure reliable beam predictions.

Proposal 4: RSRP accuracy should be one of the KPIs.

Both Top-K/1 (%) and Top-1/K (%) from Option 2 use similar approaches, evaluating the probability that either the Top-1 strongest beam is part of the Top-K predicted beams, or the Top-1 predicted beam is one of the Top-K strongest beams. However, when K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams—and vice versa—increases. This makes it easier for the test to pass and reduces the ability to validate the reliability of the beam prediction. Therefore, it is important to keep K small to maintain meaningful validation. Furthermore, these metrics only ensure the presence of the Top-1 beam within the set of predicted beams, which may limit the network’s ability to select the best beam, particularly when the top beam cannot be allocated. To better address this, update option 2 such that more than one strongest beam is part of the top-K predicted beams.

Observation 3: When K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams increases.

Proposal 5: For Option 2, the value of K should be kept small to ensure more meaningful validation and reduce the likelihood of the Top-1 strongest beam easily fitting into the Top-K predicted beams.

Proposal 6: Updated Option 2: Top N/K (%): Top-N/K (%): The percentage of the Top-N predicted beams that are among the Top-K strongest beams.

If the threshold for RSRP accuracy is adequately maintained, the deviation of all predicted beams from the ground truth relative to the threshold has already been verified. Option 3 verifies the prediction accuracy of the Top-1 predicted beam within a specified error margin (x dB) compared to the RSRP of the strongest beam, which is somewhat redundant in relation to the RSRP accuracy definition. However, it is important to note that it also does not fully address cases where multiple top beams could be valid. Furthermore, it is essential to emphasize that, for the Top-K predicted beams, consistency in both RSRP accuracy and beam prediction accuracy should be maintained. Consistency means that the list of predicted beams should contain more than one potential option for the network to select from and must align with the actual beam strength order. This allows the network to make informed decisions, especially in cases where the strongest beam is not available or assignable.

Observation 4: Option 3 alone is insufficient, as it only validates a single beam.

Proposal 7: RSRP accuracy should be included as one of the KPIs, combined with the updated Option 2 and Option 3.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: If the gap between the best beam and the second-best beam is large, the UE may fail to meet any of the requirements in the test.

Observation 2: An unknown TCI state change can result in increased delay during the state transition.

Observation 3: When K is large, the likelihood of the Top-1 strongest beam becoming part of the Top-K predicted beams increases.

Observation 4: Option 3 alone is insufficient, as it only validates a single beam.

Proposal 1: The definition should capture the idea of relative L1-RSRP accuracy, not relative L1-RSRP values.

Proposal 2: The relative RSRP accuracy for AI/ML-based beam prediction = Absolute L1-RSRP Accuracy of the predicted beam index i – Absolute L1-RSRP Accuracy of the best-predicted beam.

Proposal 3: TCI state associated with the predicted beam should be known.

Proposal 4: RSRP accuracy should be one of the KPIs.

Proposal 5: For Option 2, the value of K should be kept small to ensure more meaningful validation and reduce the likelihood of the Top-1 strongest beam easily fitting into the Top-K predicted beams.

Proposal 6: Updated Option 2: Top N/K (%): Top-N/K (%): The percentage of the Top-N predicted beams that are among the Top-K strongest beams.

Proposal 7: RSRP accuracy should be included as one of the KPIs, combined with the updated Option 2 and Option 3.

- References

- R4-2502856, WF on AI/ML air interface part 1, Qualcomm Incorporated, Athens, Greece, 17th – 21st February, 2025

- R4-2503012, AI/ML ad-hoc meeting minutes-1, Qualcomm Incorporated, Athens, Greece, 17th – 21st February, 2025

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18, ver. 18.0.0.