3GPP TSG-RAN WG4 Meeting #114bis	R4-2503702

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Low Band CA operation

Agenda item:	7.7.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM issues for low band CA via switching between FDD and SDL CC based on the WF [1].

Discussion

Assumptions for LB CA via switching

In previous meeting, following agreement has been achieved for assumption for LB CA

Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting.

Regarding SSB-based or SSB-less SCell operation, it is better to specify both requirement. SSB-based SCell operation is default operation, furthermore, deployment assumption (e.g. co-located and synchronized network for both carriers) can be leveraged for SSB-less SCell operation.

- Proposal 1: RAN4 to specify SSB-based SCell operation as baseline, and also specify SSB-less SCell operation.

Interruption requirements

Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM.

In the last meeting, switching period values were agreed in RF session, but there is ongoing discussion on time mask. Depending on the time mask discussion in RF session, interruption requirements might not be required. Therefore, interruption requireiment should be discussed after the RF conclusion on time mask issue is reached.

- Proposal 2: The interruption requirements should be discussed after the RF conclusion on time mask issue is reached discussion.

SDL SCC related RRM requirements

Activated SDL SCell measurement requirement

Activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application.

If SDL SCell transmits SSB and the SCell is activated, UE need to perform L3 measurement for the SCell. Even if UE switches to SCell based on switching pattern, SCell L3 measurement should be performed with measurement gap since single antenna is assumed in Low-Low band CA. Therefore, regardless of switching pattern, activated SDL SCell L3 measurement requirements could be based on measurement gap.

- Proposal 3: Activated SDL SCell L3 measurement requirements could be based on measurement gap.

Deactivated SCell measurement requirement

De-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used.

Before the discussion on deactivated SDL SCell measurement, RAN4 needs to discuss when the switching pattern configured by RRC would apply. The main purpose of switching to SDL SCell is to receive data signals, so switching to deactivated SDL SCell is not usefull and would impact PCell measurements behavior. Therefore, switching to deactivated SDL SCell by switching pattern should be precluded.

- Proposal 4: RAN4 to discuss when the switching pattern configured by RRC would be applied. To reduce unneccesary SDL switching, the switching pattern cannot be applied to deactivated SDL SCell.

If the switching pattern is not applicable before SDL SCell is activated, measurement gap can be configured for deactivated SCell L3 measurement. For example, when a UE is required to measure deactivated SDL SCell for LB CA, the UE can measure SDL SCell within the MG. It means that no switching is required when the SDL SCell is deactivated. It is natural that no switching is required when the SDL SCell is not activated, since it may lead throughput loss on PCell. In our view, switching can be performed after SCell is activated. In order to measure the deactivated SCell, the UE may report whether MG is required for LB CA or not. NW can configure legacy MG pattern taken into account measCycleSCell for deactivated SDL SCell.

- Proposal 5: Deactivated SDL SCell measurement can be performed within the measurement gap if no switching pattern is applied to deactivated SDL SCell.

Other SDL SCell requirements

Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA

For SDL activation delay, after SCell activation command is received, the UE has to switch to SDL SCell after the HARQ-ACK feedback + MAC-CE processing to complete SCell activation. After switching, the UE would perform SDL SCell activation. If the UE settles at SDL SCell when SCell activation, everything would be fine. However, if the UE switches to PCell during SCell activation by semi-static switching pattern configured by RRC, it may not be completed. In that sense, a certain SCell activation window(e.g., Tactivation_window) that can guarantee the UE to complete SCell activation is required as shown in Figure 1. In the SCell activation window, the UE is required to complete the SCell activation, and shall be able to switch back to PCell. The semi-static switching pattern can be applied after SCell activation window.

Figure 1 Example of SCell activation based on single antenna equipped UE

- Proposal 6: RAN4 to specify SCell activation window, which guarantees the UE to complete SCell activation while switching on SDL SCell, and semi-static switching pattern can be applied after SCell activation window.

Regarding BFD/CBD, there can be an issue when the UE declare beam failure in SDL SCell. Usually, a UE evaluate DL beam quality of serving cell utilizing RS set in q0. If every the DL beam quality of RS in q0 are below Qout_LR, physical layer sends BFI (beam failure indication) to higher layer. If the certain number of BFI is occurred within cerain time window, the UE can declare beam failure, and try to transmit PRACH applying one of the candidate beams. After transmit PRACH, the UE can follow initial access procedure. The problem is when the UE needs to transmit PRACH, there is no UL symbol in SDL SCell.

- Observation 1: When beam failure is declared in SDL SCell, no UL resource exists to transmit PRACH

We think it is natural to switch back to PCell in case of beam failure. For instance, the UE can switch to PCell right after the UE declares beam failure, or before transmitting PRACH considering RF tunning time. Furthermore, in order to perform beam failure recovery process properly, it would be helpful for the UE to pause the semi-static switching pattern during initial access, and resume the pattern when beam failure recovery is completed. Hence, RAN4 need to specify requirement for beam failure recovery procedure in SDL SCell.

- Proposal 7: RAN4 to specify requirement or UE behaviour for beam failure recovery procedure in SDL SCell

PCell related RRM requirements

Collision between PCell SMTC and SCell DL reception

In general, NW can configure PCell SMTC, and RS for RLM/L1 irrelevant to SCell. Although semi-static pattern has not been specified yet, there can be a case that PCell SMTC/RS can be overlapped with SCell DL reception as illustrated in Figure 2, where periodic RS in PCell, and PCell SMTC are collided with sem-static switching pattern in SDL SCell. In Rel-19 LB CA, if the switching pattern is SDL SCell, a UE cannot perform measurement on PCell, which may degrade mobility performance. Of course, NW can avoid such situation by configuration/re-configuration, however, we are not sure NW can handle every perioidic RS or configuration. RAN4 needs to discuss whether to switch to PCell or maintain operation in SCell. Based on the conclusion, clear UE behavior should be defined in case of collision between PCell measurement and SCell data reception.

Figure 2 Example of collision between SMTC and SCell DL reception

- Proposal 8: RAN4 to define UE behavior or related requirements in case of collision between PCell measurement and SCell data reception based on RAN1 semi-static switching pattern design

In order to handle collision, below options are considered before RAN1’s conclusion for semi-static switching pattern design:

- Option 1) UE does not expect to overlap with semi-static switching pattern in SDL SCell with SMTC

- Option 2) consider scaling factor Kp, e.g.,

- Window W = max(SMTC, MGRP, switching pattern periodicity )

- Ntotal = total number of SMTC within W

- Navailable = the number of SMTC not overlapped MG and switching pattern(SCell DL duration)

- Kp = Ntotal / Navailable

- Proposal 9: In order to handle the collision, following options can be considered before RAN1’s conclusion for semi-static switching pattern

- Option 1) UE does not expect to overlap with semi-static switching pattern in SDL SCell with SMTC

- Option 2) consider scaling factor Kp, e.g.,

- Window W = max(SMTC, MGRP, switching pattern periodicity )

- Ntotal = total number of SMTC within W

- Navailable = the number of SMTC not overlapped MG and switching pattern(SCell DL duration)

- Kp = Ntotal / Navailable

Conclusion

In this contribution, we provide our initial views on RRM issues for low band CA via switching between FDD and SDL CC, and we propose

- Proposal 1: RAN4 to specify SSB-based SCell operation as baseline, and also specify SSB-less SCell operation.

- Proposal 2: The interruption requirements should be discussed after the RF conclusion on time mask issue is reached discussion.

- Proposal 3: Activated SDL SCell L3 measurement requirements could be based on measurement gap.

- Proposal 4: RAN4 to discuss when the switching pattern configured by RRC would be applied. To reduce unneccesary SDL switching, the switching pattern cannot be applied to deactivated SDL SCell.

- Proposal 5: Deactivated SDL SCell measurement can be performed within the measurement gap if no switching pattern is applied to deactivated SDL SCell.

- Proposal 6: RAN4 to specify SCell activation window, which guarantees the UE to complete SCell activation while switching on SDL SCell, and semi-static switching pattern can be applied after SCell activation window.

- Observation 1: When beam failure is declared in SDL SCell, no UL resource exists to transmit PRACH

- Proposal 7: RAN4 to specify requirement or UE behaviour for beam failure recovery procedure in SDL SCell

- Proposal 8: RAN4 to define UE behavior or related requirements in case of collision between PCell measurement and SCell data reception based on RAN1 semi-static switching pattern design

- Proposal 9: In order to handle the collision, following options can be considered before RAN1’s conclusion for semi-static switching pattern

- Option 1) UE does not expect to overlap with semi-static switching pattern in SDL SCell with SMTC

- Option 2) consider scaling factor Kp, e.g.,

- Window W = max(SMTC, MGRP, switching pattern periodicity )

- Ntotal = total number of SMTC within W

- Navailable = the number of SMTC not overlapped MG and switching pattern(SCell DL duration)

- Kp = Ntotal / Navailable

Reference

[1] R4-2502588, “WF on RRM requirements for NR_LBCA_Sw” Apple