3GPP TSG-RAN WG4 Meeting #114bis	R4-2503703

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for ATG CA operation

Agenda item:	7.12.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on open issues for ATG with DL CA operation.

Discussion

General

UE Antenna Type signaling

Agreement: - To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed - FFS for the details.

In the last RAN4 meeting, RAN4 agreed to introduce Case 4 antenna type. In Rel-18, UE indicates the capability whether to support antenna array per band. For inter-band CA, if antennaArrayType is to set ‘supported’ for each band, the UE needs to indicate common Rx beam or separate Rx beam for the band combination like FR2 CBM and IBM.

- Proposal 1: Introduce UE capability to indicate the supported Rx beam type for inter-band CA per BC.

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-x | Rx beam type in inter-band CA | Indicate the support of common Rx beam or separate Rx beam in inter-band CA support antennaArrayType for each band | yes | If UE does not support the capability, performance cannot be guaranteed. | Per BC | No | FR1 only | Mandatory with capability signaling for UE supports NR communication via ATG |

Measurement procedure

Deactivated SCell measurement

Agreement: - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed - FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

For deactivated SCell measurement, there is no L1 measurement, so RAN4 agreed not to introduce L1/L3 measurement sharing factor for ATG UE antenna types except Case 4 antenna type. For Case 4 antenna type, while deactiavtaed SCell measurement is performed, PCell L1 RSRP measurement could be overlapped. However, the deactivated SCell will be the co-located with PCell since there is only one deactivated SCell for ATG DL CA. Therefore, L1/L3 measurement sharing factor is not needed for Case 4 antenna type.

- Proposal 2: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed for deactivated SCell measurement.

Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

Agreement: - For ATG UE antenna types except Case 4 - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured. - FFS for ATG Case#4 UE

For ATG Case 4 antenna type UE, even if the UE has common Rx beam, the signals from each CC are processed separately. Therefore, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- Proposal 3: For ATG Case 4 antenna type in inter-band CA, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

Sharing factor between L1 measurement and RRM measurement

Agreement: Agree on the following Proposal 1 for the intra-frequency measurement and L1-RSRP measurement, and apply the similar principle to other related requirements. - Proposal 1: - The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example): For UE with the antenna array, - Klayer1_measurement=1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement; Klayer1_measurement=1.5, otherwise.

For Case 4 antenna type UE, RAN4 agreed to define MRTD as 33usec. It means that the reception time difference between PCell and SCell could be 33usec for ATG DL CA using common Rx beam. When the UE sweeps Rx beam for neighbor cell measurement, 33usec difference considering MRTD would be impact on 1 data symbol of one of two CCs since the UE has common Rx beam for two CC. If the UE perform RRM measurement within SMTC, one symbol of one of the CC cannot be received due to Rx beam sweeping. Therefore, this should be considered sharing factor Klayer1_measurement, e.g.,

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

And also, scheduling restriction might be considered.

- Proposal 4: Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Conclusion

In this contribution, we provide views on RRM requireiments for ATG with CA operation, and we propose

- Proposal 1: Introduce UE capability to indicate the supported Rx beam type for inter-band CA per BC.

| Features | Index | Feature group | Components | Need for the gNB to know if the feature is supported | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Mandatory /Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-x | Rx beam type in inter-band CA | Indicate the support of common Rx beam or separate Rx beam in inter-band CA support antennaArrayType for each band | yes | If UE does not support the capability, performance cannot be guaranteed. | Per BC | No | FR1 only | Mandatory with capability signaling for UE supports NR communication via ATG |

- Proposal 2: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed for deactivated SCell measurement.

- Proposal 3: For ATG Case 4 antenna type in inter-band CA, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- Proposal 4: Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Reference

[1] R4-2502591, “WF on RRM requirements for NR_ATG_enh,” CMCC