3GPP TSG-RAN WG4 Meeting #114bis	R4-2503705

Wuhan, China April 07 – April 11, 2025

Source:	Tejas Networks

Title:	Study of impacts on RAN4 requirements for AIML mobility in NR

Agenda item:	7.21.2

Document for:	Discussion

Introduction

The discussions on AI/ML mobility in NR were continued in RAN4#114 in the scope of Rel-19 SI. Agreements achieved during this meeting are summarized in the WF [1].

In this paper, we provide our views on issues related to factors that potentially impact RAN4 requirements.

Discussion

During the RAN4 #112bis meeting, RAN4 agreed to study aspects related to NR AIML mobility requirements and prioritized the following scenarios and use cases.

Measurement event prediction use-cases

Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The study of the use cases is driven mainly by two study goals:

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.
The following metrics are considered:

  - MRRS	Measurement reduction rate in spatial domain

  - MRRT	Measurement reduction rate in temporal domain

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

- In Sub-use Case 2, L3 cell-level measurement results are predicted based on actual L3 cell level measurement results, primarily supporting Goal 2 by improving handover performance through better predictions. However, since real-time L3 measurements are still required for prediction, this approach does not inherently reduce the number of measurements (Goal 1). Similarly, in Sub-use Case 3, L3 cell-level results are predicted from actual L1 beam-level measurements, which also relies on real-time L1 measurements. While this approach enhances handover performance (Goal 2) by leveraging actual measurements, it does not reduce the number of measurements, as real-time L1 measurements collection remains necessary.

- When the AI/ML model forecasts a potential handover based on predicted L3 cell-level measurements, the network can take proactive actions, such as preparing for the handover in advance or deciding whether to proceed with the handover. However, both sub-use cases 2 and 3 with reliable prediction models enhances the performance of mobility management, but do not directly contribute to measurement reduction without additional strategies.

- Observation 1: Sub-use Case 2 and 3 with robust prediction model enhance handover performance through reliable predictions.

- Observation 2: Sub-use case 2 and 3 do not directly contribute to measurement reduction without additional strategies

- Proposal 1: RAN4 should prioritize the study and performance evaluation of Sub-use case 2 and Sub-use case 3.

- Exploring the possibility of relaxing the L1 beam-level measurement requirement based on the confidence level of the L3 cell-level measurement prediction.

- Proposal 2: RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements.

RRM Measurement prediction

- Following three domains have been identified for the AIML-based mobility enhancements:

- Temporal domain: Predicting future measurements values based on real time actual measurements. Two cases (Case A and Case B) are under discussion to evaluate for the 1st and 2nd study goals.

- Frequency domain: Measuring the serving cell and predicting unmeasured frequencies using the measured frequency as a reference or input.

- Spatial domain: Predicts the measurements in different directions based on the serving cell measurements.

- In the case of intra frequency temporal domain, among the two cases under discussion, Case B prediction is evaluated for the 1st study goal, while Case A is given lower priority. Below are two implementation examples of Case b, both of which help achieve substantial measurement reduction.

Figure 1: Example 1 of Intra-frequency temporal domain case B

Figure 2: Example 2 of Intra-frequency temporal domain case B

- Unlike example 2, where the prediction and observation window may overlap, example 1 has larger gap between consecutive observation windows because the prediction window does not overlap with the observation window. This can make it challenging to accurately assess the measurement prediction performance, especially in dynamic channel conditions. In, particular rapid channel variations can impact reliability of the predicted values, making performance evaluation more complex.

- Observation 3: Intra-frequency temporal domain Case A has been categorized as low priority.

- Observation 4: larger gap between consecutive observation windows and the absence of overlap between the prediction and observation windows make Intra-frequency temporal domain Case B, Example 1, prediction less reliable.

- Proposal 3: Prioritize intra frequency temporal domain Case B example 2, for study and performance evaluation.

- Figure 3 illustrates RRM measurements prediction in the frequency domain. Predicting measurements for unmeasured frequencies can be challenging due to varying channel conditions across frequency bands. Interference patterns also differ, which can lead to inaccurate predictions if not properly accounted for. Additionally, in scenarios with carrier aggregation or frequency hopping, managing multiple carriers or changing patterns can further reduce the reliability of predictions.

Figure 3: Inter frequency measurement prediction

Observation 5: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies

In scenarios where measurements are limited to the serving frequency, it can be beneficial to consider monitoring neighbouring frequencies for short durations. By incorporating measurements from inter-frequency neighbours and combining them with predictions based on temporal trends, the accuracy of these predictions can be improved. Temporal domain prediction, which relies on past measurement trends over time, can be enhanced by the additional context provided by inter-frequency measurements. This integration could provide a more comprehensive understanding of the network, potentially leading to more accurate predictions without significantly increasing measurement overhead.

Similarly, the same concept can be applied to the spatial domain. By integrating spatial predictions with temporal trends, a more robust and accurate prediction can be made, as spatial variations (such as differences in signal strength across locations) are taken into account alongside time-based changes. This integration helps account for the dynamic nature of the environment, where both spatial and temporal factors influence performance.

Proposal 4: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

Relative prediction accuracy

Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

- The agreed definition of absolute L3-RSRP prediction accuracy is:

- Absolute L3 predicted RSRP accuracy = Reported predicted L3-RSRP – Ground truth of L3-RSRP

RAN4 #113 agreed the following definition for relative RSRP accuracy

Predicted relative RSRP accuracy = (Reported predicted RSRP of cell 1/beam1 – Reported RSRP of cell 2/beam2) – (Ground truth of RSRP of cell 1/beam1 – Ground truth of RSRP of cell 2/beam2)

- With a minor rearrangement, we can rewrite the above expression as:

- Predicted relative RSRP accuracy = (Ground truth of RSRP of cell 2/beam2 - Reported RSRP of cell 2/beam2) – (Ground truth of RSRP of cell 1/beam1- Reported predicted RSRP of cell 1/beam1)

- Predicted relative RSRP accuracy = (Reported predicted RSRP of cell 1/beam1 - Ground truth of RSRP of cell 1/beam1) – (Reported RSRP of cell 2/beam2 - Ground truth of RSRP of cell 2/beam2)

- By substituting the absolute L3 predicted RSRP accuracy definition to the predicted relative RSRP accuracy equation above, the expression can be simplified to the following:

- Predicted relative RSRP accuracy = Absolute L3 predicted RSRP accuracy of cell 1/beam 1 - Absolute L3 predicted RSRP accuracy of cell 2/beam 2.

- Proposal 5: Update the definition of relative RSRP accuracy as follows:

- Predicted relative RSRP accuracy = Absolute L3 predicted RSRP accuracy of cell 1/beam 1 - Absolute L3 predicted RSRP accuracy of cell 2/beam 2.

- Relative RSRP accuracy provides a better understanding of how accurately the predictions are made relative to the best prediction.

- Proposal 6: RAN4 to consider both absolute and relative RSRP accuracy requirements for study.

- Proposal 7: Use the same ground truth definition of RSRP accuracy for relative RSRP accuracy

Ground Truth Definition for RSRP accuracy

Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

- Previous RAN4 meeting agreed to use conducted testing for FR1 as the baseline. Conducted test and has lower uncertainty due to the controlled and predictable nature of the test. In this case, the ground truth can be accurately estimated using the test equipment. Additionally, relying on reported measurements may not always provide the most accurate results, as they can be influenced by factors at the UE, such as signal quality or interference.

- Since FR2 involves a radiated test setup, extracting the ground truth RSRP is challenging. Relying on reported L3 cell measurement results bring up concerns due to the unknown Rx beamforming gain, which needs to be addressed. Maintaining a high SNR environment can be considered to ensure accurate reporting.

- Observation 6: Ground truth can be determined easily and accurately from the TE, it provides a clear and consistent reference point for comparing the predicted L3-RSRP values reported by the UE during the test.

- Observation 7: L3 RSRP measurement results can be calculated.

- Proposal 8: For FR1 conducted tests, the ground truth should be derived from the test equipment.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

- Observation 1: Sub-use Case 2 and 3 with robust prediction model enhance handover performance through reliable predictions.

- Observation 2: Sub-use case 2 and 3 do not directly contribute to measurement reduction without additional strategies

- Observation 3: Intra-frequency temporal domain Case A has been categorized as low priority.

- Observation 4: larger gap between consecutive observation windows and the absence of overlap between the prediction and observation windows make Intra-frequency temporal domain Case B, Example 1, prediction less reliable.

Observation 5: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies

- Observation 6: Ground truth can be determined easily and accurately from the TE, it provides a clear and consistent reference point for comparing the predicted L3-RSRP values reported by the UE during the test.

- Observation 7: L3 RSRP measurement results can be calculated.

- Proposal 1: RAN4 should prioritize the study and performance evaluation of Sub-use case 2 and Sub-use case 3.

- Proposal 2: RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements.

- Proposal 3: Prioritize intra frequency temporal domain Case B example 2, for study and performance evaluation.

Proposal 4: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

- Proposal 5: Update the definition of relative RSRP accuracy as follows:

- Predicted relative RSRP accuracy = Absolute L3 predicted RSRP accuracy of cell 1/beam 1 - Absolute L3 predicted RSRP accuracy of cell 2/beam 2.

- Proposal 6: RAN4 to consider both absolute and relative RSRP accuracy requirements for study.

- Proposal 7: Use the same ground truth definition of RSRP accuracy for relative RSRP accuracy

- Proposal 8: For FR1 conducted tests, the ground truth should be derived from the test equipment.

- References

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia), Athens, Greece, 17th – 21st February 2025.

- RP-250168, Status Report to TSG, Incheon, South Korea, March. 12-14, 2025

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Nokia, OPPO, MediaTek, Hefei, China, Oct 14 – Oct 18, 2024