3GPP TSG RAN WG4 #114-bis												      R4-2503708

Wuhan, China, April 7th – 11th, 2025

Agenda Item:		7.10.2

Source:				ViaSat Holdings Ltd, Inmarsat

Title:	On NR-NTN HPUE requirements

Document for:	Discussion and Decision

# Background

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN. In this document, we provide our views on the following issues that were discussed in the RAN4 #114 meeting [2], [3]:

- ACLR requirements

- SAR related issues

# ACLR Requirements

The following proposals were made in RAN4 #114 meeting [2]:

| Issue | Proposals | Recommended WF |
| --- | --- | --- |
| Issue 2-4-1: PC2 ACLR | - Option 1: Re-use TN PC2: 31 dBc (Apple, Qualcomm, Ericsson) - Option 2: Re-use TN PC3: 30 dBc. (vivo, Samsung) - Option 3: 22 dBc (Xiaomi) | PC2 ACLR is [30 or 31] dBc for NR-NTN UE |
| Issue 2-4-2: PC1.5 ACLR | - Option 1: Re-use TN PC1.5: 31 dBc (Apple, Qualcomm, Ericsson) - Option 2: Re-use TN PC3: 30 dBc. (vivo, Samsung) - Option 3: 23 dBc (Xiaomi) | For further discussion |
| Issue 2-4-3: PC1 ACLR | - Option 1: 37 dBc for non-handheld (Ericsson, Samsung) - Option 2: 31 dBc for non-handheld (Apple, Qualcomm) - Option 3: Re-use TN PC3: 30 dBc. (vivo) - Option 4: 24 dBc (Xiaomi) | For further discussion |

Observation 1: Based on the TN-NTN coexistence study, some companies want to relax the ACLR requirements beyond the TN specifications.

Observation 2: There has been no discussion of self-interference when considering aggregate ACI at the satellite if the requirements are relaxed beyond the TN specifications.

For example, consider the case of PC1 vehicular UEs with high antenna gain which are in adjacent frequencies to PC3 UEs with a -5.5dBi antenna. At the RAN, there would be a serious degradation in sensitivity of the PC3 UE. The spacecraft would very likely not support adequate filtering especially when considering the potential loss of orthogonality of OFDM carriers associated with a brick-wall filtering response.

The in-band SEM test requirements will not protect the weaker transmission, since this interference is in the radiated domain, not in conducted domain as per the in-band SEM test requirement.

Proposal 1: Do not relax the NTN ACLR requirements beyond the TN ACLR requirements for each power class.

Observation 3: The TN specification for NR HPUE PC1 has a high ACLR requirement. This specification is concerning UEs with a larger form factor than handheld devices.

Proposal 2: Specify two separate ACLR requirements for PC1 handheld and non-handheld devices. For non-handheld PC1 devices, reuse the PC1 TN ACLR requirements.

# SAR Related Issues

SAR methods have conventionally involved either

a) a transmit power backoff, 
b) a transmission duty cycle restriction, or 
c) a combination of both a) and b)

when SAR limits are reached by a UE.

Observation 4: In a non-terrestrial network, if the UE is using a UL duty cycle significant enough to exceed SAR limits, it is likely at the edge of coverage (using a low MCS with lots of parity), in which case, transmit power backoff is not a preferred option as it will almost invariably lead to radio link failure.

Any solution that is based solely on network-side or UE-side control of duty cycle restriction is going to be inefficient:

- If the RAN autonomously starts dropping UL slots it may pre-emptively reduce the UE’s uplink throughput.

- If the UE has a self-monitoring function (this is already the case) and it drops packets to manage SAR, we may get RLF and RRC disconnection.

An optional SAR mitigation method based on the “UL duty cycle” is specified in [4], where the UE can report a maximum UL duty cycle to the network. If the percentage of transmitted uplink symbols at the UE exceeds the corresponding duty cycle threshold, the UE should do a power-class fallback. However, this method is not apparently used for UEs operating in FDD bands in the field.

Proposal 3: For NTN, extend the existing TN signalling mechanism for UL duty cycle reporting to inform the RAN that it is close to the SAR limit, then the RAN will reduce the number of UL slots allocated to the UE. When the SAR limit has fallen, the UE can signal the improved condition back to the RAN so that the UL slots are picked back up.

# Conclusions

The observations and proposals are summarized as follows:

Observation 1: Based on the TN-NTN coexistence study, some companies want to relax the ACLR requirements beyond the TN specifications.

Observation 2: There has been no discussion of self-interference when considering aggregate ACI at the satellite if the requirements are relaxed beyond the TN specifications.

Observation 3: The TN specification for NR HPUE PC1 has a high ACLR requirement (38 dB). This specification is concerning UEs with a larger form factor than handheld devices.

Observation 4: In a non-terrestrial network, if the UE is using a UL duty cycle significant enough to exceed SAR limits, it is likely at the edge of coverage (using a low MCS with lots of parity), in which case, transmit power backoff is not a preferred option as it will almost invariably lead to radio link failure.

Proposal 1: Do not relax the NTN ACLR requirements beyond the TN ACLR requirements for each power class.

Proposal 2: Specify two separate ACLR requirements for PC1 handheld and non-handheld devices. For non-handheld PC1 devices, reuse the PC1 TN ACLR requirements.

Proposal 3: For NTN, extend the existing TN signalling mechanism for UL duty cycle reporting to inform the RAN that it is close to the SAR limit, then the RAN will reduce the number of UL slots allocated to the UE. When the SAR limit has fallen, the UE can signal the improved condition back to the RAN so that the UL slots are picked back up.

# References

[1] RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

[2] R4-2500680, “Topic summary for [114][128] NR_IoT_NTN_HPUE_part1”, Moderator (Samsung)

[3] R4-2500681, “Topic summary for [114][129] NR_IoT_NTN_HPUE_part2”, Moderator (vivo)

[4] 3GPP TS38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.