3GPP TSG-RAN WG4 Meeting #114bis	R4-2503717

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	KDDI

Title: 	(NR_newRAT-Core) Further study on NR A-MPR reevaluation scenarios on n1 for Japan

Agenda Item:	5.3.1

Document for:	Discussion

- 1. Introduction

In the previous meeting, RAN4 triggered NR A-MPR reevaluation scenarios and to specify new NS_05/NS_05U for n1 PC3 and PC2 on n1 for Japan [1]. On the other hand, based on the previous meeting’s WF [2], we recognize that there is one issue to be solved considering RAN1 spec.

- 2. Discussion

WF on PHS protection update (R4-2420481, RAN4#113)

In the last meeting, regarding A-MPR RB region definition for PC3, RAN4 agreed with WF [3] on PHS protection update as follows.

## < Issue 1-1-5: PC3 A-MPR simulation results for NS_05 > Agreement: - Use the harmonized A-MPR region definition as below table as the starting point. NS_05; Fc MHz; Region A1; Region A2. LCRB; RB start; LCRB; RB start. 5M; N/A; N/A; N/A; N/A. 10M; < 1930.7; ≥ 1.5*RBstart + 4.7MHz/12/scs; >0; <= 2.16MHz/12/scs; <= 1.8MHz/12/scs. 10M; N/A; N/A; N/A; N/A. 15M; 1927.5≦fc＜1932.5; >= RBstart*1.45 + 4.68MHz/12/scs; >0; <= 3.6MHz/12/scs; <= 3.6MHz/12/scs. 15M; 1932.5≦fc＜1938.2; ≥ 12.96/12/scs; ≤ 0.54/12/scs; <= 2.16MHz/12/scs; <= 2.16MHz/12/scs. 20M; < 1945.7; >= RBstart*1.5 + 4.14MHz/12/scs;; >0; <= 5.76MHz/12/scs; <= RBstart*1.5 + 4.14MHz/12/scs;; <= 5.04MHz/12/scs.

38.213 section 9.2.1 PUCCH Resource Sets (PRB offset)

In 38.213 section 9.2.1 “PUCCH Resource Sets”, there are restrictions for PRB offset as follows.

### 9.2.1 PUCCH Resource Sets If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs. For operation in FR2-2, nrofPRBs provided in PUCCH-ConfigCommon can also provide a number of NRB RBs for the PUCCH resource set; otherwise NRB=1. The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset RBBWPoffset, and a cyclic shift index set for a PUCCH transmission. The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping.

Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset RBBWPoffset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 | NBWPsize4 | {0, 3, 6, 9} |

Based on 38.213 section 9.2.1 PUCCH Resource Sets, NBWPsize4 can be offset on either side.

Observation 1: Based on 38.213 section 9.2.1 PUCCH Resource Sets, NBWPsize4 can be offset on either side.

Observation 2: The restriction is only for NR, and also there is no similar restriction in LTE.

Proposal 1: Considering 38.213 section 9.2.1 PUCCH Resource Set, RAN4 further study to improve A-MPR region definition.

- 3. Conclusion

Observation 1: Based on 38.213 section 9.2.1 PUCCH Resource Sets, NBWPsize4 can be offset on either side.

Observation 2: The restriction is only for NR, and also there is no similar restriction in LTE.

Proposal 1: Considering 38.213 section 9.2.1 PUCCH Resource Set, RAN4 further study to improve A-MPR region definition.

- 4. Reference

[1] R4-2418439 (NR_newRAT-Core) NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, RAN4#113

[2] R4-2420481 WF on PHS protection update, KDDI,OPPO, RAN4#113