3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503724

Wuhan, China, Apr. 7th – 11st, 2025

Source: 	Murata Manufacturing Co. Ltd

Title: 	DC_(n)3CA configuration and MSD

Agenda item:	6.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)	[DC_R19_xBLTE_yBNR-Core]

Document for:	Discussion

- Introduction

Our views on the DC_(n)3CA configuration are presented in this contribution.

- Discussion

2.1 Background Information

In RAN#106, DC_(n)3CA has been requested [1]. Table 1 shows bandwidth combinations for these DC combinations.

Table 1: Bandwidth combinations for DC_(n)3CA

| DC configuration | Uplink DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3CA | DC_(n)3AA | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30, 40 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30, 40 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)3CA | DC_3A_n3A | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 10, 20, 25, 30, 40 |  | 50 | 0 |
|  |  |  | 10, 20, 25, 30, 40 | See CA_3C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |

This contribution shows our views on DC_(n)3CA configuration.

2.2 Configuration options for DC_(n)3CA

Figure 1 describes allocations of component carriers for DC_(n)3CA. To maximize the interference on LTE DL carrier(s) by NR UL carrier, the NR UL carrier should be configured to be as wide as possible. In this configuration, LTE DL1 is allocated to be closest to the NR UL carrier.

Figure 1: Landscape of DC_(n)3CA with 1UL configurations

Regarding LTE carrier bandwidth for DC_(n)3CA, the configuration table refers to CA_3C BCS in TS36.101 Table 5.6A1-1, as shown in Table 2. Table 3 represents bandwidth classes for LTE defined in table 5.6A-1 in TS 36.101.

Table 2: CA_3C BCS0, extracted from TS 36.101 Table 5.6A 1-1

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated | Bandwidth combination set |
|  | (NOTE 3) | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | bandwidth [MHz] |  |
| CA_3C | CA_3C | 5, 10, 15 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5,10,15,20 |  |  |  |  |  |
| NOTE 3: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |  |  |  |  |  |

Table 3: Bandwidth classes for LTE, extracted from TS36.101 Table 5.6A-1

| CA Bandwidth Class | Aggregated Transmission Bandwidth Configuration | Number of contiguous CC | Nominal Guard Band BWGB |
| --- | --- | --- | --- |
| A | NRB,agg ≤ 100 | 1 | a1 BWChannel(1) - 0.5f1 (NOTE 2) |
| B | 25 < NRB,agg ≤ 100 | 2 | 0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1 |
| C | 100 < NRB,agg ≤ 200 | 2 | 0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1 |
| D | 200 < NRB,agg ≤ 300 | 3 | 0.05 max(BWChannel(1),BWChannel(2), BWChannel(3)) - 0.5f1 |
| E | 300 < NRB,agg ≤ 400 | 4 | 0.05 max(BWChannel(1),BWChannel(2), BWChannel(3) , BWChannel(4)) - 0.5f1 |
| F | 400 < NRB,agg ≤ 500 | 5 | 0.05 max(BWChannel(1),BWChannel(2), BWChannel(3) , BWChannel(4) , BWChannel(5)) - 0.5f1 |
| I | 700 < NRB,agg ≤ 800 | 8 | NOTE 3 |
| NOTE 1: BWChannel(j), j = 1, 2, 3, 4 is the channel bandwidth of an E-UTRA component carrier according to Table 5.6-1 and f1 = f for the downlink with f the subcarrier spacing while f1 = 0 for the uplink. NOTE 2: a1 = 0.16/1.4 for BWChannel(1) = 1.4 MHz whereas a1 = 0.05 for all other channel bandwidths. NOTE 3: Applicable for later releases. |  |  |  |

Based on these specifications, CA_3C refers to two contiguously aggregated LTE carriers with a total bandwidth of 40MHz or less. The minimum aggregated bandwidth of CA_3C is 25MHz as one component carrier must use 20MHz, while the bandwidth of the other component carrier bandwidth can be optionally selected from 5,10,15, or 20MHz. This notes that the minimum aggregated LTE DL carrier bandwidth for DC_(n)3CA is 25MHz, therefore, the maximum NR UL carrier bandwidth will also be limited to 25MHz.

Observation 1: As per the initial bandwidth combinations for DC_(n)3CA, the maximum NR UL carrier bandwidth will be limited up to 25MHz.

During an offline discussion, one company shared their views on MSD concepts for multiple LTE DL carriers, using example test points for DC_(n)3CA which are derived from DC_(n)3AA test points. Table 4 presents the existing test point for DC_(n)3AA, extracted from TS38.101-3.

Table 4: The existing test point for DC_(n)3AA, extracted from TS38.101-3

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770 | 30 | [10 (RBstart = 150)] | 1865 | N/A |  |

As shown in Table 4, the test point uses 30MHz NR UL configuration, which does not align with the available NR uplink configuration for DC_(n)3CA.

Observation 2: For DC_(n)3CA test point, the existing test point for DC_(n)3AA cannot be reused because the NR bandwidth configuration is not aligned with that of DC_(n)3CA.

If the bandwidth combinations for DC_(n)3CA are updated such that the minimum aggregated bandwidth for LTE carriers is limited to 20MHz, the NR UL carrier can be expanded to up to 30MHz. This adjustment may introduce an additional candidate test point, potentially causing interference to spread across both LTE DL1 and DL2 carriers.

Observation 3: Specifying the minimum aggregated LTE bandwidth to 20 MHz in DC_(n)3CA may allow expanding the NR UL carrier to 30 MHz, potentially causing interference that impacts both LTE DL1 and DL2 carriers.

Table 5 shows MSD test points for NR 25MHz 1UL case analysis. The Lcrb for the first and second bullets are derived from uplink configuration for reference sensitivity in Table 7.3.2-3 in TS38.101-1, allowing a comparison of their impacts with those of the third and fourth bullets. The third and fourth test points are based on the approach where Lcrb is adjusted to ensure a maximum impact of Tx emission. Compared to the candidates in [2], Lcrb for the third bullet uses the same Lcrb configuration as the fourth bullet for simplifications.

Table 5: Candidate test points for DC_(n)3CA with 1UL configuration

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 50 (RBstart = 83) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [TBD] |  |
|  | 3 | N/A | 20 | N/A | 1845 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 3 (RBstart = 130) | 1867.5 | N/A |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [TBD] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [TBD] |  |
|  | n3 | 1772.5 | 25 | 3 (RBstart = 130) | 1867.5 | N/A |  |

Observation 4: For Lcrb configuration, there are two approaches available. One is by referring to uplink configuration for reference sensitivity in Table 7.3.2-3 in TS 38.101-1. The other is to adjust Lcrb to ensure a maximum impact of Tx emission.

These test points are described in Figure 2.

Figure 2: Landscape of candidate test points for DC_(n)3CA with 1 UL

When the NR UL carrier bandwidth is configured as 25MHz, it can be observed that any Lcrb configuration will only impact on LTE DL1 carrier.

Observation 5: When the NR UL carrier bandwidth is configured as 25MHz, any Lcrb configuration will only impact on LTE DL1 carrier.

For further study on the DC_(n)3CA configuration, this contribution provides MSD simulation results using the 25MHz NR 1UL test points.

2.3 MSD evaluation

2.3.1 PA Calibration

PC3 PA is calibrated at MPR1 for -30 dBc ACLR using a fully allocated 20 MHz DFT-s-OFDM QPSK waveforms.

2.3.2 Simulation results

We evaluated MSD values for PC3 DC_(n)3CA using the following assumptions.

- Tx-Rx duplexer isolation: 50dB

- Tx antenna duplex rejection in Rx range: 45dB

- Primary antenna to diversity antenna isolation: 10dB

- MRC combining and -1 dB Signal to-Noise Ratio (SNR)

Figure 3: MSD analysis for DC_(n)3CA

As per Observation 5, REFSENS will be more degraded for the LTE DL1 carrier, which is closest to the NR UL carrier, while there will be minimal impact on the LTE DL2 carrier. From the perspective of UE RF-FE performance, it may be sufficient to specify MSD only for the first LTE DL carrier closest to the NR UL carrier. Furthermore, the results indicate that adjusting Lcrb has a greater impact of Tx emission compared to deriving Lcrb from the uplink configuration for reference sensitivity, as specified in Table 7.3.2-3 of TS 38.101-1.

Observation 6: From the perspective of UE RF-FE performance, it may be sufficient to specify MSD only for the first LTE DL carrier closest to the NR UL carrier.

Observation 7: By adjusting Lcrb, a greater impact of Tx emission is produced compared to by deriving Lcrb from the uplink configuration for reference sensitivity, as specified in Table 7.3.2-3 of TS 38.101-1.

Proposal 1: Specify MSD values and test points for DC_(n)3CA highlighted in Table 6 in case bandwidth configuration for DC_(n)3CA is defined as requested in [1]. However, we are open to discussion.

Table 6: MSD for DC_(n)3CA

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [1.6] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [0] |  |
|  | n3 | 1772.5 | 25 | 50 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 83) |  |  |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [0.8] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [0] |  |
|  | n3 | 1772.5 | 25 | 50 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 83) |  |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [3.9] |  |
|  | 3 | N/A | 20 | N/A | 1845 | [0] |  |
|  | n3 | 1772.5 | 25 | 3 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 130) |  |  |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [1.6] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [0] |  |
|  | n3 | 1772.5 | 25 | 3 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 130) |  |  |  |

Proposal 2: RAN4 should clarify how to specify MSD requirements for intra-band contiguous EN-DC combinations with multiple LTE DL carriers.

- Conclusion

This contribution shared our views of DC_(n)3CA configuration.

Observation 1: As per the initial bandwidth combinations for DC_(n)3CA, the maximum NR UL carrier bandwidth will be limited up to 25MHz.

Observation 2: For DC_(n)3CA test point, the existing test point for DC_(n)3AA cannot be reused because the NR bandwidth configuration is not aligned with that of DC_(n)3CA.

Observation 3: Specifying the minimum aggregated LTE bandwidth to 20 MHz in DC_(n)3CA may allow expanding the NR UL carrier to 30 MHz, potentially causing interference that impacts both LTE DL1 and DL2 carriers.

Observation 4: For Lcrb configuration, there are two approaches available. One is by referring to uplink configuration for reference sensitivity in Table 7.3.2-3 in TS 38.101-1. The other is to adjust Lcrb to ensure a maximum impact of Tx emission.

Observation 5: When the NR UL carrier bandwidth is configured as 25MHz, any Lcrb configuration will only impact on LTE DL1 carrier.

Observation 6: From the perspective of UE RF-FE performance, it may be sufficient to specify MSD only for the first LTE DL carrier closest to the NR UL carrier.

Observation 7: By adjusting Lcrb, a greater impact of Tx emission is produced compared to by deriving Lcrb from the uplink configuration for reference sensitivity, as specified in Table 7.3.2-3 of TS 38.101-1.

Based on the above observations, this contribution provided following proposals.

Proposal 1: Specify MSD values and test points for DC_(n)3CA highlighted in Table 6 in case bandwidth configuration for DC_(n)3CA is defined as requested in [1]. However, we are open to discussion.

Proposal 2: RAN4 should clarify how to specify MSD requirements for intra-band contiguous EN-DC combinations with multiple LTE DL carriers.

Table 6: MSD for DC_(n)3CA

| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [1.6] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [0] |  |
|  | n3 | 1772.5 | 25 | 50 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 83) |  |  |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [0.8] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [0] |  |
|  | n3 | 1772.5 | 25 | 50 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 83) |  |  |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [3.9] |  |
|  | 3 | N/A | 20 | N/A | 1845 | [0] |  |
|  | n3 | 1772.5 | 25 | 3 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 130) |  |  |  |
| DC_(n)3CA | 3 | N/A | 20 | N/A | 1840 | [1.6] |  |
|  | 3 | N/A | 5 | N/A | 1852.5 | [0] |  |
|  | n3 | 1772.5 | 25 | 3 | 1867.5 | N/A |  |
|  |  |  |  | (RBstart = 130) |  |  |  |

References:

[1] RP-243257, “Revised WID for DC_R19_xBLTE_yBNR”, Nokia, RP#106, Madrid, Spain

[2] R4-2501078, “MSD test point for DC_(n)3CA”, Murata Manufacturing Co. Ltd, RAN4#114, Athens, Greece