TSG-RAN Working Group 4 (Radio) meeting #114-bis	R4-2503725

Wuhan, China, 7th to 11th April 2025

Source:	Ericsson

Title:	Considerations on Rel-20 BS RF package objectives

Agenda item:	10

Document for:	Discussion

# Introduction

At the last RAN#107 meeting in Incheon the discussion about RAN4 Rel-20 5G-Advanced work packages started. All the input proposals were collected and categorized into different groups with the aim to further describe justification and objectives at coming meetings, with the intention to define relevant Rel-20 RAN4 work items at RAN#109 in China. The following work package categorizes have been identified by RAN:

- UE RF centric evolution

- BS RF centric evolution

- UE OTA enhancement

- RRM enhancement

- Demodulation performance evolution

- NTN enhancements

- Cross RF and baseband topics

In this contribution we focus only on BS RF centric evolution in Rel-20 with the intention to clarify technical justification and objectives for work to be included in BS RF work package.  It is essential to find relevant scope of each work package category to be able to fit it within RAN4 Rel-20 Time Unit (TU) budget. The information provided in this contribution can be used when new Work Item(s) (WI) are defined for RAN4 Rel-20 time period.

# Discussion

From the moderator summary from last RAN plenary meeting [1], the work packages related to BS RF evolution was collected in a list without priority of high-level candidate objectives for BS RF centric evolution and/or enhancement in Rel-20 as:

- Develop improved co-location requirements and test solutions for FR1 high band(s). The scope of this objective depends on the progress in Rel-19 BS RF evolution WI.

- Introduction of new additional NR BS type and associated requirement set for AAS with large antenna array.

- Re-evaluate and validate receiver in-band blocking requirement levels for FR1 high bands(s).

- Add support for NB-IoT in AAS BS specifications.

- Add support for LP-WUS in TN MSR specifications.

- Specification structure simplifications for OBUE requirements. FFS on whether to do it in Rel-20 WI or as maintenance.

The intention is to use the list of candidate objectives as the starting point for 5G-Advanced BS RF centric evolution and/or enhancement in Rel-20, considering the TU splitting between 5G and 6G in RAN4.

In following sections specific details related to identified topics are presented. Some objectives are linked together, while other is independent.

  - Enhanced co-location requirement concept

In TS 38.104, requirements to guarantee co-location of two BS at the same site have been define. The first is a requirement for transmitter spurious emission within another 3GPP band and the second is receiver blocking due to another 3GPP band transmitting. These are the two requirements normally referred to as co-location requirements.

In addition, the scope of co-location has been increased since also other requirements based on BS-to-BS isolation of 30 dB is also often referred to as co-location requirements. The complete set of co-location requirements are listed in Table 2.1-1.

Table 2.1-1: Overview of co-location requirements

| Requirement | Type | Brief description |
| --- | --- | --- |
| Transmit ON/OFF power | Mandatory | This requirement specifies a maximum limit of the aggressor wanted carrier emission leakage during the receive period falling within victim receiver and maximum allowed transient period. The requirement also includes a maximum transient period between ON and OFF power. The CLTA is used to measure emission. |
| Spurious emission | Optional by declaration | This requirement specifies the emission falling within victim receiver bandwidth. The CLTA is used to measure emission. |
| Transmitter intermodulation | Mandatory | This requirement specifies the victim capability to maintain unwanted emission requirement levels when an aggressor interferer signal is injected. The CLTA is used to inject the interferer signal. |
| Out-of-band receiver blocking | Optional by declaration | This requirement specifies the ability to maintain sensitivity when an aggressor interferer signal is injected. The CLTA is used to inject the interferer signal. |

For BS type 1-O co-location requirements 3GPP developed a concept based on a co-location reference antenna in Rel-15. For the purpose of conformance testing the co-location reference antenna is referred to as the Co-Location Test Antenna (CLTA).

The co-location requirements are requirements which are based on assuming the BS type 1-O is co-located with another non-AAS BS of the same BS Class (Wide Area BS), they ensure that both co-located systems can operate with minimal degradation to each other. The CLTA shall be a single column passive antenna which has the same vertical radiating dimension (h), frequency range, polarization, as the composite antenna of the AAS BS and nominal 65° horizontal half-power beamwidth (suitable for 3-sector deployment) and is placed at a distance d from the edge of the test object, as shown in Figure 2.1-1.

# Figure 2.1-1: Rel-15 co-location requirement concept

The enclosure edge-to-edge separation d between the test object and the CLTA shall be set to 0.1 m. The test object and the CLTA shall be aligned in a common plane perpendicular to the mechanical bore-sight direction. The vertical radiating regions of the CLTA and the test object composite antenna shall be aligned. For co-location requirements where the frequency range of the signal at the CLTA is different from the test object, a CLTA suitable for the frequency stated in the requirement is assumed.

OTA co-location requirements are based on the power at the conducted interface of a CLTA, depending on the requirement this interface is either an input or an output. For test object with dual polarization the CLTA has two conducted interfaces each representing one polarization. More details on required CLTA characteristics are listed in Table 2.1-2.

Table 2.1-2: CLTA characteristics

| Parameter | In-band CLTA | Out-of-band CLTA |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | Test object vertical radiating length ±30% (Note 2) |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2) |
| Polarization (Note 3) | Match (Note 4) | Match to in-band (Note 4) |
| Conducted interface return loss | > 10 dB | > 10 dB |
| Note 1: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. Note 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise, the vertical beam width definition shall be used. Note 3: For BS type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization. Note 4: Matched to the polarization of EUT antenna. |  |  |

As an alternative, the specification allows for other antennas than the CLTA to be used. Unfortunately, the standard doesn’t capture the translation of the requirements to other test antennas. Other test antennas are not precluded but suitable translations between the co-location reference antenna and test antenna must be provided to demonstrate that the method is within the specified Measurement Uncertainty (MU). To allow a different type of antenna to be used additional information needs to be added to specification to make this alternative feasible.

In addition, the CLTA is not available for upper frequency ranges of FR1, and thus alternative approaches would be beneficial to investigate e.g. near-field probing or re-formulation of requirement as an E-field strength requirement.

Observation 1: It would be appropriate in Rel-20 to develop a requirement concept to allow for use of other antennas than array antennas, such as standard gain horn, dipole or other type appropriate for the purpose of conformance testing. The current specification indicates that other test antennas can be used but lack description on how to do it.

The co-location requirements are defined based on a concept of having a CLTA mimicking a specific co-location scenario. The intention was that the CLTA would be easily available, which is true for frequencies where passive BS antennas is used (e.g. blow 3 GHz) which is not the case for upper part of FR1. The technical background and challenges related to use current test specification and adding support for upper parts of FR1 and above 7125 MHz can be summarized as:

Requirement issues:

- RAN4 needs to re-evaluate relevant co-location scenarios, such as side-by-side (used as baseline for Rel-15), on top each other, in different sites pointing towards each other (Used as baseline before Rel-15), or any other relevant scenario. The scenario considered for Rel-15 may not be representative for many deployed networks now (close to end of Rel-19). The antenna arrays used at higher frequencies are typically electrically much larger than what was assumed in the technical background in Rel-15.

- The Rel-15 co-location scenario consider an AAS co-located with a non-AAS with a passive single column antenna. With a single column antenna, the array factor in the horizontal domain is not captured. During the introduction of AAS this was a reasonable scenario to consider, but now with new bands above 2.5 GHz also scenarios with AAS to AAS may be highly relevant. For AAS, a statistical approach would be needed to capture relevant coupling due to beam forming.

- The Rel-15 co-location scenario is focused on wide area deployment. Its relevance for other BS classes like medium range BS and local area BS, etc. is not clear. Eventually, new co-location antennas or scenario may be needed for those BS classes to make specifications complete.

- All requirement levels are derived based on an assumed port-to-port isolation of 30 dB. When 30 dB was assumed, the isolation between a single column antenna and a AAS with few branches was considered. Now RAN4 consider higher frequencies and much larger array structures. The isolation value to assume above 2.5 GHz with more than 32 transceiver branches and considering array excitations should be further studied. To find a relevant isolation a statistical analysis including beamforming aspects on both victim and aggressor would be required. As coupling given same physical separation would be frequency dependent, the requirement levels need to be adjusted accordingly.

- To maintain degradation level used for non-AAS BS, RAN4 should reconsider the AAS BS co-location spurious emission and not include the 9 dB relaxation part of requirement derivation background. The current requirement levels for AAS BS may not guarantee the same level of degradation as for non-AAS BS.

Conformance test issues:

- A band specific Co-location Test Antenna (CLTA) is required per declared supported co-location band. This will create a logistical problem during conformance testing since a product is typically declared to support many co-location situations. This will result in many CLTAs are needed per tested product, which create a logistical challenge for test laboratories.

- For bands defined above 2.5 GHz it is very difficult to find commercially available single column BS antennas or multi-columns with similar characteristics as specified in TS 38.141-2, subclause 4.12. Therefore, the availability and access to relevant CLTAs are very limited for bands above 2.5 GHz.

- By experience, it can be concluded that the test setup for transmitter intermodulation is very complex. A large PA capable of very high output power is required to generate the interfering signal clean enough to be injected by the CLTA, while TRP unwanted emissions is measured and evaluated. The TRP level for unwanted emission can be measured in a far-field test environment or in a reverberation chamber test environment, as described in TS 38.141-2.

- The CLTA puts additional burden on the OTA test environments, such as volume restrictions and additional weight on the positioner used in the test chamber. It is therefore not suitable to use a passive BS antenna with multiple columns, even though current specification allows it.

- Due to the very complex test setup required to test transmitter intermodulation (TX IMD) careful review of the test scope is required to minimize total test time. The test is based on measuring TRP at the same time an interferer signal is injected using the CLTA. Further evaluation of the test scope in TS 38.141-2, subclause 6.8.4 to capture only relevant cases of e.g., carrier, bandwidth, multi-band and carrier aggregation configurations.

- It should be noted that many co-location requirements either require very high OTA chamber dynamic range (co-location spurious emission) or require generation of very high power in the chamber as CLTA and existing co-location concept would imply blocking interferer level 30 dB higher than existing conducted requirements.

For bands in the upper region of FR1 and the frequency range between 7 to 15 GHz the co-location requirement concept needs to be evolved to resolve issues listed above. It would be appropriate to define requirement such that the need for a specified dedicated CLTA can be avoided. Hence, determine relevant parameters and requirement levels based on OTA parameters such as field-strength or power density at a given test distance. In [3], some basic technical background for such approach was presented. Further elaborations and studies may be needed to fully define an evolved requirement concept for co-location requirements.

Observation 2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, sublease 4.9 is not practical for conformance testing of product operating above 3.5 GHz.

Proposal 1: In Rel-20, include WI to define enhanced co-location requirements for AAS BS with focus above 3.5 GHz. The aim would be to define requirement based on OTA parameters without need for specific specified test antenna.

In current Rel-19 BS RF evolution WI work is ongoing to better understand BS-to-BS isolation based on measurements and simulation results. Also, aspects of considering standard antennas as test antennas is studied. So far it can be concluded that the BS-to-BS isolation assumed to be 30 dB is not relevant for frequency bands in the upper region of FR1.

  - Introduction of new AAS BS type

In Rel-15, the concept of NR BS types was established to allow the specification to support different types of base stations for FR1 (non-AAS and AAS) and FR2 (AAS). The requirement applicability for the different base station types is different since requirements are defined at different requirement interfaces referred to as TAB and RIB. The interfaces are visualised in Figure 2.2-1.

# Figure 2.2-1: BS architecture

The currently supported BS types and a potential new additional BS type (here referred to BS type 1-H-enh) are listed in Table 2.2-1.

Table 2.2-1: BS types

| BS type | Requirement interfaces | Note |
| --- | --- | --- |
| 1-C | TAB | FR1 non-AAS BS with few RF ports and passive antenna |
| 1-H | TAB, RIB | FR1 AAS BS with access to RF ports at TAB |
| 1-H-enh | TAB2, RIB | New BS type for FR1 AAS BS with access to RF ports at TAB2 |
| 1-O | RIB | FR1 AAS BS without access to RF ports |
| 2-O | RIB | FR2 AAS BS without access to RF ports |

Considering the coverage capability for AA BS to be the same for bands in the upper region of FR1 with FR1 mid bands would lead to larger number of antenna elements needed for AAS BS products in the upper region of FR1 compared with FR1 mid bands. Since the number of antenna elements needed will be significantly larger than the supported number of layer 1 logical ports, different types of port expansion techniques will be required to map signal from logical ports to physical antenna elements. Traditionally the port expansion is done by combining elements as vertical sub-arrays. This combination is done using passive combination networks in the Radio Distribution Network (RDN) and as digital port expansion between baseband and radio. For large arrays, the vertical sub-array tends be very large which would restrict the vertical steering range. Therefore, hybrid beamforming schemes are often considered to add steering capabilities in the RDN by adding analog components, such as phase shifters, switched delay lines, etc.

The current BS architecture described in TS 38.104, subclause 4.3 do not explicitly describe type of beamforming schemes supported. However, for electrically large arrays it is essential to define the requirement interface such that all emission generated by the electronics is captured. In the case of having active components in the RDN, a requirement interface should be defined after the RDN. With this interface TAB2 all emission generated by the radio electronics will be captured at TAB2 see Figure 2.2-1.

Another aspect to consider is that with very many transceiver branches it is essential to define relevant requirement level per branch. For BS type 1-H, the legacy requirement for the receiver is re-used with the consequence that the receiver for a BS with a large antenna will be over specified, costly and power hungry. Therefore, it would be natural to move to the OTA domain for in-band receiver requirements and in-band transmitter requirements. Similar to the transmitter, additional considerations might be needed to ensure that receiver requirements are properly defined, and relevant test framework is in place as we move towards large electrically antennas including the architecture which cover such receiver beam forming and corresponding requirement interface.

In addition, the OTA test lab capabilities in the industry are not mature for AAS BS FR1 all OTA conformance testing. Therefore, we suggest introducing a new BS type with a requirement set consisting of in-band requirements to be specified as OTA requirement while out of band requirements are defined as conducted requirements.

The general principles for establishing a complete requirement set for the new BS type are listed below:

- All in-band requirements are done OTA at RIB

- All unwanted emission requirements are done conducted at TAB2

- All co-location requirements are done OTA at RIB

- Possibly all receiver requirements should be done OTA at RIB given that improved blocking requirement (co-location) and testing is concluded during Rel-19 ongoing work.

With the given general principles, further details related to the exact mapping required for the applicability table in TS 38.104, subclause 4.6 can be worked out.

Observation 3: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to enhancement of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 4: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Proposal 2: In Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

If a new BS type is defined in addition to BS type 1-C, 1-H and 1-O, RAN4 need to work out guidelines on applicability for when different BS types are relevant, e.g. defining caps on BS types based on number of ports, number of transceivers and supported beamforming schemes for each AAS BS type.

For the OTA receiver requirement for BS type 1-O, declarations related to the element/sub-array is needed to translate requirement levels from legacy conducted requirement levels to OTA requirements levels. With the introduction of more advanced RDN structures with fast analog phase shifters combining several sub-arrays in more complex sub-array structures it’s not clear how the declaration defined in current specification should be done. More work is needed to clarify interfaces in relating to the RDN and the antenna array and clearly describe what type of functionality is included in the RDN.

Observation 5: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations.

  - Re-evaluation of in-band blocking requirement

The technical background related to the derivation of in-band receiver blocking requirement can be traced back to UTRA and the period around 1999. Now in Rel-20 networks are typically build in different manner as originally assumed for UTRA. For NR, the network grid is denser, UEs are using high output power and BS antennas are significantly larger. It is therefore time to re-evaluate the relevance of the requirement for in-band blocking current stated in 3GPP specification for bands in the higher region of FR1. For this work RAN4 need to establish a new set of simulation assumptions reflecting NR network deployments.

Similar work is ongoing in Rel-19 BS RF evolution WI for transmitter co-existence spurious emission levels.

Observation 6: It can be noticed that requirement levels for co-existence spurious emission is re-evaluated in Rel-19. Based on similar technical background the same work is needed for the receiver in-band blocking requirement.

Proposal 3: In Rel-20, include WI to re-evaluate relevance of current receiver in-band blocking requirement levels for bands in the upper region of FR1.

  - Adding support for new features

When AAS BS specifications was developed in Rel-15 it was decided not to include support for NB-IoT. For Rel-20, the industry is ready to extend the support for AAS to include support for NB-IoT. The work to make specifications complete would require updates to TS 37.105 and TS 38.104 BS type 1-H and BS type 1-O to include proper requirements.

When AAS specification was established support for NB-IoT was not included. Now when AAS BS have been adopted on the marked for many bands within FR1 it may be time to evolve the AAS BS specifications to include support for NB-IoT.

Due to the recent introduction of LP-WUS in BS specifications it would be natural to schedule the introduction to MSR specifications (TS 37.104 and TS 37.105).

Proposal 4: Further discuss the need and the required scope for the work package required to include NB-IoT support to AAS specifications (TS 37.105, TS 37.145-2, TS 38.104 and TS 38.141-2) and LP-WUS in BS specifications (TS 38.104, TS 38.141-1 and TS 38.141-2).

  - Specification improvements

In Rel-19, RAN tasked RAN4 to study different approaches to improve the specification structure with respect to co-existence spurious emission requirements and co-location spurious emissions requirements. The idea is to find more efficient was of capturing requirements rather than very long tables in numerous specifications.

For Rel-20, it’s proposed to also look at Operating Band Unwanted Emission (OBUE) requirements and study if there are any better way to define these requirements by avoiding duplications of similar tables. The intention is to reduce the number of tables required to capture all bands in a common format.

Proposal 5: The objective related to specification improvements of OBUE can be done as regular maintenance or RAN Task in Rel-20 time period. There is no need to define a separate WI for this type of work package.

# Conclusion

In this contribution we provide some additional information related to the discussion started at RAN#107 to identify potential objectives for a REL-20 BS RF evolution WI(s).

Based on the information we observe following:

Observation 1: It would be appropriate in Rel-20 to develop a requirement concept to allow for use of other antennas than array antennas, such as standard gain horn, dipole or other type appropriate for the purpose of conformance testing. The current specification indicates that other test antennas can be used but lack description on how to do it.

Observation 2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, sublease 4.9 is not practical for conformance testing of product operating above 3.5 GHz.

Observation 3: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to enhancement of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 4: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Observation 5: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations.

Observation 6: It can be noticed that requirement levels for co-existence spurious emission is re-evaluated in Rel-19. Based on similar technical background the same work is needed for the receiver in-band blocking requirement.

Based on the observations we propose the following to stimulate further discussions related to identifying relevant work package objectives for the Rel-20 BS RF evolution in RAN4:

Proposal 1: In Rel-20, include WI to define enhanced co-location requirements for AAS BS with focus above 3.5 GHz. The aim would be to define requirement based on OTA parameters without need for specific specified test antenna.

Proposal 2: In Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

Proposal 3: In Rel-20, include WI to re-evaluate relevance of current receiver in-band blocking requirement levels for bands in the upper region of FR1.

Proposal 4: Further discuss the need and the required scope for the work package required to include NB-IoT support to AAS specifications (TS 37.105, TS 37.145-2, TS 38.104 and TS 38.141-2) and LP-WUS in BS specifications (TS 38.104, TS 38.141-1 and TS 38.141-2).

Proposal 5: The objective related to specification improvements of OBUE can be done as regular maintenance or RAN Task in Rel-20 time period. There is no need to define a separate WI for this type of work package.

# References

[1]	RP-250817, “Moderator's summary for RAN4 led REL-20 topics”, RAN4 Chair (Huawei)

[2] 	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson

[3]	R4-2319398, “Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements”, Ericsson