TSG-RAN Working Group 4 (Radio) meeting #114-bis	R4-2503726

Wuhan, China, 7th to 11th April 2025

Source:	Ericsson

Title:	Rel-19 BS RF evolution overview

Agenda item:	7.13.1

Document for:	Discussion

# Introduction

At the last RAN4 meeting (RAN4#114 in Athens) a contribution with high level status overview was presented in [1] with information for each work package included in Rel-19 BS RF evolution Work Item (WI). The intention with this contribution is to provide guidance for this meeting and coming meetings.

At last, RAN4 meeting the work related to the introduction of a new protection requirement for FSS UL was finalised. This meeting a new discussion topic will be started for the re-evaluation of transmitter spurious emission levels for bands in the upper region of FR1.

The BS RF evolution WI consists of multiple running work packages with different objectives as described in last revision of WID [2].

In this contribution we provide a status overview of work packages included in BS RF evolution WI. For each work package we have collected the relevant findings in respective area to prepare for creating CRs for BS RF specifications and preparing work in Rel-20.

# Discussion

The work plan for BS RF evolution WI consists of multiple work packages with different start and end dates running in parallel and series as visualised in Figure 2-1.

# Figure 2-1: Work plan

According to the Status Report [5] presented at last RAN plenary meeting (RAN#107 in Incheon) the activities ongoing in the BS RF evolution WI is summarized in following sub-sections.

  - Maintenance of protection requirement for FSS UL within band n104

As part of the outcome from WRC-23 in Resolution COM4/7 the frequency range 6425 to 7125 MHz was identified in every ITU region as in the Radio Regulation Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the Expected Equivalent Isotropically Radiated Power (EEIRP) spectral density for International Mobile Telecommunications (IMT) base stations for protecting Earth-to-Space  Geo Stationary Orbit (GSO) Fixed Satellite Services (FSS). EEIRP is a new figure of merit used for the new regulatory requirement. The technical background captured in Annex to the Resolution COM4/7 (WRC-23) outlines the theoretical calculation of the EEIRP of an IMT base station for assessing the compliance of IMT base station equipment with the limit on EEIRP. Based on the technical condition a new requirement was defined in 3GPP BS RF specifications (TS 38.104 and TS 38.141-2 and corresponding NCR specifications). With the requirement included in the 3GPP standard harmonized terminology, declarations and conformance test method have been established in relevant specifications providing a solid foundation for future network deployments using band n104.

Status: A new requirement has been included in Rel-19 BS RF specifications. The work is complete, except for the Measurement Uncertainty (MU) evaluation part of the conformance test specification.

Some minor additions, editorial improvements and corrections will be needed. This work will be done as regular maintenance of Rel-19 specifications.

  - Evolution of co-location requirements

The WID for Rel-19 BS RF evolution WI have a dedicated objective to investigate whether the BS co-location requirements concept adopted in Rel-15 is still relevant for frequency bands in the upper region of FR1.

Status: Companies have provided information on BS-to-BS coupling loss based on measurements and simulations relevant for bands in the upper region of FR1. An overview the results was captured in [6] and listed in Table 2.2-1. It can be noticed that early simulations and measurement results indicates that the assumption of 30 dB may not be relevant for bands in the upper region of FR1. In addition, information related to practical test aspects such as more practical test antennas have been discussed.

Several different co-location scenarios (Vertical and Horizontal) have been studied as well as different types of coupling loss. The coupling loss between aggressor and victim have been studied considering Array-to-Array (A-to-A) coupling loss and Array-to-Sub-Array (A-to-SA) with and without beamforming.

Table 2.2-1: In-band BS-to-BS coupling loss

| Scenario | Coupling type | Type | Coupling loss (dB) |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7.0 GHz |  |
| Vertical | A-to-A, passive antenna | Measurement | 54 | 52 | 56 |  | ZTE, R4-2501875 |
| Horizontal | A-to-A, passive antenna | Measurement | 31 | 34 | 46 |  | ZTE, R4-2501875 |
| Horizontal | A-to-SA, beamforming | Simulation |  | 55±20 |  |  | Ericsson, R4-2418398 |
| Horizontal | A-to-A | Simulation |  |  |  | 77±25 | Ericsson, R4-2502160 |
| Horizontal | A-to-SA | Simulation |  |  |  | 70±20 | Ericsson, R4-2502160 |

When evolving the co-location requirement concept there are two essential aspects to consider:

- By tradition BS-to-BS isolation have been set to 30 dB. The question is if this assumption still relevant for co-location situations now spanning from 410 MHz up to 7125 MHz. Indications shows that for derivation of requirement levels for bands in the upper region a value different than 30 dB should be used.

- To find general test approach, the requirement level should be anchored as an OTA parameter. This would allow any antenna to be used rather than a specific antenna such as the Co-Location Test Antenna (CLTA).

In Figure 2.2-1, different approaches used for co-location requirements is visualised. For BS type 1-C and BS type 1-H the requirements are defined as conducted requirements at the Transceiver Array Boundary (TAB) interface. For BS type 1-O a new requirement concept was introduced, where a CLTA is used to convert the emission and interferer signal to a conducted interface. If would be natural to evolve the colocation requirements to true OTA requirements defined at the Radiated Interface Boundary (RIB).

# Figure 2.2-1: Co-location requirement evolution

Observation 1: Based on the work done so far in Rel-19 BS RF evolution WI, we can conclude that both measurement and simulation results indicate that 30 dB BS-to-BS isolation may not be a relevant assumption for co-location requirement derivation for bands in the upper region of FR1.

It is essential to understand BS-to-BS coupling characteristics to be able to evolve the co-colocation requirement concept and corresponding test methods. In addition, its essential to study test aspects, such as alternative measurement antennas. In Rel-19, attempts to use a common measurement antenna, such as a horn antenna have been conducted.

It would be beneficial to define the requirement in such a way that the need for a very specialized antenna such as the CLTA goes away. This can be done is the requirement is anchored as an OTA parameter at RIB. Potential OTA parameters to consider is power density and field strength levels, in the similar fashion as used today for EMC requirements. Having the requirement defined at RIB would open up for solving issues previously indicated for the current BS type 1-O requirement based on the CLTA.

Observation 2: To further evolve the co-location concept, it is essential to understand BS-to-BS isolation for relevant co-location situations and how to measure emission and inject an interferer properly. Both these aspects have been studied in Rel-19, proving a good start for follow up work in Rel-20.

  - OTA test enhancement

The work aims to improve the technical quality of OTA conformance test specifications for network nodes (BS, IAB, repeaters), to address significant testing time for at least some of OTA requirements. Issues related to technical relevance and exaggerated test scope have been identified for some requirements for BS type 1-O, e.g., Transmitter Intermodulation (TX IMD) requirement testing and receiver out-of-band blocking requirement testing. Improve description of OTA unwanted emission test methods in TS 38.141-2, Annex I. Evolving the BS test specification for BS type 1-O is essential for the evolution of BS RF specifications preparing specifications to facilitate the industry to switch to all OTA conformance testing for AAS and M-MIMO capable BS. Moving to all OTA conformance testing is essential for BS operating in bands defined in the upper region of FR1 and above. This work can be divided into two sub-categories:

- Improvements of TRP test methods in TS 38.141-2, Annex I.

- Reduction of test scope for TX IMD requirement and receiver blocking requirement in TS 38.141-2.

Status: Good discussion on improvements for co-location requirements and test scope reduction. The discussion has resulted in some ideas how to reduce the test scope for the TX IMD requirement. For receiver blocking it have been noticed that the number of frequencies for the interferer signal can be reduced to save test time. At this stage, reduction of number of TX IMD requirement test frequencies need to be settled.
 
Eventually, when all details are agreed draft CRs to TS 38.141-2 can be created to implement updates to achieve the goal to reduce the test scope for TX IMD and Out-of-band receiver blocking.

  - Re-evaluation of co-existence requirement levels

In the Liaison Statement (LS) from ETSI MSG TFES [7] RAN4 received a request to re-evaluate co-existence transmitter spurious emission requirement levels for BS. RAN4 have acknowledged the LS in [8]. The current requirement levels were originally determined based on old assumptions summarized in contribution [9] presented at RAN4#113. These assumptions were relevant for UTRA operating at 2 GHz with network deployment assumption relevant for UTRA networks deployed 1999. The re-evaluation would include new aspects such as new bands at higher frequencies and AAS BS to be considered. Proper and relevant requirement levels are essential to guarantee operation of multiple IMT networks in the same geographical area with reasonable complexity and cost.

Status: The work will start this meeting. It is essential to define re-evaluation methodology and simulation parameters at an early stage of the work, since the Rel-19 time schedule is limited.

It can be noticed that the time left in Rel-19 is limited, hence the work should focus on bands above 5.9 GHz.

# Conclusion

In this document we summarize the status of the work in the ongoing Rel-19 BS RF evolution WI. Progress is made several of running work packages. The outcome for some work packages will produce CRs to Rel-19 specifications, while some work packages will study essential aspects related to requirement background and test aspects.

Based on the information we observe following:

Observation 1: Based on the work done so far in Rel-19 BS RF evolution WI, we can conclude that both measurement and simulation results indicate that 30 dB BS-to-BS isolation may not be a relevant assumption for co-location requirement derivation for bands in the upper region of FR1.

Observation 2: To further evolve the co-location concept, it is essential to understand BS-to-BS isolation for relevant co-location situations and how to measure emission and inject an interferer properly. Both these aspects have been studied in Rel-19, proving a good start for follow up work in Rel-20.

It is essential to use the Rel-19 period to study and improve essential aspects making 5G advanced a solid foundation for 6G.

# References

[1]	R4-2500881, “General aspects related to BS RF evolution WI”, Ericsson

[2] 	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC

[3]	RP-250817, “Moderator's summary for RAN4 led REL-20 topics”, RAN4 Chair (Huawei)

[4]	R4-2503725, “Rel-20 BS RF package objectives”, Ericsson

[5]	RP-250436, “Status report for NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE

[6]	R4-2502279, “Way Forward for [114][303] NR_BS_RF_Part2_CLTA”, Huawei

[7]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[8]	R4-242447, “Reply LS to ETSI MSG TFES on Tx spurious emissions: co-existence requirements limits for high frequency bands”, Huawei

[9]	R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson