3GPP TSG-RAN WG4 Meeting #114bis	R4-2503740

Wuhan, China, 7th – 11st April 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters for NR NTN Ku band

# Agenda item:	7.11.3

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a revised Work Item [1] was approved on introduction of Ku Band for NR NTN with the following RAN4 objective. In this contribution, we want to share some further views about NR NTN Ku band from system parameters perspective.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

# 2 Discussion

- Sub-topic 1-2: Definition of extended FR1-NTN and FR2-NTN frequency ranges Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN (Note 1); 410 MHz – 14500 MHz. FR2-NTN (Note 2); 10700 MHz – 30000 MHz. NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.. - Sub-topic 1-3: Ku bands definition Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504 MHz; FDD. NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT... Table 5.2.3-2: Satellite operating bands in FR1-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n248; 14000 MHz - 14500 MHz; 10700 MHz – 127501 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 127501 MHz; FDD. NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT... - Sub-topic 3-6: FR1-NTN ARFCN definitions Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN NTN satellite operating band; ΔFRaster (kHz); Uplink Range of NREF (First – <Step size> – Last); Downlink Range of NREF (First – <Step size> – Last). n248; 30; 1333334 – <2> – 1366666; 1113334 – <2> – 1250000. n247; 30; 1316668 – <2> – 1333332; 1113334 – <2> – 1250000. - Sub-topic 3-7: FR2-NTN ARFCN definitions Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN SAN operating band; ΔFRaster (kHz); Uplink range of NREF (First – <Step size> – Last); Downlink range of NREF (First – <Step size> – Last). n509; 120; 1333336 – <8> – 1366664; 1113336 – <8> – 1250000. n508; 120; 1316672 – <8> – 1333328; 1113336 – <8> – 1250000. - Sub-topic 3-8: FR1-NTN GSCN definitions Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN) NTN satellite operating band; SS Block SCS; SS Block pattern1; Range of GSCN (First – <Step size> – Last). n248; 30 kHz; Case C; 12850 – <1> – 14266. n247; 30 kHz; Case C; 12850 – <1> – 14266. - Sub-topic 3-9: FR2-NTN GSCN definitions Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN) SAN operating band; SS Block SCS; SS Block pattern (NOTE); Range of GSCN (First – <Step size> – Last). n509; 120 kHz; Case D; 12864 – <12> – 14256. n508; 120 kHz; Case D; 12864 – <12> – 14256. - Sub-topic 3-10: Default Tx Rx frequency separation Agreement: Variable TX RX spacing is assumed, how it is specified is FFS. Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.

- The above is the agreement in last meeting. We already reached some agreements on band definition, ARFCN and GSCN definitions for 30 kHz and 120 kHz SCS. In RAN #107 meeting, the approved revised WID includes 15 kHz SCS for FR1-NTN in Ku band. We need to define the corresponding channel bandwidths, ARFCN and GSCN definitions for 15 kHz for Ku band.

- Channel bandwidths

- Based on the objective in the revised WID, we propose to define the following channel bandwidths for Ku band for two sets of bands.

Table 2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

Table 2-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

Proposal 1: It is proposed to define the following channel bandwidths for Ku band for two sets of bands.

- Channel raster and sync raster

- Based on the above, we propose to define the channel raster entries and SS raster entries for Ku band in FR1-NTN.

Table 2-3: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316667 – <1> – 1333332 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Table 2-4: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14232 |

- Proposal 2: It is proposed to define the channel raster entries and SS raster entries for Ku band in FR1-NTN as follows.

- TX-RX frequency separation

- In last meeting, we agreed that variable TX RX spacing is assumed. So we propose to specify the following TX-RX frequency separation for Ku band in FR1-NTN.

Table 2-5: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n248 | -1260 to -3790 MHz |
| n247 | -1010 to -3290 MHz |

- Proposal 3: It is proposed to specify the following TX-RX frequency separation for Ku band in FR1-NTN.

# 3 Conclusion

In this contribution, we shared some views on system parameters for NTN Ku band and the proposal is made as follows:

Proposal 1: It is proposed to define the following channel bandwidths for Ku band for two sets of bands.

Table 2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

Table 2-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

- Proposal 2: It is proposed to define the channel raster entries and SS raster entries for Ku band in FR1-NTN as follows.

Table 2-3: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316667 – <1> – 1333332 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Table 2-4: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14232 |

- Proposal 3: It is proposed to specify the following TX-RX frequency separation for Ku band in FR1-NTN.

Table 2-5: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n248 | -1260 to -3790 MHz |
| n247 | -1010 to -3290 MHz |

# 4 Reference

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat et al.

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)