3GPP TSG-RAN WG4 Meeting #114bis	R4-2503745

Wuhan, China, 7th – 11st April 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for IoT-NTN TDD band

# Agenda item:	7.31.3

# Document for:	Approval

# 1 Introduction

In RAN#106 meeting, a revised Work Item [1] was approved on introduction of introduction of IoT-NTN TDD mode with the following objective. In this contribution, we want to share some views about the new NB-IoT TDD operating NTN band from system parameters and UE RF requirements perspective.

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

# 2 Discussion

  - 2.1 Reference sensitivity level

Issue 3-3-1: NTN NB-IoT SAN FRC for reference sensitivity and in-channel selectivity Agreement: RAN4 to further discuss FRC for reference sensitivity and in-channel selectivity in SAN TS 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8). - Option 1: Further discuss if to reuse A14-1 reference channel and segment the signal into 2 different 8 ms Tx/Rx times (e.g. with 90 ms periodicity between 2 transmissions, if 1 transmission can be postponed) for TDD NTN NB-IoT SAN requirements. Further discuss if A14-2 could be descoped for TDD NTN NB-IoT SAN requirements. - Option 2: Further discuss if to define new FRC(s) for NTN NB-IoT to cope with 8 ms Tx/Rx time, other options are not precluded. Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity Reference channel; A14-1; A14-2. Sub-carrier spacing (kHz); 15; 3.75. Number of tone; 1; 1. Diversity; No; No. Modulation; π/2 BPSK; π/2 BPSK. Frequency offset; 0; 0. Channel estimation length (ms) (NOTE); 4; 16. Number of NPUSCH repetition; 1; 1. IMCS / TBS; 0 / 0; 0 / 0. Payload size (bits); 32; 32. Allocated resource unit; 2; 2. Code rate (target); 1/3; 1/3. Code rate (effective); 0.29; 0.29. Transport block CRC (bits); 24; 24. Code block CRC size (bits); 0; 0. Number of code blocks - C; 1; 1. Total number of bits per resource unit; 96; 96. Total symbols per resource unit; 96; 96. Tx time (ms); 16; 64. NOTE: Channel estimation lengths are included in the table for information only..

Regarding FRC for reference sensitivity level, the issue is whether to modify FRC parameters to better align to 8ms TDD UL transmission time. From our understanding, Tx time in FRC parameters table is determined by the number of allocated resource unit. In NB-IoT, it introduced the definition of resource unit, which is defined as following table. For 3.75 kHz SCS case, there is only one definition of resource unit with single tone and 32ms duration. Any revision of FRC parameters to accommodate the 8ms TDD UL transmission time would necessitate the introduction of new resource unit definitions. This is not a straightforward modification, as it would require RAN1 to define.

Table 2.1-1 The definition of resource unit for NB-IoT

|  | Frequency (tone) | Time (ms) |
| --- | --- | --- |
| SCS=15 kHz | 1 | 8 |
|  | 3 | 4 |
|  | 6 | 2 |
|  | 12 | 1 |
| SCS=3.75 kHz | 1 | 32 |

In RAN1 discussion, when a signal cannot be fully transmitted within one 8ms UL transmission time, it is permissible to continue the transmission in subsequent 8ms UL transmission opportunities. Therefore, we prefer option to reuse reference channel in A.14 for REFSENS and segment the signal into 2 different 8 ms Tx/Rx times.

Proposal 1: It is proposed to reuse reference channel in A.14 for REFSENS for IoT-NTN TDD band 249 and segment the signal into 2 different 8 ms Tx/Rx times.

In terms of REFSENS for IoT-NTN, it is defined by the following equation and it is band agnostic. For IoT-NTN TDD band 249, the existing REFSENS in Table 7.2.2-3 and Table 7.2.2-4 in 36.108 can be reused.

PREFSENS = -174+10*log10(BW) + NF + IM + SNR

Proposal 2: It is proposed to reuse the existing REFSENS requirements in Table 7.2.2-3 and Table 7.2.2-4 in 36.108 for IoT-NTN TDD band 249.

  - 2.2 Dynamic range

Issue 3-3-2: NTN NB-IoT SAN FRC dynamic range Agreement: RAN4 to further discuss FRC for dynamic range in SAN TS 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8). - Further discuss if reference channel A15-1 can be reused for NTN TDD IoT NTN. - Further discuss if reference channel A15-2 can be descoped for NTN TDD IoT NTN. - Other options are not precluded. Table A.15-1 FRC parameters for NB-IoT dynamic range Reference channel; A15-1; A15-2. Subcarrier spacing (kHz); 15; 3.75. Number of tone; 1; 1. Modulation; π/4 QPSK; π/4 QPSK. Diversity; No; No. Frequency offset; 0; 0. IMCS / ITBS; 7 / 7; 7 / 7. Payload size (bits); 104; 104. Allocated resource units; 1; 1. Transport block CRC (bits); 24; 24. Coding rate (target); 2/3; 2/3. Coding Rate; 0.67; 0.67. Code block CRC size (bits); 0; 0. Number of code blocks – C; 1; 1. Total symbols per resource unit; 96; 96. Total number of bits per resource unit; 192; 192. Tx time (ms); 8; 32. Frequency offset; 0; 0. Channel estimation length (ms) (NOTE); 4; 16. NOTE: Channel estimation lengths are included in the table for information only..

With regard to FRC for dynamic range, we also think reference channel in A.15 can be reused for dynamic range. If Tx time is more than 8ms, the signal could be segmented into 2 different 8 ms Tx/Rx times.

Proposal 3: It is proposed to reuse reference channel in A.15 for dynamic range for IoT-NTN TDD band 249.

For dynamic range, it is also band agnostic. For IoT-NTN TDD band 249, the existing dynamic range in Table 7.3.2-2 and Table 7.3.2-3 in 36.108 can be reused.

Proposal 4: It is proposed to reuse the existing REFSENS requirements in Table 7.3.2-2 and Table 7.3.2-3 in 36.108 for IoT-NTN TDD band 249.

# 3 Conclusion

In this contribution, we shared some views on the impact on RF due to the periodic pattern and the observations are made as follows:

Proposal 1: It is proposed to reuse reference channel in A.14 for REFSENS for IoT-NTN TDD band 249 and segment the signal into 2 different 8 ms Tx/Rx times.

Proposal 2: It is proposed to reuse the existing REFSENS requirements in Table 7.2.2-3 and Table 7.2.2-4 in 36.108 for IoT-NTN TDD band 249.

Proposal 3: It is proposed to reuse reference channel in A.15 for dynamic range for IoT-NTN TDD band 249.

Proposal 4: It is proposed to reuse the existing REFSENS requirements in Table 7.3.2-2 and Table 7.3.2-3 in 36.108 for IoT-NTN TDD band 249.

# 4 Reference

- RP-243293, Revised WID on introduction of IoT-NTN TDD mode, Iridium Satellite LLC

- R4-2502292, Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF, Moderator (THALES)