# 3GPP TSG-RAN WG4 Meeting # 114bis		  R4-2503747

# Wuhan, Hubei, China, 07th – 11th April, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on band definition and system parameters for supporting new SDO band for LTE-based 5G Broadcast

# Agenda Item:	7.9.2

# Document for:	Approval

- Introduction

During RAN#107 meeting, a new Work Item [1] has been approved on specifying a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO. In this contribution, band definition and system parameters are discussed. The objective of this WI is shown as below:

Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO:

  - Specify related UE RF core requirements for 10MHz PMCH

    - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as  baseline (reuse assumption of 10 MHz UE Rx filter)

  - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements:

    - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline

    - Requirements based on Resolution 761 in WRC-19

    - Only consider QPSK and 16QAM modulation

Note: UE only needs to support 5G broadcast for this band

Note: Release independence applicability will be further discussed

- Discussion

- 2.1 New Suffix for LTE based 5G terrestrial broadcast

In TS 36.102, suffix A is used for Cat-M1 UE, and suffix B is used for NB1/NB2 UE, as shown in table 4.3-1 below:

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| A | Cat-M1 |
| B | NB1, NB2 |

So for 5G terrestrial broadcast UE, suffix C can be used sequentially.

Proposal 1: To use suffix C for the new defined 5G terrestrial broadcast UE.

2.2 Operating band

In RAN4#114，two Ku FDD band has been agreed for NR NTN, where bands n248 and n247 are used. We think the band number 248 and 247 should be reserved. So it is suggested band number 246 is used for the new SDO band operating in 1467 to 1492 MHz.

Proposal 2: The new SDO band operating in 1467 to 1492 MHz can be numbered as 246.

2.3 Transmission bandwidth configuration(NRB)

According to the WID, only channel bandwidth 10 MHz needs to be specified, so 50 PRBs can be considered for the Transmission bandwidth configuration according to Table 5.6-1 in TS 36.101.

Proposal 3: To define 50RB transmission bandwidth configuration for 10MHz channel bandwidth.

2.4 Channel arrangement

2.4.1 Channel spacing

For broadcast operation, it can be seen the Nominal Channel spacing is the PMCH bandwidth in TS36.101 for LTE based 5G terrestrial broadcast, following the same logic, channel spacing for LTE-based 5G Broadcast operating for satellite is also equivalent to PMCH bandwidth.

Proposal 4: To reuse the same PMCH channel spacing for LTE-based 5G Broadcast operating for satellite.

2.4.2 Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster ΔFRaster is 100 kHz, and the step size for the channel raster is given as <1>. This can be reused for LTE based 5G terrestrial broadcast.

For the channel number of new proposed band 246, the key point is how to decide the start number of NDL. The newest channel E-UTRA channel numbers for FDD band is 252 of which the range of NDL is 228301 –<1>- 228500, a reasonable way is to start the NDL of the new proposed band 246 after band 252. Then the upper bound of 246 can be calculated as: 228301-1 = 228300. So the channel number of 246 can be determined according to the 25 MHz frequency range as 228051 -<1>- 228300.

Proposal 5: The channel numbers for LTE based 5G terrestrial broadcast band are proposed as 228051 -<1>- 227480:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 246 | 100 | 1467 | 228051 | 228051 -<1>- 228300 | N/A |  |  |

- Conclusion

In this contribution, some initial views on band definition and system parameters are provided and the proposals ares made as following:

Proposal 1: To use suffix C for the new defined 5G terrestrial broadcast UE.

Proposal 2: The new SDO band operating in 1467 to 1492 MHz can be numbered as 246.

Proposal 3: To define 50RB transmission bandwidth configuration for 10MHz channel bandwidth.

Proposal 4: To reuse the same PMCH channel spacing for LTE-based 5G Broadcast operating for satellite.

Proposal 5: The channel numbers for LTE based 5G terrestrial broadcast band are proposed as 228051 -<1>- 228300:

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 246 | 100 | 1467 | 228051 | 228051 -<1>- 228300 | N/A |  |  |

- Reference

“RP-250788, New band for 5G Broadcast Utilizing Geosynchronous Satellite.”

3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access"