3GPP TSG-RAN WG4 Meeting #114 bis	   R4-2503750

# Wuhan, China, 7th – 11st , April, 2025

Agenda Item:	7.23.3.1

Source: 	Ericsson

Title:	Discussion on RRM requirements for UE-to-UE CLI handling (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#114 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

# 2.1 UE to UE CLI handling

### General issues

#### Issue 1-1-3: Measurement resources - Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

For L1-SRS-RSRP, the measurement resource will be within UL subband, the measurement does not need to be done simultaneously across two DL subband.

For L1-CLI-RSSI, within DUD configuration, the UE have to perform measurement across two DL subbands at the same time.

From measurement capability point of view, there is a maximum number of measurement resource of RSSI can be configured and the RAN4 requirements is set based on agreed maximum number of RSSI measurement resource that the UE can monitor. There is no frequency continuity of this monitoring capability, we don’t see why UE can monitor RSSI measurement resource to multiple number across entire Active BWP while can not combine the measurement results into one wideband report.  Based on implicit agreements regarding wideband reporting, the frequency location of the measurement resource configured does not need to be reflected in the reporting.

Also to mention, the RSSI measurement does not need to be done after the demodulation, there is no reference signal sequence, we don’t think there is any technical challenge for the UE to measure RSSI for 2 DL subband simultaneously.

RAN4 also agreed that the RSSI measurement will be based on single shot which is the same as legacy, we think the measurement period shall follow the single shot agreement.

- Proposal 1: RAN4 shall follow the L1-CLI-RSSI measurement agreement for single shot for DUD configuration, 1 sample on the measurement period.

#### Issue 1-1-5: Scheduling restriction - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

For scheduling restriction applicability, we think it is good to wait for RAN1 conclusion on whether to support SRS FDM-ed with DL, in principle, since the L1-SRS-RSRP follow the same measurement timing as Rel-16, the restriction causing by timing error is still valid. We did calculation on the ratio between the symbol duration and T_error for both intra and inter cell case.

For Y number of symbol before the measurement symbol, since Rel-16 requirement was being set based on the inter-cell scenario (worst case) for example if we look at the FR1 60KHz SCS case, we think the number of symbols for FR2, the Rel-16 numbers can be reused.

- Proposal 2: The scheduling restriction due to L1-SRS-RSRP on Y symbol before measurement symbol can be defined as: for FR1,15KHz SCS ,1 symbol, 30KHz SCS, 1 symbol, 60KHz SCS, 2 symbols; for FR2, 60KHz SCS, 1 symbol, 120KHz SCS, 2 symbols.

We would like to clarify the root causes behind the scheduling restrictions for performing CLI measurements. Based on our understanding, there are three main reasons for these restrictions:

- UE requires time to switch between transmission and reception (Tx-Rx or Rx-Tx).

- Measurement timing errors, the measurement time may deviate from the downlink (DL) reference timing. As a result, data symbols cannot be scheduled if the UE only has a single FFT for processing both data and measurement simultaneously. However, if the UE has separate FFT resources for processing data and measurement sequences, this restriction will not apply.

- UE operates in half-duplex mode, meaning it cannot transmit and receive simultaneously.

Regarding the first root cause, which relates to scheduling restrictions caused by the UE’s switching between uplink (UL) and downlink (DL), we believe this is already addressed in the RAN1 specification, specifically in TS 38.211, Clause 4.3.2.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than Rx-Txc after the end of the last received downlink symbol in the same cell where Rx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than Tx-Rxc after the end of the last transmitted uplink symbol in the same cell where Tx-Rx is given by Table 4.3.2-3. ……

If we further check the restrict symbol number from Rel-16 requirement, there is no switching time considered in the restriction symbol before the measurement symbol. We think same logic shall be consider here for both method #1 , method#2 and method#3 in regarding whether x symbol is needed for the symbol before the measurement symbol.

- Observation 1: The UL/DL switching time for the Rel-16 legacy requirement is being covered from RAN1 specification TS 38.211 clause 4.3.2, not being counted for the scheduling restriction in RAN4 requirements .

For the symbol after the measurement symbol, the root cause is due to whether UE need to switch from receiving measurement to transmit uplink data. Let’s assume there is UL traffic right after the measurement symbol, which can be avoid by network configuration of the TDD pattern. Even when there is UL traffic right after the measurement symbol, the UL/DL switching time shall follow the same logic as the symbol before measurement symbol, which is covered by the RAN1 specification.

- Proposal 3: No need to specify scheduling restriction for the symbol after the measurement symbol .

- For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

One thing to notice for Rel-19 L1-CLI measurement, the difference from Rel-16 L3-CLI measurement is the measurement timing. The legacy L3-CLI measurement follow the same timing as L3-SRS-RSRP which is DL reference timing with an error up to UE implementation.

Based on RAN4 agreement on L1-CLI measurement method#1 and method #3, DL reference timing will be used.

- Observation 2: Different measurement timing is agreed for L1-RSSI measurement, no error will apply to the DL reference timing.

The causing of the scheduling restriction for the symbol before measurement symbol due to timing error is not valid in the L1-RSSI measurement anymore.

- Proposal 4: RAN4 do not specify scheduling restriction for the symbol before the L1-RSSI measurement symbol due to no timing error for L1-RSSI measurement for both method#1 and method#3.

### L1-SRS-RSRP measurement requirements

#### Issue 1-2-1: Side conditions #### Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement - Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition. - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB

From network total performance point of view, the side condition not only impact the accuracy requirement however also impact the requirement coverage. Due to the flexible configuration of the SBFD for DUD and DU, the BW of the UL subband needs to be declared by the base station. Based on RF session agreement and existing RB configuration table from TS 38.104 table 5.3.2-1, we see there is case that the SRS BW cannot be guaranteed for more than 48 RBs for FR1.

- Proposal 5: For L1-SRS-RSRP measurement, SRS BW for setting minimum requirement shall be 24RBs to have better requirement coverage .

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN 4 shall follow the L1-CLI-RSSI measurement agreement for single shot for DUD configuration, 1 sample on the measurement period.

	Proposal 2: The scheduling restriction due to L1-SRS-RSRP on Y symbol before measurement symbol can be defined as: for FR1,15KHz SCS ,1 symbol, 30KHz SCS, 1 symbol, 60KHz SCS, 2 symbols; for FR2, 60KHz SCS, 1 symbol, 120KHz SCS, 2 symbols.

	Proposal 3: No need to specify scheduling restriction for the symbol after the measurement symbol .

	Proposal 4: RAN4 do not specify scheduling restriction for the symbol before the L1-RSSI measurement symbol due to no timing error for L1-RSSI measurement for both method#1 and method#3.

	Proposal 5: For L1-SRS-RSRP measurement, SRS BW for setting minimum requirement shall be 24RBs to have better requirement coverage .

# References

- R4-2502699 “WF on SBFD RRM”, Huawei, February ,2025

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024

# Annex

3GPP TSG-RAN WG4 Meeting #114 bis	   R4-250xxxx

# Wuhan, China, 7th – 11st , April, 2025

Title:	Reply to LS on L1 based UE-to-UE CLI measurement and reporting

Response to:

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Zhanxian Wang

griselda.wang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

The following working assumption was made in RAN1 on L1-based CLI reporting since RAN1 specifications must capture the format and bitwidths of the CLI reports.

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

RAN1 assumes that the reporting range and granularity for L3 SRS-RSRP measurement in Table 10.1.22.1.2-1, TS38.133 can be reused for L1 SRS-RSRP. In addition, the reporting range and granularity for L3 CLI-RSSI measurement in Table 10.1.22.2.2-1 of TS38.133 can be reused for L1 CLI-RSSI.

For a CLI report, RAN1 agreed that the absolute value of the most interfered (largest) measured L1-SRS-RSRP or L1-CLI-RSSI is reported. RAN1 also agreed to use differential reporting for L1 SRS-RSRP and L1 CLI-RSSI when the number of reported measurement resources is larger than one, in the same manner as for existing L1-RSRP/SINR reporting.

The answer to RAN1’s question for L1-CLI measurements:

- Is the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI in the RAN1 working assumption reasonable from RAN4 perspective?

Yes, the reporting range and granularity for L1-CLI working assumption is reasonable.

- Is the range and resolution for differential reporting of L1 SRS-RSRP and CLI-RSSI reporting reasonable from RAN4 perspective?

Yes, the resolution for differential reporting for L1-CLI reporting is reasonable.

# 2	Actions

To RAN1

ACTION: RAN4 respectfully ask RAN1 to take the reply into consideration of further discussion.

# 3	Dates of next TSG RAN WG1 meetings

RAN1#121	19th – 23rd May 2025	 		Malta

RAN1#122    25th -29th August 2025		India