3GPP TSG-RAN WG4 Meeting #114 bis	   R4-2503751

# Wuhan, China, 7th – 11st , April, 2025

Agenda Item:	7.23.3.2

Source: 	Ericsson

Title:	Discussion on RRM impacts for SBFD operations (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#114 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## 2.1 Other RRM core requirements for SBFD operation

### Impact on SSB based measurements

#### Issue 1-2-4: Requirements for scheduling restriction #### Agreements (from Wed AH session) #### For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. #### Further discuss the impact to inter-frequency SSB measurement without gap case.

From legacy TDD since Rel-15, the DL/UL transition time is specified TS 38.211 clause 4.3.2.

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than Rx-Txc after the end of the last received downlink symbol in the same cell where Rx-Tx is given by Table 4.3.2-3. A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than Tx-Rxc after the end of the last transmitted uplink symbol in the same cell where Tx-Rx is given by Table 4.3.2-3.

For the Rel-19 SBFD aware UE, this legacy requirement shall still be valid since the SBFD aware UE is not capable of full-duplex. There is no need in general to specify scheduling restriction due to the TX/RX or RX/TX switching.

For the case right after SSB measurement, the existing RAN1 agreement for collision case

- Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

Our interpretation of the priority rule agreed on RAN1 is based on the exact OFDM symbols that contains SSB. Within one DL slot the typical configuration of the SSB is starting from symbol#2 and the symbol#8 span over 4 OFDM symbols. In this case the symbols that not covered by the SSB within 1 slot are only 2 OFDM symbols. Also considering there will be switching gaps being introduced from gNB point of view based on the SBFD and non-SBFD slot, the scheduling restriction on these 2 symbols is not necessary.

- Proposal 1: Scheduling restriction on symbol level right after the SSB measurement within the SBFD slot is not necessary.

### Impact on CSI-RS based measurements

RAN4#114 agreements were reached for CSI-RS:

- Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

For the DU configuration within the SBFD symbols, the continuous CSI-RS resource can be guaranteed. However, in the DUD configuration, this CSI-RS resource continuity in the frequency domain cannot be guaranteed due to there is an UL subband within the 2DL subband.

From network performance point of view, the accuracy requirement shall not only consider the UE capability but only consider network configuration limitation to guarantee the measurement requirement applicable.

Based on the RAN1 LS reply [3], we would like to separate the discussion between RLM/BFD/CBD measurement from the CSI-RS L1-RSRP and LI-SINR. Our view that CSI-RS based RLM/BFD/CBD shall not be within the scope of L1 measurement discussion.

The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig.

For the case 1 and case 2, we don’t think there shall be any impact from the legacy CSI-RS L1-RSRP measurement, since the CSI-RS BW is being guaranteed for at least one subband. The measurement accuracy requirement per at least 1 subband shall remain the same as legacy.

The difference between case1 &2 and the legacy CSI-RS L1-RSRP is the reporting granularity in frequency location.

For case1, since both DL subband fulfill the minimum CSI-RS BW, the CSI-ReportConfig will point to specific CSI-RS resource set. The only issue left is whether this two DL subband can be combined from results processing point of view.

For CSI processing time for CQI etc the RAN1 agreement on the processing time can be reused here.

RAN1#118-bis meeting agreement: Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

- Proposal 2: For DUD configuration, the measurement period for CSI-RS based L1-RSRP/L1-SINR can be doubled with the assumption that UE measures one DL subband on each CSI-RS resource occasion .

For Case 2, since only 1 DL subband fulfill the minimum BW, there are 2 options to handle the requirements for this case.

- Option 1: 1 set requirement and the requirement only applicable for the DL subband that fulfill the minimum

- Option 2: 2 sets of requirements, one for X3 which can be legacy 48PRBs and the other for X3’ which can be smaller X3 but larger than certain value for example 24 PRBs.

Based on current agreement from RF session.

Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead.

We analysis the typical 40MHz total BW case to see how to guarantee the CSI-RS BW configuration figure 1. If we add X3’ value from 24PRBs on top of the 48PRBs we can increase the requirement coverage by 35%, even though still 45% of the potential configuration will end up have no requirements.

Figure 1: typical configuration of the DUD of SBFD PRBs numbers within 1 DL subband

We think 2 set requirements can cover the requirement in a better way and the 3 cases can all be covered.

- Case 1: X1 >= 48PRBs and X2 >= 48PRBs, requirement set 1

- Case 2: X1 >= 48PRBs and 24PRBs<X2 < 48 PRBs, can be further discussed

- Case 3: X1 >= 24PRBs and X2 >= 24PRBs, requirement set 2

- Proposal 3: 2 sets of requirements can be defined based on 48PRBs and 24PRBs for CSI-RS based L1-RSRP and L1-SINR.

#### Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

For CSI-RS-based L3 measurements, we believe that without signaling for the neighbor cell SBFD configuration, or without knowing the frequency start point and the number of PRBs allocated for the CSI-RS, it is unclear how an SBFD-aware UE can perform measurements unless it operates as a legacy UE.

For DU configuration, the requirement is simpler. If the CSI-RS is punctured in the frequency domain and does not meet the bandwidth conditions, then no requirements will apply.

However, for DUD configuration, the situation is more complex. When considering legacy UE operation in an SBFD-capable base station, CSI-RS-based L3 measurements will be impacted because certain TDD downlink symbols are unavailable to legacy UEs. As a result, measurement performance will degrade, as certain samples must be dropped. However, this performance degradation is not captured within the work item.

For SBFD-capable UEs, referring to the typical bandwidth based on base station declarations in Figure 1, applying the legacy 48 PRBs bandwidth condition would result in more than 55% of declared configurations not meeting the requirements.

L3 measurement reports are intended for mobility decisions. To ensure fair decision-making between SBFD and non-SBFD capable UEs, we believe it is difficult to specify appropriate requirements for DUD configuration. This difficulty arises because the network cannot guarantee requirement coverage for more than half of the configurations, and SBFD-aware UEs cannot perform measurements based on their SBFD capability due to the lack of neighbor cell SBFD configuration.

The proper resolution to this issue is to request that RAN2 support SBFD-aware UEs by providing the neighbor cell SBFD configuration.

- Proposal 4: RAN2 neighbour cell SBFD configuration is necessary for setting up proper CSI-RS based L3 requirements. RAN4 can send LS to RAN2 to ask for supporting the configuration.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: Scheduling restriction on symbol level right after the SSB measurement within the SBFD slot is not necessary.

	Proposal 2: For DUD configuration, the measurement period for CSI-RS based L1-RSRP/L1-SINR can be doubled with the assumption that UE measures one DL subband on each CSI-RS resource occasion .

	Proposal 3: 2 sets of requirements can be defined based on 48PRBs and 24PRBs for CSI-RS based L1-RSRP and L1-SINR.

	Proposal 4: RAN2 neighbour cell SBFD configuration is necessary for setting up proper CSI-RS based L3 requirements. RAN4 can send LS to RAN2 to ask for supporting the configuration.

# References

- R4-2502699 “WF on SBFD RRM”, Huawei, February ,2025

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024

- R1-2501560 “Reply LS on CSI-RS measurement with SBFD operation“, Mediatek February ,2025