3GPP TSG-RAN WG4 Meeting #114 bis	   R4-2503752

# Wuhan, China, 7th – 11st , April, 2025

Agenda Item:	7.27.2.3

Source: 	Ericsson

Title:	On general RRM aspects for Rel-19 conditional Intra-CU LTM

Document for:	Discussion

# Introduction

After RAN plenary in September 2024, the following objective has been agreed in the WID [1] for Conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

For RAN4#114 meeting, many open issues are listed in the WF[2]. In this contribution, we will address several aspects which are related to RRM requirements for conditional LTM.

# Discussion

## Scope of the RRM requirement for conditional LTM

Certain agreements have been reached during the RAN4#114 meeting regarding the Rel-19 CLTM scope. One remaining issue is regarding the subsequent CLTM and how to specify RAN 4 RRM requirement for the subsequent CLTM.

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#113 < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC. RAN4#114 < Agreement > - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included. - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

Currently, there is no specified L1 measurement requirement for CSI reporting on deactivated SCC. Instead, deactivated SCCs adhere to an explicitly configured measurement cycle for L3 measurements based on TS 38.321 clause 5.9.

The MAC entity shall for each configured SCell:​ 1> if the SCell is deactivated:​ 2> …​ 2> not report CSI for the SCell;​ 2> not transmit on RACH on the SCell;

The Release-18 LTM candidate cell configuration, based on TS 38.331, allows the network to configure a deactivated SCC as an SpCell within the CellGroupConfig IE, and then afterwards as an LTM candidate cell in the ltm-CandidateConfig Following this configuration, the UE is required to perform L1 measurements for the LTM candidate evaluation after receiving the RRC reconfiguration for the LTM candidate cell configuration. The RAN2 specification only specify the UE behavior not to report for CSI, there is no specified behavior that the UE shall stop from continuing the L1 measurement.

While we acknowledge that current L1 measurement requirements for deactivated SCCs are lacking, once the UE receives the LTM candidate configuration, it should no longer treat the frequency carrier as a deactivated SCC, given that the frequency is designated as an SpCell in the LTM candidate configuration.

From network configuration perspective, we do not observe any differences between the Release-19 CLTM and the Release-18 LTM; the same configuration process is applicable to CLTM.

From the perspective of UE measurement behaviour, it is unlikely for the UE to deviate from the configuration and omit evaluating a specifically configured candidate cell, as the UE performs measurements on the configured frequency carrier.

The SCC is based on the traffic demand whether UE needs to setup another Scell. When the data has been transmitted, there is no need to waste energy consumption for UE to monitor extra Scell. However, from candidate measurement evaluation point of view, when this frequency being configured to the UE, it is not for Scell, it is for the Spcell candidate for LTM/CLTM purpose. The consequence of no L1 measurement requirement on the deactivated SCC will lead to certain frequency will not be evaluated as CLTM/LTM candidate cells, which limits the network and UE choice for candidate cell selection.

- Observation 1: Network may configure deactivated SCC as a spCell and as a LTM candidate for evaluation based on Rel-18 LTM procedure.

- Observation 2: Currently there is no L1 measurement requirement for deactivated SCC to be applied after the LTM candidate configuration even though this frequency is being configured as spCell.

- Proposal 1: Define requirements for L1 measurement on deactivated Scell.

## Conditional LTM early synchronization and TCI state activation

Based on RAN4#114 meeting agreements for DL sync, the early TCI state indication and PDCCH order RACH for early UL sync will be supported as Rel-18 legacy LTM and the CLTM requirements will take the consideration of the early sync and early TCI indication.

Issue 1-2-1: Early UL Synchronization

RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) Do not define RRM requirements for UE based TA measurement. (Apple, MTK) RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

Besides the PDCCH ordered early TA acquisition, the Rel-18 LTM already supports UE based TA measurement, the issue remains for Rel-19 CLTM is whether to define requirements considering the UE based TA estimation.

We acknowledge a valid TA before the cell switch happens will benefit the performance especially for the RACH-less case, however set requirements for UE based TA measurement have certain issue in practice.

Firstly, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. Currently there are proposals on how exactly the TA measurement can be performed, still the detail accuracy will be based on how UE implement the measurement.

UE need to estimate the TA value for the target cell based on current serving cell TA value. If sync between serving and target is ideal, the TAtarget =TAserving+2* (Propagation delaytarget-Propagation delayserving)

For the UE estimated TA value error to be acceptable, the error of the total TA estimation shall be within half CP length. Assuming the sampling time Tc = 0.509 ns, the TA estimation error from UE point of view shall be considering:

- UL to DL reference timing error Te

- TA resolution error:  which is TA adjustment step size16*64/2u

- TA adjustment accuracy: which defined in TS 38.133 7.3.2.2-1

Considering the Synchronization error between target and serving cell, by comparing half CP length with total error Te+ TA resolution error+ TA adjustment accuracy, we can see how much margin left for Sync error between the target and serving from table 1.

| SCS of UL siganl (KHz) | Half CP length (us) | UL to DL error Te | TA resolution error | TA adjustment accuracy | Synchronization error margin (us) |
| --- | --- | --- | --- | --- | --- |
| 15 | 2.345 | 12*64*Tc | 16*64*Tc | 256 Tc | 1.303 |
| 30 | 1.15 | 8*64*Tc | 8*64*Tc | 256 Tc | 0.498 |
| 60 | 0.585 | 3.5*64*Tc | 3.5*64*Tc | 32 Tc | 0.34 |

Table 1: Sync error margin between target and serving.

Considering the existing sync requirement for the network specified in TS 38.104 for TAE values for intra-band non-contiguous CA is 3us, comparing the margin from table 1 with 2*TAE 6us, the margin is too short.  We don’t see proper and accurate UE estimated TA value can be achieved with existing Synchronization requirement.

- Observation 2: Existing sync requirement between gNB will not be able to guarantee the TA estimation accuracy.

Based on current RAN2 signaling design, the network may also send a TA value in a separate MAC CE. When network indicated and UE estimated TA value both exist, the UE shall follow the value based on network ordered TA acquisition. The UE based TA estimation is only for the case that the network has not indicate TA value.

Since how UE measures the TA value is up to UE implementation, also the accuracy still within concern, we don’t see the motivation to set requirements on this matter.

- Proposal 3: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.

## Conditional LTM delay

Issue 1-4-1: General aspects on Subsequent Conditional LTM delay RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.. < Way forward > FFS the following proposals: - Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. - Proposal 1b: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM. - Proposal 2: Wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for subsequent conditional LTM.

The subsequent CLTM is one of another aspect that needs to be further discussed. Based on RAN4#113 meeting agreement, both subsequent CLTM and 1st time CLTM ending point of the delay is the same. The ending point defined as the time when UE transmits the first UL transmission on the target cell.

The only difference to have a generic CLTM delay requirement including both 1st time and subsequent CLTM is the definition of the starting point.

We think the subsequent CLTM will not have the RRC delay in comparing with the 1st time CLTM since the RRC processing delay only exist for the 1st time decoding the RRC message for the CLTM full configuration.

Also to mention the starting point can be different in compare the subsequent CLTM and the 1st time CLTM. The subsequent CLTM starting point can be the time when UE transmits the RRCReconfigurationcomplete for the 1st time CLTM which follows similar logic as the subsequent CPC from Rel-18 legacy.

The Rel-18 LTM interruption is the component Tinterrupt.

- Proposal 4: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .

- Proposal 5: The 1st time CLTM delay can be defined as  DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interuption +   + Tfirst-RS + TRS-proc where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU

- Proposal 6: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU

# Summary

The following have been proposed in the current contribution:

	Proposal 1: Define requirements for L1 measurement on deactivated Scell.

	Proposal 3: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.

	Proposal 4: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .

	Proposal 5: The 1st time CLTM delay can be defined as  DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interuption +   + Tfirst-RS + TRS-proc where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU

	Proposal 6: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.

- R4-2502602 “WF on RRM requirements for NR_Mob_Ph4_Part2”, China Telecom, February, 2025