# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503755

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.22.3.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was discussion on NR MIMO Phase 5, and a WF was agreed [2]. This contribution provides discussion on RRM requirements for event-driven beam management.

- Discussion

Issue 1-4: Components of Event triggered measurement reporting delay? <Way forward > - Option 1: (Apple, vivo, Samsung, E///, MTK) - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel - FFS on whether other components are needed. - FFS the definition of Tfirst UL channel - Option 2: (CMCC, Xiaomi) - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. - Option 3: (Nokia) - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

It was agreed in last meeting that for UE-initiated/event-driven beam reporting, the starting point is to reuse the same definition in L3 event trigger report which is the event is met over the air. The ending point is the point when the UE transmits a first PUCCH triggered by the event.

For legacy L3 based event triggered measurement report, as duplicated as following, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. It can be seen that the definition of L1 based event triggered measurement reporting delay is similar as the definition for L3 based event triggered measurement reporting delay. And it is proposed to follow the approach for L3 measurement report delay to add clarification that this beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

TS 38.133 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

Besides, there is similar discussion on event triggered L1 measurement reporting for Rel-19 mobility WI. It was agreed in last meeting that for event triggered L1 measurement reporting, measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. Since both are for event triggered L1 measurement reporting, we do not see the reason to have different solutions between Rel-19 MIMO and Rel-19 mobility.

- Agreement on event triggered L1 measurement reporting for Rel-19 mobility WI - For event triggered L1 measurement reporting, measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

Proposal 1: it is proposed that the event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on.

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M? <Way forward > - Option 1: (Qualcomm, Apple, Ericsson) - Only define requirements for the number of events is met at once - Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable M - Option 2a: (ZTE) - Event1&Event 2: Case 2: multiple samples M and timer X should be considered - Event1&Event 7: FFS - Option 2b (Samsung) - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. - Option 2c (MediaTek) - For multiple events is met by counter: - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS - C: event instance counter is configured by NW.

According to RAN1 agreements, as duplicated as below, both no time window based and with time window based are supported. Since the basic feature is without time window, it is necessary to specify requirements. For time window based case, in RAN4#113 meeting, it was agreed that it is RAN4 common understanding that no additional filtering applies for this case compared to the case without time window. However, the evalution time may be different since the report is triggerred when at least one same new beam is greater than or equal to a configurable number M. With above consideration, it is proposed to specify requirements to cover both cases of no time window based and with time window based with configurable M.

Agreements in RAN1#117 meeting Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable M.

Issue 1-9: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: (Qualcomm, Nokia, MediaTek(descoped)) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. - Option 2: (Apple, Huawei, Ericsson) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 2a (Apple): The measurement delay is the same as that in serving cell in legacy L1-RSRP - Option 2b (Ericsson): reuse the principle of sharing agreed for SSB based measurement. - Option 3: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Option 4: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

According to RAN1 agreements, on UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management. And in previous meeting, RAN4 also agreed that L1-RSRP is used as measurement metrics for beam report, and agreed to define CSI-RS based L1-RSRP measurement requirements.

The CSI_RS resources configured for new beam measurement can be from serving cell or from the cell with different PCI. CSI-RS is assumed to be QCL’ed with SSB, and the SSB could be the one with different PCI. From this perspective, it is proposed to define CSI-RS based L1-RSRP measurement requirements for both intra-cell and inter-cell.

Proposal 3: it is proposed to define CSI-RS based L1-RSRP measurement requirements for both intra-cell and inter-cell.

Issue 1-14: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: (Apple, Ericsson) - Defined by RAN1. To be discussed later if any RAN4 specific UE capabilities need to be added. - Option 2: (China Telecom) - Align the agreement on whether to define the capability for support of L1 event triggering and reporting in TS 38.133 for R19 MIMO WI and R19 mobility WI. - Option 3: (CMCC, MediaTek) - Define capabilities similar as legacy L3 (like legacy L3 in section 9.1.4).

For legacy event triggered L3 measurement reporting, the capabilities for support of event triggering reporting criteria are defined (section 9.1.4, TS38.133). Similarly, for Rel-19 event triggered L1 measurement reporting, it is necessary to define the capability. According to the discussion in last meeting, some companies mentioned that the number of events to be supported by UE shall be discussed in RAN1 first. We do not have strong view. But at least according to our knowledge, RAN1 have not discussed this. Considering that reporting criteria for L3 are specified in RAN4, it is preferred to define the reporting criteria for L1 in RAN4.

Proposal 4: it is proposed to define capabilities for support of UE-initiated/event-driven beam reporting.

Issue 1-12: Compatibility of capabilities with LTM <Way forward > - Option 1: (Apple, Ericsson) - Discuss it later. - Option 2: (CMCC, Huawei, Nokia, Samsung) - Do not mix/combine Rel-19 mobility and Rel-19 MIMO. Define the capability separately. - Option 4: (MediaTek) - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

Event triggered L1 measurement reporting are considered for both Rel-19 MIMO and Rel-19 Mobility. However, considering that Rel-19 MIMO and Rel-19 Mobility are two parallel on-going work item, it is not prefered to mix the discussion.

Proposal 5: it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

- Conclusion

This contribution provides discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5. The proposals are:

Proposal 1: it is proposed that the event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on.

Proposal 2: it is proposed to specify requirements to cover the case of no time window and the case with time window with configurable M.

Proposal 3: it is proposed to define CSI-RS based L1-RSRP measurement requirements for both intra-cell and inter-cell.

Proposal 4: it is proposed to define capabilities for support of UE-initiated/event-driven beam reporting.

Proposal 5: it is not prefered to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined seperately for Rel-19 mobility and Rel-19 MIMO.

- Reference

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2502595, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part, Samsung

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management

- R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung