# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503758

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.27.2.2

Source: 	CMCC

Title: 			 Discussion on CSI-RS based L1 measurement

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #105 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meetin, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on CSI-RS based L1 measurement.

- Discussion

Progress in in RAN4#113 meeting - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] WF in RAN4#114 meeting Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

We do not think step 2 about SSB-based L1 measurements and reports on the same candidate cell is always needed. With step 2, UE implementation could be simplified, but it also has restriction on the network side. Step 2 implies that network has to configure SSB based LTM even for CSI-RS based LTM. However, according to our understanding, CSI-RS based LTM and SSB based LTM are seperate features and each can work indepedentely. What’s more, if there are SSB based LTM, why network configure CSI-RS based LTM additonally. Based on above consideration, it is proposed to remove step 2. On the other hand, we understand it is related with UE implementation. Based on above consideration, we can compromise that whether step 2 can be skipped or not can be based on UE capability reporting.

Proposal 1: it is proposed that whether step 2 can be skipped or not is based on UE capability reporting.

- Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

For L3 measurement, a measurement is defined as a CSI-RS based intra-frequency L3 measurement provided that SCS/CP type/centre frequency of the CSI-RS resource of the neighbour cell configured for measurement are same as that of the serving cell indicated for measurement. For the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, our preference is to follow the definition of CSI-RS based L3 measurement. Based on this consideration, it is proposed to consider center frequency.

Proposal 2: for the definition of CSI-RS based intra-frequency L1 measurement, except SCS and CP, centre frequency also need to be considered, i.e. the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement.

- Issue 4-4: Measurement type - Issue 4-7: RTD assumption Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP.

CSI-RS based L1 measurement is for mobility, only consider measurement within gap is too limited. And from deployment point of view, it is normal that CSI-RS resources from neighbour cell are outside UE active BWP. In last meeting, RAN1 also agreed that from RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement, which means that both measurement within BWP and measurement outside BWP are considered.

Proposal 3: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario, and it is proposed to define requirements for CSI-RS based L1-RSRP measurement outside active BWP.

In previous meeting, it was agreed that CSI-RS based L1 measurement without gap is supported, and FFS on CSI-RS based L1 measurement with gap. As we discussed above, CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario, and the L1 measurement is performed within measurement gap. It is necessary to define requirements for CSI-RS based L1 measurement with gap.

According to the discussion in last meeting, one concern to support CSI-RS based L1 measurement with gap is the impact on SSB based measurement. Firstly, it is inevitable that the SSB based measurement will be impacted since more measurement objects, e.g. CSI-RS MO(s) , are considered. To minimize the impact on SSB based measurement, there are multiple candidate solutions, e.g. allocating more measurement resources to SSB based measurement. However, the related discussion may be not easy and require a lot effort. Considering limited timeline and workload, one simple way in this release is to assume equally sharing among SSB measurement and CSI-RS measurement.

Proposal 4: it is proposed to define requirements for CSI-RS based L1 measurement with gap.

- For delay requirements definition, for simplicity, equally sharing among SSB measurement and CSI-RS measurement can be considered

From deployment point of view, it is proposed to consider both RTD<CP and RTD>CP. The reason is that only support RTD<CP is too limited from deployment point of view. In Rel-18, we already consider RTD>CP with UE capability. Similar approach can be considered for CSI-RS based L1 measurement.

Proposal 5: it is proposed to consider both RTD<CP and RTD>CP, and UE capability for supporting RTD>CP can be introduced.

- Issue 4-5: Definition of “inside/outside active BWP” Candidate options: - Option 1: (Apple) The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP. - Option 2: (CATT) RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’. - Option 3: (vivo) From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

Physically, the CSI-RS of candidate cell is considered inside the active BWP if all the resources are confined within the BWP. The CSI-RS of candidate cell is considered outside the active BWP if all the resources are not within the BWP. For partially overlap case, whether it is considered as within BWP or outside BWP need more discussion. Some companies propose that a CSI-RS resource is regarded as outside active BWP if at least one RB that is outside any active BWP. We have concern on this proposal. CSI-RS based L1 measurement is for mobility, from deployment point of view, it is normal that CSI-RS resources from neighbour cell are partially overlap UE active BWP.  According to legacy CSI-RS based L1 measurement requirements, the requirements are defined with the assumption of CSI-RS BW of 48 RBs. If we follow the similar assumption for CSI-RS based L1 measurement for neighbour cell, the CSI-RS resource can be considered as within BWP as long as the RBs of the CSI-RS within the active BWP is no less than 48 RBs.

Proposal 6: it is proposed that the CSI-RS of candidate cell is considered inside the active BWP if

- all the CSI-RS resources are confined within the BWP (for fully overlap case), or

- the RBs of the CSI-RS within the active BWP is no less than 48 RBs (for partially overlap case)

- Issue 4-8: CSI-RS resource type Candidate options: - Option 1: (Apple, OPPO, Nokia, vivo) RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision. - Option 2: (CMCC, Ericsson) Semi-persistent CSI-RS resource for LTM candidate cells is supported. - Option 3: (CATT) RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

In RAN1#118bis, the working assumption is that in addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective. And  RAN1 sent LS to RAN3 to ask RAN3 for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) [4]. In November meeting, RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and reply LS was sent to RAN1 [5]. Based on above progress in RAN1 and RAN3, semi-persistent CSI-RS resource for LTM candidate cells is supported.

Observation 1: According to the progress in RAN1 and RAN3, semi-persistent CSI-RS resource for LTM candidate cells is supported.

Since periodic CSI-RS and semi-persistent CSI-RS are supported for candidate cell L1-RSRP measurement in RAN1 and RAN3. From RAN4 perspective, it is necessary to define requirements for both cases. As for aperiodic CSI-RS, there is no conclusion in RAN1. It can be further considered if it is supported in RAN1.

Proposal 7: it is proposed that RAN4 define CSI-RS based L1 measurement requirements for periodic and semi-persistent CSI-RS resource. Aperiodic CSI-RS can be further considered if it is supported in RAN1.

- Conclusion

This contribution provides discussion on CSI-RS based L1 measurement. The proposals are:

Proposal 1: it is proposed that whether step 2 can be skipped or not is based on UE capability reporting.

Proposal 2: for the definition of CSI-RS based intra-frequency L1 measurement, except SCS and CP, centre frequency also need to be considered, i.e. the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement.

Proposal 3: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario, and it is proposed to define requirements for CSI-RS based L1-RSRP measurement outside active BWP.

Proposal 4: it is proposed to define requirements for CSI-RS based L1 measurement with gap.

- For delay requirements definition, for simplicity, equally sharing among SSB measurement and CSI-RS measurement can be considered

Proposal 5: it is proposed to consider both RTD<CP and RTD>CP, and UE capability for supporting RTD>CP can be introduced.

Proposal 6: it is proposed that the CSI-RS of candidate cell is considered inside the active BWP if

- all the CSI-RS resources are confined within the BWP (for fully overlap case), or

- the RBs of the CSI-RS within the active BWP is no less than 48 RBs (for partially overlap case)

Observation 1: According to the progress in RAN1 and RAN3, semi-persistent CSI-RS resource for LTM candidate cells is supported.

Proposal 7: it is proposed that RAN4 define CSI-RS based L1 measurement requirements for periodic and semi-persistent CSI-RS resource. Aperiodic CSI-RS can be further considered if it is supported in RAN1.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R1-2409283, LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

- R3-247911, Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4