# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503760

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.28.2

Source: 	CMCC

Title: 			 Discussion on XR (eXtended Reality) for NR Phase 3

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on XR (eXtended Reality) for NR Phase 3 was approved [1]. According to the WID, one of the objectives having RAN4 impact is to enable transmission/reception in gaps/restrictions that are caused by RRM measurements, the details are duplicated as following.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In last meeting, there was discussion on XR (eXtended Reality) for NR Phase 3, and a WF was agreed [2]. This contribution provides discussion on this topic.

- Discussion

Issue 2-1: Types of measurement gaps <Way forward>: - In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

In previous meetings, it was agreed that for the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI [3]. According to the WID, the objective is to specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements [1]. It can be seen that no specific measurement gap is the target, which means that all the measurement gaps introduced so far need to be considered. Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps, MUSIM gaps, positioning gaps are introduced in RAN4. considering that positioning gaps are introduced for positioning, and MUSIM gaps are introduced for the measurement of another network, whether positioning gaps and MUSIM gaps can be cancelled for XR need further discussion. All other measurement gap types are for measurement for the mobility purpose. Since Type-1 MG is considered, we do not see issues to preclude other measurement gap types.

Proposal 1: for types of measurement gaps, it is proposed that Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps are first priority.

Issue 3-1: Principles for measurement delay calculation <Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension. Issue 3-2: Formulas for measurement delay extension <Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases.

It was agreed to follow NR-U-like approach for designing the formulas for measurement delay extension. Following is the duplication of legacy measurement period for NR intra-frequency measurements with CCA. It can be seen that Lmeas,gaps is used to reflect the number of SMTC occasions not available at the UE due to LBT. For XR, measurement gap occasions indicated to be skipped are not available for measurement. It can be seen that the situation are similar between NR-U and XR. The way to define delay requirements for CCA can be reused to calculate measurement delay extension due to MG skipping for XR.

Table 9.2A.6.2-1: Measurement period for intra-frequency measurements with gaps (FR1)

| Condition | TSSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(200 ms, (5+Lmeas,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x (5+Lmeas,gaps)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | (5+Lmeas,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When DRX is configured, Lmeas,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lmeas,gaps,max = 7 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,gaps,max = 5 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lmeas,gaps,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,gaps,max over the TSSB_measurement_period_intra_CCA period of time, the UE has to restart the measurement procedure. |  |

Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, the measurement delay extension due to measurement skipping is proposed as following:

- Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 3 ms optional capability only apply when UE is only configured with NR measurement. - Option 2: No need to define additional condition for the optional capability of 3 ms

Time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network. In previous meeting, it was agreed that the baseline of time offset is 5 ms, and 3ms is also supported wit UE capability, and FFS if any condition is needed [3]. In our view, there is UE capability to indicate whether 3ms of time offset is supported or not, no need to have any condition.

Proposal 3: for time offset, there is no need to have condition for the optional capability of 3ms.

Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

It was agreed to agreed to define UAI and further discuss the content of UAI. How UE perform measurement is up to UE implementation. Some measurement gap occasion not used for measurement can be used for data reception/transmission. But network has no information about whether measurement is performed in a cetain measurement occasion. From this point of view, UE assistance information related to measurement occasions, e.g. the number of unused measurement gap occasions and/or the location of unused measurement gap occasions within a time period, could help network to perform configuration, e.g. to indicate whether a measurement gap(s) is skipped or not.

Proposal 4: it is proposed that the content of UE assistance information could be the number and/or location of measurement gap occasions not used for measurement.

Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

We see the benifit to introduce semi-static schemes additionally. On one hand, semi-static scheduling mechanisms, i.e. CG and SPS, are used for XR service. RRC configuraion is more suitable for semi-static scheduling mechanisms. On the other hand, it is more efficient to indicate the skipped measurement occasions by RRC configuration, which not only reduces signaling overhead, but also simplifies UE implementation.

Proposal 5: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

- Conclusion

This contribution provides discussion on XR (eXtended Reality) for NR Phase 3. The observations and proposals are:

Proposal 1: for types of measurement gaps, it is proposed that Type 1 MG, Type 2 MG, NCSG, Pre-MG, concurrent gaps are first priority.

Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, the measurement delay extension due to measurement skipping is proposed as following:

- Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

Proposal 3: for time offset, there is no need to have condition for the optional capability of 3ms.

Proposal 4: it is proposed that the content of UE assistance information could be the number and/or location of measurement gap occasions not used for measurement.

Proposal 5: for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

- Reference

- RP-234080, New WID: XR (eXtended Reality) for NR Phase 3

- R4-2502700, WF on RRM requirements for XR_Ph3

- R4-2420103, WF on RRM requirements for XR_Ph3, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia

- R1-2209536, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R1-2405736, LS on UE assistance information

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements