# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503763

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.17.2.1

Source: 	CMCC

Title: 			 Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

There is discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode in last meeting, and a WF was approved [2]. This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor.

- Discussion

- Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

In last meeting, it was agreed that serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) is the triggering condition to activate/deactivate multi-Rx for L3 measurement. If the serving cell’s signal quality is below the threshold, multi-Rx for L3 measurement is activated. If the serving cell’s signal quality is above the threshold, multi-Rx for L3 measurement is deactivated.

One of the open issues is whether to limit the serving cell to FR2 serving cell. It is straightforward that the serving cell can be FR2, since the objective, according to the WID [1], is for FR2-1 SSB based L3 measurement delay reduction. However, it is not harmful to consider FR1 serving cell, which is also not contradict with the WID. The target to apply the optimizing RX BSF is FR2 cell, but the trigger condition can be based on FR1 serving cell. In detail, if the signal quality of FR1 serving cell meet the threshold, the measurement on FR2 target carrier can be performed by activating multiple panels simultaneously.

Proposal 1: for FBS triggering condition of serving cell, the serving cell can be FR1 serving cell or FR2 serving cell.

- Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue.

The intention to consider quit mechanism is from the perspective of power consumption. According to previous discussion, some companies propose that UE is allowed to fall back to normal measurement mode (i.e. deactivated multiple panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. We have different views, HO and SCell related configuration is up to network implementation. If the conditions to trigger UE to activate multiple panel reception is met, it is expected that UE can continue in this mode until reception of network command, e.g. HO command. The reason is that if UE fall back to normal measurement mode after UE has reported qualified measurement results, there may be mobility issue since the reported measurement results are outdated. It is not preferred to allow UE quit multi-Rx for L3 measurement autonomously.

We understand UE’s concern on power consumption. Similar issue exist in Rel-18 multi-RX reception. And UAI “multiRx-PreferenceFR2” is introduced. Similar approach can be considerd for L3 measurement delay reduction by optimizing Rx BSF. UAI similar as “multiRx-PreferenceFR2” can be introduced to inform network there is overheating issue. Then netwok can enable UE to deactivate multi-Rx for L3 measurement by reconfiguring the threashold.

Proposal 2: it is proposed to introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF - Proposals - Option 1a: (ZTE, CATT, Ericsson, CT, CMCC) - Introducing the UE capability of reduced N, the candidates could be 2, 4 or 6. - Option 1b: (QC) - At least, support reduction of beam sweeping factor from 8 to 4 for multi-RX capable UEs in Rel-19. Support of other beam sweeping factors can be FFS. - Option 1c: (MTK) - On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either: - Single value for reduced BSF (e.g., 6 or 4) - Multiple candidate values for BSF (e.g., 6 and 4) - Option 2: (vivo) - RAN4 to consider the following approaches as solutions: - Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc. - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 3: (HW) - The UE capability design shall allow enough flexibility on supporting this feature considering: - Different BC - Number of Carriers/MOs to measured - Intra-frequency/inter-frequency - Option 4: (LGE) - Introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay

Rel-18 multi-Rx chain WI mainly focus on the L1 related measurement. According to the discussion in Rel-18 multi-Rx chain WI, it was agreed to introduce reduced beam sweeping factor for SSB-based L1-RSRP measurement if the UE is capable of multi-Rx operation, which are applied to L1-RSRP/RLM/BFD. The candidate values for reduced beam sweeping factor are: {2,4,6} for FR2-1. In our view, for UE capable of multi-Rx operation, we do not see issues to extend the Rel-18 RX beam sweeping factor reduction to L3 measurement. It is proposed to reduce L3 measurement delay by reducing RX beam sweeping factor, and the Rel-18 RX beam sweeping factor reduction can be used as baseline.

As for the detail about how to reflect the reduction of measurement delay, one solution is that UE report the reduced RX beam sweeping factor directly, the other solution is that UE report M values which is used in the legacy measurement delay requirements.

If we look at the legacy L3 delay requirements, BSF is relected by the value of M. M is related with the number of samples and BSF. However, the number of samples are differeent for different cases, for example, the sample number is 5 for intra-frequency measurement and 8 for inter-frequency measurement. If we go wih the solution of reporting M values, there are will be more candidate valus, and we may also need to differentiate which values are for intra-frequency measurement and which values are for inter-frequency measurement. If we go with solution of reporting BSF, we can simply update the M value in the spec.

Proposal 3: it is proposed to reduce L3 measurement delay by reporting the reduced RX beam sweeping factor, e.g. N= {2,4,6} can be used as baseline.

- Conclusion

This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor. The proposals are:

Proposal 1: for FBS triggering condition of serving cell, the serving cell can be FR1 serving cell or FR2 serving cell.

Proposal 2: it is proposed to introduce UAI similar as “multiRx-PreferenceFR2” to inform network there is power consumption issue.

Proposal 3: it is proposed to reduce L3 measurement delay by reporting the reduced RX beam sweeping factor, e.g. N= {2,4,6} can be used as baseline.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements