# 3GPP TSG-RAN WG4 Meeting #114bis                             R4-2503764

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.17.2.2

Source: 	CMCC

Title: 			 Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

There is discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode in last meeting, and a WF was approved [2]. This contribution provides discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC.

- Discussion

## 2.1 How to reflect 3 searcher’s impact on CSSF

Issue 1-1: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF - Option 1 (Apple, CMCC, HW, Ericsson, vivo): The enhanced CSSF apply to all the 3 aspects and do not define the priority. - Option 2 (Nokia, ZTE, CATT, CTC, vivo): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion. Agreement: - Option 1 is agreed Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19.

In last meeting, it is agreed to perform CSSF enhancement based on 3 searchers. For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing. As for the applied scenario for 3 searchers, it was agreed to support FR1 CA and FR1+FR2 scenario where PCell is FR1. FFS for NR-DC. Furthermore, it was agreed that enhanced CSSF apply to all the 3 aspects including intra-frequency measurement, inter-frequency measurement and inter-RAT measurement.

How to reflect the impact due to 3 searchers on CSSF is discussed as below.

For legacy 2 searcher case, one searcher is dedicated for PCC measurement. For FR1, all the SCCs share the 2nd searcher. For FR2, 50% of the 2nd searcher is for FR2 SCC where neighbour cell measurement is required, and the other 50% of the 2nd searcher is for FR2 SCC where neighbour cell measurement is not required, inter-frequency MO with no measurement gap and E-UTRA inter-RAT MO with no measurement gap.

It was agreed that the 3rd searcher resource sharing is same as the second searcher sharing. With this assumption, for FR1, all the SCCs share the 2nd and 3rd searcher. For FR2, 50% of the 2nd searcher and 50% of the 3rd searcher are for FR2 SCC where neighbour cell measurement is required, and the other 50% of the 2nd searcher and the other 50% of the 3rd searcher are for FR2 SCC where neighbour cell measurement is not required, inter-frequency MO with no measurement gap and E-UTRA inter-RAT MO with no measurement gap. Based on above consideration, it is proposed to have a scaling factor ti reflect the impact due to different number of searchers, for example Nsearcher. Nsearcher is 1 for UE not support 3 searchers, and Nsearcher is 2 for UE support 3 searchers. The details can be found as following table.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | (NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) / Nsearcher | N/A | N/A | N/A | (NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) / Nsearcher | (NSCC_SSB +Y+Z+2x NSCC_CSIRS) / Nsearcher |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) / Nsearcher Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 12: Nsearcher is 1 for UE not support 3 searchers, and Nsearcher is 2 for UE support 3 searchers. |  |  |  |  |  |  |  |

Proposal 1: as for how to reflect 3 searchers on CSSF, it is proposed to take above table.

## 2.2 UE capability

According to the discussion on optimizing CSSF outside gap in CA/DC, there are three candidate solutions. Solution 1 is to measure one serving CC per band. Solution 2 is to change the searcher occupancy ratio of measurement among the measured CCs. Solution 3 is to support 3 searchers for CSSF. For now, it is agreed not to support solution 2 in Rel-19. And solution 1 and solution 2 are supported.

For solution 3: 3 searchers for CSSF, it was agreed to define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). no more UE capability is needed.

For solution 1: measure one serving CC per band, the whole solution is as below. With solution 1, UE only need to measure one serving CC per band. While in legacy, UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band. It can be seen that compared with legacy way, UE measurement behaviour is simplified since the number of CC need to measure is reduced. From this point of view, no UE capability is needed.

Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

Proposal 2: for UE capability on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it is proposed as following:

- For solution 3 (3 searchers for CSSF), define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) , as agreed in last meeting.

- For solution 1 (measure one serving CC per ban), no UE capability is needed since UE measurement is simplified compared with legacy way

- Conclusion

This contribution provides discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC. The proposals are:

Proposal 1: as for how to reflect 3 searchers on CSSF, it is proposed as following table:

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | (NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) / Nsearcher | N/A | N/A | N/A | (NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO) / Nsearcher | (NSCC_SSB +Y+Z+2x NSCC_CSIRS) / Nsearcher |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) / Nsearcher Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) / Nsearcher |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| NOTE 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. NOTE 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. NOTE 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. NOTE 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. NOTE 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. NOTE 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured NOTE 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. NOTE 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. NOTE 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. NOTE 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. NOTE 12: Nsearcher is 1 for UE not support 3 searchers, and Nsearcher is 2 for UE not support 3 searchers. |  |  |  |  |  |  |  |

Proposal 2: for UE capability on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it is proposed as following:

- For solution 3 (3 searchers for CSSF), define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC), as agreed in last meeting.

- For solution 1 (measure one serving CC per ban), no UE capability is needed since UE measurement is simplified compared with legacy way

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements