# 3GPP TSG-RAN WG4 Meeting #114bis                            R4-2503776

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.21.2.2

Source: 	CMCC

Title: 			 Discussion on RAN4 impacts for measurement event prediction

Document for:	Discussion

- Introduction

In RAN #105 meeting, the revised SID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [1]. The objectives for RAN4 are duplicated as following.

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In last meeting, there was discussion on AI/ML for mobility, and a WF was agreed [2]. It was agreed that RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting. This meeting is the first meeting that RAN4 discuss RAN4 impacts for measurement event prediction. This contribution provides initial discussion on the topic.

- Discussion

According to the RAN2 agreements, for measurement event prediction, event A3 is taken as baseline. Both indirect measurement event prediction and direct measurement event prediction are supported.

Agreements in RAN2#125bis: - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed. - Start with A3 as a baseline. Agreements in RAN2#127bis Agreements - Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement). FFS what is KPI. - Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing. Focus on temporal case B. - Companies will prioritize simulations on indirect method. Companies can bring simulations on direct method and should report what method is being used. - Measurement event prediction results are expected in RAN2#129 Agreements on simulation assumptions Companies will focus on sub-case 2 for measurement event prediction. Companies can simulate other sub-cases if they wish and report what they are using. Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference. Agreements in RAN2#128 Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional. 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’ Agreements in RAN2#129 Agreements on Measurement event case A - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. Agreements on Measurement event case B - Measurement event case B can have very good F1 score depending on filtering approach/PW - F1 score is good for the PW window for case B (i.e. low PW) - F1 score decreases with an increasing MRRT value - We will focus on finding cases that bring benefits to the system rather than trying to compare - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism. Other agreements - Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required. Agreements on direct measurement event prediction - F1 score for direct measurement is very good based on the current simulation results.

RAN2 agreed the definition of indirect measurement event prediction for intra-frequency temporal domain case A, intra-frequency temporal domain case B, frequency domain and spatial domain. Taking temporal domain case A as an example, in indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells.  The final output is the expected occurrence time of a certain measurement event.

It can be seen that for indirect measurement event prediction,the intermediate output is same as that for RRM measurement prediction. The discussion and conclusion can be reused.

- Proposal 1: for the intermediate output of indirect measurement event prediction, it is proposed to reuse the conclusion on RRM measurement prediction.

For direct measurement event prediction, the model output is the probability of event occurrence within a time window. And for indirect measurement event prediction, final output is the expected occurrence time of a certain measurement event. In legacy, there is no RAN4 requirements on the probability of event occurrence or expected occurrence time of a event. It is necessary to discuss whether and how to define related RAN4 requirements.

Proposal 2: for the final output of measurement event prediction (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction), it is proposed to discuss whether and how to define RRM requirements.

- Conclusion

This contribution provides discussion on RAN4 impacts for measurement event prediction. The proposals are:

- Proposal 1: for the intermediate output of indirect measurement event prediction, it is proposed to reuse the conclusion on RRM measurement prediction.

Proposal 2: for the final output of measurement event prediction (e.g. the probability of event occurrence for direct prediction or expected occurrence time of a event for indirect prediction), it is proposed to discuss whether and how to define RRM requirements.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR

- R4-2502594, WF on AI/ML Mobility, Nokia

- R4-2420095, WF on AI/ML Mobility, Nokia

- R4-2416855, WF on AI/ML Mobility, Nokia

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- Report of 3GPP TSG RAN WG2 meeting #126