3GPP TSG-RAN WG4 Meeting #114bis	R4-2503783

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	RF Rx requirements for PC2 RedCap in FDD bands

Agenda Item:	7.6.1

Document for:	Discussion

- Introduction

This contribution discusses the REFSENS requirements for PC2 RedCap with 1Rx in FDD bands.

- Discussion

The latest WF [1] was approved with reaching the following agreements regarding PC2 RedCap with 1Rx in FDD bands.

Issue 1-2-3: REFSENS for 1Rx in FD-FDD Agreement: - Further discuss the following formats for further evaluations - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR - Evaluate the values of ΔR band by band - Note: Other options are not precluded Way forward: It is encouraged for companies to provide analysis for the above options and give the preference in next meeting.

Thus, based on the above mentioned agreements and way forward, we think Option 2 is more straightforward for a PC2 RedCap UE equipped with 1 Rx antenna ports from specification definition point. TheΔR1R method that applies to PC3 RedCap UE can be reused. The REFSENS requirement for PC2 RedCap UE with 1Rx is modified by the amount of ΔR1R based on PC2 RedCap UE with 2Rx. In total, the specific value could be derived by

REFSENSPC2 RedCap 1Rx=REFSENSPC2 2Rx/1Tx + ΔR1R

By option 2, the specification modification is quite simple, a spec change example could be like

For a power class 2 RedCap UE equipped with 1 Rx antenna ports operating in FD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1c shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R Operating band; Channel bandwidth (MHz); ΔR1R (dB). TDD band; 5, 10, 15, 20; 2.5. FDD band; 5; 2.5. FDD band; 10, 15, 20; 3.0.

Proposal: For a PC2 RedCap UE with 1Rx, the REFSENS requirement is specified by the formula:

REFSENSPC2 RedCap 1Rx=REFSENSPC2 2Rx/1Tx + ΔR1R

- Conclusions

In this contribution, we discuss the REFSENS for PC2 RedCap with 1Rx in FDD bands and have the following proposal:

Proposal: For a PC2 RedCap UE with 1Rx, the REFSENS requirement is specified by the formula:

REFSENSPC2 RedCap 1Rx=REFSENSPC2 2Rx/1Tx + ΔR1R

- References

R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom, Skyworks, ZTE, Qualcomm, CATT, Murata, Ericsson, Huawei, Apple, Nokia, RAN4#114, Feb., 2025.