3GPP TSG-RAN WG4 Meeting #114bis	R4-2503787

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.21.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM measurement impacts of AIML for mobility

Document for:	Discussion

- Introduction

In the last meeting, AI/ML for mobility was discussed and discussions and agreements were captured in WF [1]. According to this WF, this paper proposes possible RRM measurement impacts.

- Discussion

During previous meeting, the definition of relative RSRP prediction accuracy was discussed and following agreement was obtained.

Issue 2-1-1a: Relative RSRP Prediction Accuracy Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

In addition, ground truth definition for RSRP accuracy was discussed and agreed as follows.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2. Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

According to these agreements, the ground truth for relative RSRP prediction accuracy should be same as the ground truth definition for RSRP accuracy and it should be reported value for FR2.

Proposal 1: The ground truth for relative RSRP prediction accuracy should be same as the ground truth definition for RSRP accuracy.

Proposal 2: For FR2, the ground truth for relative RSRP prediction accuracy should be the reported L3 RSRP measurement under sufficient high SNR.

The purpose to introduce predicted relative RSRP accuracy is to guarantee the accuracy of relative RSRP based on the prediction. Thus it should be based on the current relative RSRP definition and then beam 1 and beam 2 can be from different cells.

Proposal 3: As same as current relative RSRP definition, beam 1 and beam 2 can be from both same cell and different cells.

In addition, considering the mobility purpose, the RSRP difference between source cell and target cell shall be calculated within certain accuracy and target cell RSRP can be both predicted and measured. Considering this situation, the reported RSRP of cell2/beam2 in predicted relative RSRP accuracy also should be both predicted and measured value. Combination of both predicted and measured samples is not needed because the performance of this case should be the middle ground of only measured value case and only predicted value case.

Proposal 4: The reported RSRP of cell2/beam2 in predicted relative RSRP accuracy should be both predicted and measured value considering mobility purpose.

Proposal 5: Combination of both predicted and measured samples for reported RSRP of cell2/beam2 is not needed because the performance of this case should be the middle ground of only measured value case and only predicted value case.

- Conclusion

In this contribution, we proposed our views on RAN4 requirement impacts of AI/ML for mobility.

Proposal 1: The ground truth for relative RSRP prediction accuracy should be same as the ground truth definition for RSRP accuracy.

Proposal 2: For FR2, the ground truth for relative RSRP prediction accuracy should be the reported L3 RSRP measurement under sufficient high SNR.

Proposal 3: As same as current relative RSRP definition, beam 1 and beam 2 can be from both same cell and different cells.

Proposal 4: The reported RSRP of cell2/beam2 in predicted relative RSRP accuracy should be both predicted and measured value considering mobility purpose.

Proposal 5: Combination of both predicted and measured samples for reported RSRP of cell2/beam2 is not needed because the performance of this case should be the middle ground of only measured value case and only predicted value case.

References

[1] 3GPP, R4-2502594, “WF on AI/ML Mobility”, Nokia