3GPP TSG-RAN WG4 Meeting #114bis	R4-2503788

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.24.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM requirements for A-IoT

Document for:	Discussion

- Introduction

According to the agreed TU allocation for A-IoT WI, RRM core requirements are discussed from this meeting. Based on the objective in the WID [1], the preliminary considerations of RRM requirements are proposed in this document.

- Discussion

According to the latest WID, the work scope of RRM is described as follows.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. […] - RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

Observation 1: The RRM scope for this WI is only for device 1

The device 1 is defined in TR 38.769 copied in below.

## ~1 µW devices (Device 1) The architecture of such a device is summarised in Figure 5.1-1, with the blocks described as follows. - Antenna could be either shared or separate for RF energy harvester and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester and receiver related blocks). - RF energy harvester can include rectifier performing RF signal (AC) to DC conversion. - Energy storage (e.g., capacitor) stores harvested energy from RF energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and supplying power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - RF Envelope Detector converts RF signal to baseband. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator. - Depending on implementation, it may not exist. Presence of BB LPF is assumed for the study. - Comparator determines high/low of input signal. - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with transmitted signal from BB logic. Figure 5.1-1: Architecture of device 1

In addition, the targeted topology is Topology 1 only, thus direct and bidirectional communication between A-IoT devices and BS is considered.

Observation 2: Only Topology 1, i.e. direct and bidirectional communication between A-IoT devices and BS, is considered for this WI.

Moreover, A-IoT device is assumed to support only A-IoT MAC/PHY layer and not required to have functionalities of RRM L3 measurement reporting and Mobility.

Observation 3: A-IoT device is assumed to support only A-IoT MAC/PHY layer and not required to have functionalities of RRM L3 measurement reporting and Mobility.

Based on these observations, the RRM requirements defined for this WI are at least timing requirements and delay requirements.

Proposal 1: The necessary RRM requirements for this WI are at least timing requirements and delay requirements.

For timing requirements, the device initial transmission timing error and gradual timing adjustment requirements should be specified under the assumption of no TA command and large initial sampling frequency offset. For delay requirements, since timing related parameters, i.e. TR2D_min, TR2D_max, TD2R_min, and TD2R_max are already defined, corresponding delay requirements should be considered.

Proposal 2: For timing requirements, the device initial transmission timing error and gradual timing adjustment requirements should be specified under the assumption of no TA command and large initial sampling frequency offset.

Proposal 3: Delay requirements corresponding to TR2D_min, TR2D_max, TD2R_min, and TD2R_max should be considered.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: The RRM scope for this WI is only for device 1

Observation 2: Only Topology 1, i.e. direct and bidirectional communication between A-IoT devices and BS, is considered for this WI.

Observation 3: A-IoT device is assumed to support only A-IoT MAC/PHY layer and not required to have functionalities of RRM L3 measurement reporting and Mobility.

Proposal 1: The necessary RRM requirements for this WI are at least timing requirements and delay requirements.

Proposal 2: For timing requirements, the device initial transmission timing error and gradual timing adjustment requirements should be specified under the assumption of no TA command and large initial sampling frequency offset.

Proposal 3: RAN4 waits for RAN1 decisions to determine delay requirements.

References

[1] 3GPP, RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.