# 3GPP TSG-RAN WG4 Meeting #114                             R4-2503790

# Wuhan, China, 7th – 11st April, 2025

Source:  ZTE Corporation, Sanechips

Title:   Discussion on the RRM requirements aspects of AI/ML positioning

Agenda Item:	7.19.4

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some views on the interoperability and testability for AI positioning from the general test framework perspective.

- Discussion

Based on the progresses in the previous meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - AI positioning - UE Direct positioning - UE Assisted positioning

For AI/ML based positioning, direct positioning and assisted positioning are considered. For direct AI/ML positioning, the output is the estimated/predicted location, on the contrary, for assisted positioning the output of the AI model is the input of the non-AI positioning method, and the estimated location will go through two steps as the figure shown:

Figure 1 AI/ML direct positioning

Figure 2 AI/ML assisted positioning

From RAN4 perspective, we need to evaluate the performance metrics which are agreed in RAN1 and in TR38.843 and the following aspects need to be studied:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In TR 38.843, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study：

- Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

And in previous meeting, RAN4 has reached the related agreements for AI positioning as below:

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS

First of all, we can see the case 1, case 2a and case 2b as below:

| (a) Case 1 | (b) Case 2a |
| --- | --- |
| (c) Case 2b |  |

Figure 3 The illustration of Case1, Case2a and Case2b

Case 1 and case 2b belong to direct positioning, that means the AI/ML model inference output is predicted/estimated UE location. From my perspective, there is no need for RAN4 to consider model inference for case 2b since the AI/ML model is deployed at LMF side which the output UE location on the LMF side depends on the network implementation, and also in the last meeting, RAN4 would not define any positioning accuracy requirements because positioning is LMF based.

Observation 1: There is no need for RAN4 to consider model inference for case 2b since the AI/ML model is deployed at LMF side which the output UE location on the LMF side depends on the network implementation.

Observation 2: RAN4 would not define any positioning accuracy requirements because positioning is LMF based.

Proposal 1: RAN4 shall not define the positioning accuracy requirements for case 2b, it is up to network implementation.

In the previous meeting, RAN4 kicked off the discussion on case 1 and reached the agreement as below:

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme (if defined) is clear. If reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In case 1 the AI/ML model is at UE side, that is, model inference is at UE side which is up to UE implementation. For direct UE-based positioning, the model output provides the predicted/estimated UE location. Case 1 is similar to the legacy UE-based positioning and no requirements are defined for such scenario. In previous meeting, we discussed whether to define the related accuracy requirements for case 1 and the final agreements shows if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. That is, the main discussion is what is the reporting scheme for case 1. From my perspective, the reporting scheme shall be discussed in RAN1 and once RAN1 has the agreements on this RAN4 can continue to study the accuracy requirements for case 1.

However, we can review the legacy reporting scheme firstly. In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. Some companies deem that there is no need to define accuracy requirements for case 1 since the UE triggers the location request and it is no need to report the location to NW which is up to UE implementation. If the location request is triggered by the network, the UE shall report its location. To my understanding, the above agreement contains the implicit reporting scheme which is the NW triggers the location request, otherwise no reporting scheme is needed.

Observation 3: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed.

Proposal 2: RAN4 shall wait for RAN1 agreements on defining reporting scheme.

For case 2a, it belongs to the assisted-positioning and the AI/ML model inference output is intermediate feature, e.g. LOS/NLOS indicator, ToA, RSTD, RSRP etc. For case 2a, the AI/ML model is at UE side, but for position calculation, it is all on the LMF side. In RAN4, the requirements such as RSTD, RSRP, and RSRPP have already been defined in legacy, these accuracy requirements and reporting mappings can be considered for these intermediate features, the related description such as RSTD can be seen in TS38.133 as below:

The requirements in clause 9.9.2 shall apply provided the UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report DL RSTD measurements defined in TS 38.215 [4].

The above description is the UE-assisted positioning. In legacy, the LMF sends the location request and UE perform the PRS measurement. Subsequently, these intermediate features are reported to the LMF to calculate the actual UE position. For AI/ML based positioning, the main difference compared to the legacy is that the AI model resides within the LMF. In the last meeting, RAN4 would not define any positioning accuracy requirements since positioning is LMF based. Apart from the positioning accuracy requirements, the reported metrics can be studied for case 2a.

Observation 4: For AI/ML based positioning, the main difference compared to the legacy is that the AI model resides within the LMF.

Observation 5: RAN4 would not define any positioning accuracy requirements since positioning is LMF based. Apart from the positioning accuracy requirements, the reported metrics can be studied for case 2a.

Proposal 3: The intermediate features such as RSTD, RSRP, UE Rx-Tx time diference etc. or some enhancements shall be considered.

The delay requirements shall be considered. When LMF sends the request to UE, UE receives the request and starts to perform PRS measurement and the related measurements result will be the model input for model inference, then the model output will be reported to the LMF. From request to reporting time, there are two periods of time which are PRS measurement and model inference. We shall define the delay requirements during that periods, otherwise LME takes a long time to receive an inference outcome or the intermediate features which has the impact on the performance accuracy since the accuracy of model output will also decrease over time.

Figure 4 Two periods of time

Observation 6: From request to reporting time, there are two periods of time which are PRS measurement and model inference. If there is no limitation on these two periods, LMF would take a long time to receive an inference outcome or the intermediate features. It has the impact on the performance accuracy since the accuracy of model output will also decrease over time.

Proposal 4: RAN4 shall define the delay requirements when considering PRS measurement and AI model processing time.

For requirements that do not exist in Rel-16/17 NR positioning measurements, such as ToA LOS/NLOS. For LOS/NLOS indicator, RAN1 just agreed at least LOS/NLOS indicator is supported for reporting in #116 meeting. Thus, the LOS/NLOS could be the intermediate feature to be reported and the details can be seen:

Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

However, RAN4 has not defined the related requirements for this measurement in legacy. Therefore, RAN4 shall consider whether and how to define the requirements for LOS/NLOS. For ToA, there is no reporting scheme in legacy only time information related to RSTD and UE Rx-Tx time difference are reported. RAN1 has not evaluated its feasibility and RAN4 has not defined related requirements. Thus, ToA shall be deprioritized.

Observation 7: RAN1 just agreed at least LOS/NLOS indicator is supported for reporting.

Proposal 5: Although LOS/NLOS could be the intermediate feature to be reported, RAN4 shall consider whether and how to define the requirements for LOS/NLOS.

Proposal 6: RAN4 shall deprioritize the study of ToA until RAN1 supports UE report ToA as model output for case 2a.

Case 1 is the direct AI/ML positioning, UE-based positioning with UE-side model. UE measures the downlink PRS and the related measurement results can be the inference input. Then, the inference will be directly performed at UE side, the inference output is the AI/ML predicted UE location. After inference, the UE will report the model output to LMF. Thus, the test metric is the positioning accuracy. For test equipment, one main challenge is the accuracy of UE-based direct positioning. In several past meetings, RAN4 did not discuss the ground truth for AI/ML positioning. If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified. However, how to study or define the positioning ground truth and how to test case 1 are two key points.

Observation 8: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified.

In previous meetings, we proposed several times for ground truth on AI/ML positioning which is PRU (Positioning reference unit). If the UE is the PRU, then the LMF/gNB has already known its location. To my understanding, the AI/ML predicted location can be compared to PRU and then the positioning accuracy error will be known. The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated. Thus, several companies proposed the test map granularity and test map can be defined as below [3]:

Figure 5 Grid map providing a ground-truth information for inference evaluation

While UE moves from one location point to another location, the inference result with AI/ML functionality at each location is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Also, we deem that if UEs have the GNSS capability, they also can be the reference UEs. Based on TR 38.171, the GNSS error are as below:

| Scenario Type | Weak satellite signal conditions | Ideal conditions | Satellites with rather different signal levels | Multi-path Scenario | Moving scenario with periodic update |
| --- | --- | --- | --- | --- | --- |
| Position Error | 100 m | 15 m | 100 m | 100 m | 50 m |

However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error. This kind of ground truth can be discussed.

Observation 9: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated.

Observation 10: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error.

Proposal 7: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered.

For Case 2a, RAN1 deems that LOS/NLOS indicator can be the input for model inference at LMF side. However, how to verify the indicator is one concern since RAN4 did not discuss the propagation conditions before. Whether the single path or multi channels shall be considered and which kind of channels shall be used shall be studied too. This intermediate KPI verification will ensure that the minimum performance requirements are met by the AI/ML model/functionality. To my understanding, a single path for LOS is enough based on TR 38.901. How many multiple channels will be used for NLOS shall be discussed based on which kind of channels will be used.

Observation 11: RAN1 deems that LOS/NLOS indicator can be the input for model inference at LMF side. However, how to verify the indicator is one concern since RAN4 did not discuss the propagation conditions before.

Proposal 8: Whether the single path or multi channels shall be considered and which kind of channels shall be used shall be studied.

Proposal 9: A single path for LOS is enough. How many multiple channels will be used for NLOS shall be discussed based on which kind of channels will be used.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML positioning:

Observation 1: There is no need for RAN4 to consider model inference for case 2b since the AI/ML model is deployed at LMF side which the output UE location on the LMF side depends on the network implementation.

Observation 2: RAN4 would not define any positioning accuracy requirements because positioning is LMF based.

Proposal 1: RAN4 shall not define the positioning accuracy requirements for case 2b, it is up to network implementation.

Observation 3: In legacy, there are two kinds of location request: UE-triggered location request and NW-triggered location request. The latest agreement contains the implicit location request which is the NW triggers the location request, otherwise no reporting scheme is needed.

Proposal 2: RAN4 shall wait for RAN1 agreements on defining reporting scheme.

Observation 4: For AI/ML based positioning, the main difference compared to the legacy is that the AI model resides within the LMF.

Observation 5: RAN4 would not define any positioning accuracy requirements since positioning is LMF based. Apart from the positioning accuracy requirements, the reported metrics can be studied for case 2a.

Proposal 3: The intermediate features such as RSTD, RSRP, UE Rx-Tx time diference etc. or some enhancements shall be considered.

Observation 6: From request to reporting time, there are two periods of time which are PRS measurement and model inference. If there is no limitation on these two periods, LMF would take a long time to receive an inference outcome or the intermediate features. It has the impact on the performance accuracy since the accuracy of model output will also decrease over time.

Proposal 4: RAN4 shall define the delay requirements when considering PRS measurement and AI model processing time.

Observation 7: RAN1 just agreed at least LOS/NLOS indicator is supported for reporting.

Proposal 5: Although LOS/NLOS could be the intermediate feature to be reported, RAN4 shall consider whether and how to define the requirements for LOS/NLOS.

Proposal 6: RAN4 shall deprioritize the study of ToA until RAN1 supports UE report ToA as model output for case 2a.

Observation 8: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified.

Observation 9: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated.

Observation 10: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error.

Proposal 7: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered.

Observation 11: RAN1 deems that LOS/NLOS indicator can be the input for model inference at LMF side. However, how to verify the indicator is one concern since RAN4 did not discuss the propagation conditions before.

Proposal 8: Whether the single path or multi channels shall be considered and which kind of channels shall be used shall be studied.

Proposal 9: A single path for LOS is enough. How many multiple channels will be used for NLOS shall be discussed based on which kind of channels will be used.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- [2] R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated