# 3GPP TSG-RAN WG4 Meeting #114bis                              R4-2503797

# Wuhan, China, 7th – 11st  April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for less than 5MHz in NTN enhancement

Agenda item:	7.10.3.5

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. We can see the main task on RRM performance part for less than 5MHz NTN as follows:

Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this paper we will provide our views on RRM performance requirements on less than 5MHz NTN.

Discussion

For measurement accuracy requirements, we mainly discuss the L3 and L1 accuracy requirements applies for less than 5MHz in NTN scenario. We find that one agreement was reached in TN less than 5MHz WI as below:

- Existing SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE.

The legacy SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE, thus, the same logic in NTN scenario. We conclude that the existing SSB based L3 and L1 accuracy requirements in legacy NTN could be applied for less than 5MHz BW UE in NTN scenario.

In core part discussion, we do not consider the CSI-based measurements. Consequently, only SSB based accuracy requirements shall be considered.

Observation 1: In TN scenario, existing SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE.

Observation: In core part discussion, there is no discussion on CSI-based measurements.

Proposal 1: Only SSB based accuracy requirements shall be considered.

Proposal 2: The existing SSB based L3 and L1 accuracy requirements in legacy NTN could be applied for less than 5MHz BW UE in NTN scenario.

In performance part, RAN4 shall consider defining test cases. Based on the discussion in core part, we list RRM test cases for the following requirements as below:

- Handover;

- Conditional handover;

- Satellite switching with re-synchronization;

- BFD and CBD;

- Link recovery procedure;

- Intra-frequency measurement;

- Inter-frequency measurement.

Proposal 3: RAN4 shall define RRM test cases for the following requirements:

- Handover;

- Conditional handover;

- Satellite switching with re-synchronization;

- BFD and CBD;

- Link recovery procedure;

- Intra-frequency measurement;

- Inter-frequency measurement.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR less than 5M NTN enhancement.

Observation 1: In TN scenario, existing SSB based L3 and L1 accuracy requirements applies for less than 5MHz BW UE.

Observation: In core part discussion, there is no discussion on CSI-based measurements.

Proposal 1: Only SSB based accuracy requirements shall be considered.

Proposal 2: The existing SSB based L3 and L1 accuracy requirements in legacy NTN could be applied for less than 5MHz BW UE in NTN scenario.

Proposal 3: RAN4 shall define RRM test cases for the following requirements:

- Handover;

- Conditional handover;

- Satellite switching with re-synchronization;

- BFD and CBD;

- Link recovery procedure;

- Intra-frequency measurement;

- Inter-frequency measurement.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133