# 3GPP TSG-RAN WG4 Meeting #114bis                                R4-2503798

# Wuhan, China, 7th – 11st April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for downlink coverage enhancement

Agenda item:	7.29.4.2

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved. RAN4 shall define the RRM requirements for downlink coverage enhancement based on the latest revised WID as below:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

Discussion

In past meetings, RAN4 has mainly discussed the RRM requirements on SSB periodicity larger than 20ms. There are still some issues to be discussed in this meeting.

Cell Re-selection

In the last meeting, the cell re-selection has not been discussed and the way forward could be seen as below:

Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 : FFS - Option 2 : No update is needed

For cell re-selection requirements, the normal measurement requirements and enhanced measurements could be seen as follows:

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh DRX cycle length [s]; Tdetect,NR_Intra_enh [s] (number of DRX cycles); Tmeasure,NR_Intra_enh [s] (number of DRX cycles); Tevaluate,NR_Intra_enh [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2)Note 1; 0.32 x M3 (1 x M3) Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5.

Compared to the normal measurement requirements, the number of DRX cycles are reduced to 1, and the scaling factor M2 is changed to M3. Some companies deem that the existing Tmeasurement,NR_Intra_enh shall be revised since the periodicity of SSB is extended to 160ms, others think the legacy enhanced measurement requirements shall be maintained. From our perspective, the requirements shall follow the principle as below (intra-frequency measurement as an example):

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

When DRX equals 320ms, the extended SSB periodicity is 160ms, only 2 (320ms/160ms=2) SSB occasions occur in one DRX cycle and Tmeasure,NR_Intra_enh/2 equals 160ms. If following the above two principles, it means the UE has to measure total two SSBs and decode them within a DRX cycle, which is a little bit stringent.

However, we deem that the periodicity of SMTC is one key point on this issue. If the periodicity of SMTC equals to extended SSB periodicity (160ms) , we deem that the existing requirements shall be maintained. Since the SMTC periodicity is 160ms, M3 is 2.5 based on the above highlighted NOTE 1. When DRX cycles equal to 320ms, the corresponding latency has been extended by multiple of M3, the extended number of DRX cycles equal to 2.5, the following figure could be seen:

Figure 1 DRX cycle=320ms, SSB periodicity=160ms, SMTC periodicity equals to SSB period

We conclude that 3SSBs are measured and the interval among 3SSBs is larger than 160ms, which satisfy for the above two principles in this scenario.

Observation 1: The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

Proposal 1: The existing enhanced cell re-selection measurements shall be reused if SMTC periodicity is equal to SSB periodicity or larger than 40ms.

Paging

In current spec, paging interruption has been defined, the interruption time shall not exceed TSI-NR+K*Ttarget_cell_SMTC_period ms. However, RAN4 is discussing whether any update is needed to paging interruption requirements for NGSO scenarios when the SSB period is equal to 160ms. The following way forward could be seen:

- Option 1 (Huawei, vivo): no update is needed - Option 2 (Nokia): reduce the multiplying factor K in 4.2C.2.5

To our understanding, this issue shall consider a trade off between the minimum interruption and measurement accuracy. When the SSB period is extended to 160ms, that is, SMTC periodicity will be extended to 160ms or larger than 160ms. In current spec, K is 5 if target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO. If we use the legacy value as 5, the K*Ttarget_cell_SMTC_period ms will be 900ms, which is nine times that of SSB period=20ms. we deem that this interruption is too large and the key paging message may be missed. However, if we only reduce the K due to the large interruption, the measurement accuracy will also be impacted. Consequently, we think a trade off between interruption and measurement accuracy shall be considered.

Proposal 2: K value shall be reduced if target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO.

- Note: Trade off between interruption and measurement accuracy shall be considered.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN downlink coverage enhancement.

Observation 1: The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

Proposal 1: The existing enhanced cell re-selection measurements shall be reused if SMTC periodicity is equal to SSB periodicity or larger than 40ms.

Proposal 2: K value shall be reduced if target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO.

- Note: Trade off between interruption and measurement accuracy shall be considered.

- 4References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331