3GPP TSG-RAN WG4 Meeting #114bis		R4-2503811

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.14.3.1

Source:	Apple

Title:	Views on Rel-19 TRP TRS Performance Requirements

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Perf

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The specific objectives for the TRP TRS Performance part as of RAN4#114 have been indicated in [3]

In addition, specific to Size2 UE requirements definition, the following open item was captured -

This contribution provides our views related to the above open item and based on the WF agreed at RAN4#114 in [7].

# 2	Discussion

#### 2.1  Size 2 UE Requirements: Limited Performance Campaign

The WF in [7] sought additional feedback on the possible approaches to creating requirements for Size2 UEs (PDA hand grip)

For several meetings, there has been discussions around whether to use an offset-based approach or pursue a limited or full performance campaign. While our preference was for the option to pursue a limited performance campaign, this was with the caveat that inputs from companies (especially volunteer labs) would be needed on availability of sufficient Size 2 devices for such an effort.

Observation 1:	The suggestion on a limited performance campaign was to move the topic forward but under the caveat that inputs are required from companies (especially volunteer labs) on availability of sufficient Size2 devices for such an effort.

Some parameters of such a limited performance campaign were also suggested: including but not limited to using ~15 devices; device year of production being 2022-2025 and pick one band to test (say n78). To provide time for volunteer labs to procure Size2 UEs a reasonable timeframe was proposed (keeping in mind the workplan for Rel-19 OTA WI allows measurement campaign to proceed and conclude RAN4#118 – February 2026).

However, even after several discussions no clear way forward has been possible on such a performance campaign. Therefore, we proceed to try to determine some information, first from publicly available sources on the availability of Size 2 devices and then propose a way forward.

A reasonable online resource is provided as the website in [8]. By entering the parameters (manufacturing date - 2022-2025; device width >56 mm and <=72mm and supporting at least bands n41 and n78 (which was the first set of bands on which Size 1 requirements were defined) the website's search feature returned 44 smartphones that met the above criteria.

Additional search online reveals that at least 3 devices (maybe more but this a best-effort search) from the 2022 era are not available for purchase due to any variety of reasons which bring the maximum possible pool down to 41 Size 2 devices available for a performance campaign.

If we repeat a similar analysis for the Size 1 devices available for Rel-17 campaign on the same website, there were 869 devices available! Out of this around 75 devices supporting the Rel-17 bands were available to the volunteer labs and used for the actual campaign (~10% of overall available).

In summary,

- As per information received in the early stages of Rel-17 TRP TRS discussions, it can re-iterated that Size2 devices constitute a very small percentage of the smartphones currently in circulation in the ecosystem

- The bandwidth available to perform campaign for Size2 devices is limited and the timeline is even further reduced due to

- Looking at the realistic possibility of the number of Size2 devices that might be available for a full campaign; even a most optimistic estimate (50-60% of total pool) gives us around ~20 devices which is hardly statistically sufficient to perform a CCDF based evaluation.

Observation 2: 		Size 2 devices constitute a very small percentage of smartphones (~5%) in the ecosystem; there is limited bandwidth and time available for any meaningful performance campaign; the size of devices available to volunteer labs for a 3GPP performance campaign may also be very limited.

After the above analysis, we proceeded to perform some internal analysis with the objective to determine what is the actual delta radiated transmitter performance between equivalent Wide Grip (Size1) and PDA Grip (Size2) models from the same manufacturer. In the interest of time the analysis was limited to n1, n28, n41 and n78 to cover low and high bands for which Size1 NR FR1 TRP TRS requirements have already been defined.

Observation 3: 	Based on our internal evaluation we see a minimal average (~0.1 dB) delta in TRP/TRS performance between Size1 and Size2 devices across the bands in question.

Proposal 1: 	Agree to 0 dB delta from Size1 requirements to define Size2 UE requirements. i.e same TRP/TRS requirements for n1 (PC3), n28 (PC3), n41 (PC2/PC3) and n78 (PC2/PC3) for Size 2 DUTs (Talk and Browsing Mode) as already defined for Size 1 DUTs.

# 3	Conclusions

This contribution provides our views on the topic of the Rel-19 TRP TRS performance work scope.  The following observations and proposals are made:

Observation 1:	The suggestion on a limited performance campaign was to move the topic forward but under the caveat that inputs are required from companies (especially volunteer labs) on availability of sufficient Size2 devices for such an effort.

Observation 2: 	 Size 2 devices constitute a very small percentage of smartphones (~5%) in the ecosystem; there is limited bandwidth and time available for any meaningful performance campaign; the size of devices available to volunteer labs for a 3GPP performance campaign may also be very limited.

Observation 3: 	Based on our internal evaluation we see a minimal average (~0.1 dB) delta in TRP/TRS performance between Size1 and Size2 devices across the bands in question.

Proposal 1: 	Agree to 0 dB delta from Size1 requirements to define Size2 UE requirements. i.e same TRP/TRS requirements for n1 (PC3), n28 (PC3), n41 (PC2/PC3) and n78 (PC2/PC3) for Size 2 DUTs (Talk and Browsing Mode) as already defined for Size 1 DUTs.

# 4	References

RP-240841, “New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3,” Qualcomm, 3GPP RAN #103, March 2024

RP-2406086, “Way Forward for [110bis][338] TRP_TRS_MIMO_OTA”, Vivo, 3GPP RAN #110-bis, April 2024

R4-2406084, “Workplan of Rel-19 OTA WI”, Vivo; 3GPP RAN #112-bis, October 2024

R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3”, Vivo, CAICT; 3GPP RAN #110-bis, April 2024

R4-2413537, “Reply LS on 3GPP NR TRP TRS OTA requirements”, Vivo, Apple, 3GPP RAN #112, August 2024

R4-2419808, “WF for [113][325] TRP_TRS_Ph3”, Vivo, 3GPP RAN #113, November 2024

R5-2502370, “WF for [114][325] TRP_TRS_Ph3”, Vivo, 3GPP RAN #113, November 2025

www.gsmarena.com