3GPP TSG-RAN WG4 Meeting # 114bis 	R4-2503814

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on RF requirement for AIoT BS

Agenda Item:	7.24.2.1

Document for:	Approval

- Introduction

In this contribution, we share our analysis and views on the pending requirements for AIoT BS.

- Discussion

  - Tx Requirements

- Transmitter OFF power and transient period

In R19, the spectrum usage for R2D and D2R is clearly stated in WID:

FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

Obviously, the R2D and D2R are in the different spectrum of FDD band, and the interference from BS transmitter to AIoT device is ignorable.

Observation 1: From interference perspective, the transmitter OFF power is not needed for the spectrum usage in R19 since the D2R and R2D are in the different spectrums.

However, the since the R2D and D2R naturally works like TDD mode, and the BS may only support HD-FDD operation, then the transient period need to be defined, which may have impact on the R2D and D2R timeline.

Observation 2: Considering the HD-FDD operation, the transient period for R2D may be needed and the OFF power need to be specified accordingly.

The transmitter OFF power and transient period is already specified for TDD BS, which can be used as starting point for AIoT BS.

Proposal 1: Reuse same OFF power and transient period as NR TDD BS for AIoT BS as starting point.

- Modulation quality

In the last meeting, the following requirements from RFID need to be further discussed as modulation quality:

- Modulation Depth

- RF Envelope Ripple

- RF Envelope Rise Time

- RF Envelope Fall Time

- RF Pulse width

So, first of all we need to know the explicit definition of the terminology above, and the descriptions from RFID spec are listed below:

The RF Envelope parameter define the modulation quality of OOK from time domain which make sure each ON/OFF chip has similar shape, but one issue here is that since the R2D OOK signal is generated based on OFDM and when the data rate become higher, it’s going harder to approach ideal OOK. A simple simulation is shown in Figure I.

Figure I Time domain waveform for different data rate

Observation 3: For higher data rate, it will be harder to generate the waveform in time domain approaching the ideal OOK

It is reasonable to reuse similar approach from RFID but the requirements may need to be differentiated for different R2D data rate, but it is better to define different requirements for various data rate, which is similar to different EVM value for modulation orders.

Proposal 2: Define the modulation depth, RF envelope ripple, RF envelope rise time, RF envelope fall time

, RF pulse width as modulation quality requirement, but the requirement need to be differentiated for different R2D data rate.

- ACLR

During SI co-existence study, the ACLR assumption of AIoT BS is listed below:

| ACLR | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB) |
| --- | --- |

Since we already agreed that the RF requirement in R19 is defined with the understanding that the legacy NR UE only outdoor, there is no co-existence issue identified and reuse NB-IoT ACLR should be enough.

Proposal 3: Use the ACLR assumption in SI as the AIoT BS ACLR requirement, i.e., same as NB-IoT BS.

  - Rx Requirements

- REFSENS

In the last meeting, the LLS assumption for D2R SNR evaluation was agreed and the target D2R transmission bandwidth is 15kHz. For the OOK signal, the bandwidth is related to the data rate and the BLF (backscatter link frequency) needs to considered together. Under 15kHz D2R bandwidth, each side band will be 7.5kHz and the BLF should be 3.75kHz if repetition is not configured and the data rate is 3.75kbps. In the simulation assumption agreed in the last meeting, the repetition is not allowed which mean it is impossible to reach 1kbps.

Observation 4: It is impossible to make the 15kHz D2R total data rate close to a certain data rate without repetition or FEC.

Traditionally, the repetition is not considered in NR REFSENS evaluation since the repetition of NR signal mainly for better coverage and does not influence the spectrum, but the D2R use bit-level repetition which will change the spectrum of signal, so in D2R SNR evaluation, the repetition should be allowed.

Proposal 4:  No need to restrict the repetition and FEC in D2R SNR evaluation.

The Table I shows the configuration detail we used and the simulation results are shown in Figure II

Table I Simulation configuration for D2R with 1kbps data rate

| data size | Preamble | CRC | BLF (kHz) | FEC (1/3 code rate) | Manchester repetition | reference data rate | Total data rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 96 | 56 | 16 | 20 | 3 | 6 | 3.33 | 0.82 |
|  |  |  | 40 |  | 12 |  |  |
|  |  |  | 80 |  | 24 |  |  |
|  |  |  | 160 |  | 48 |  |  |
| 96 | 56 | 16 | 20 | None | 12 | 1.67 | 0.95 |
|  |  |  | 40 |  | 24 |  |  |
|  |  |  | 80 |  | 48 |  |  |
|  |  |  | 160 |  | 96 |  |  |

Figure II Simulation results for different BLF

The simulation results shows that the BLER curves are similar if the reference data rate is same. In addition, the cases without FEC have better performance and reason here is without FEC case have lower reference data rate which means the bandwidth will be smaller (around 7kHz, only half of 15kHz), so even all cases above reach around 1kbps total data rate, the SNR should base on the cases that bandwidth is close to 15kHz, i.e., 0 dB.

Observation 5: For different BLF, the SNR@10% BLER is same if the reference data rate keeps unchanged.

Proposal 5: Consider the 0 dB SNR as the starting point for 15kHz D2R REFSENS.

Another issue in REFSENS evaluation is the value of desens target which is the performance degradation due to the impact of CW. In R19, the CW is outside of topology which means the CW signal is non-correlated to the LO in D2R down-conversion, which make it is quite hard to be cancelled. In Figure III, we provide the simulation results for different BLF when phase noise exists.

Figure III impact of phase for 15kHz D2R (SFO is disabled)

The phase noise in TR38.803 Example 2 is used in the simulation and further scaled to 900MHz, and the results shows that the impact of phase noise is highly related to the BLF.

Observation 6: The impact of phase noise will be more significant when the BLF is smaller.

Based on the analysis above, the degradation due to CW need further coordinate with RAN1 and pending on the RAN1 conclusion of the small frequency shift.

Proposal 6: The impact of CW (e.g., phase noise) is related to the minimum small frequency shift which need to be pending on RAN1 progress.

- Conclusion

Observation 1: From interference perspective, the transmitter OFF power is not needed for the spectrum usage in R19 since the D2R and R2D are in the different spectrums.

Observation 2: Considering the HD-FDD operation, the transient period for R2D may be needed and the OFF power need to be specified accordingly.

Proposal 1: Reuse same OFF power and transient period as NR TDD BS for AIoT BS as starting point.

Observation 3: For higher data rate, it will be harder to generate the waveform in time domain approaching the ideal OOK

Proposal 2: Define the modulation depth, RF envelope ripple, RF envelope rise time, RF envelope fall time

, RF pulse width as modulation quality requirement, but the requirement need to be differentiated for different R2D data rate.

Proposal 3: Use the ACLR assumption in SI as the AIoT BS ACLR requirement, i.e., same as NB-IoT BS.

Observation 4: It is impossible to make the 15kHz D2R total data rate close to a certain data rate without repetition or FEC.

Proposal 4:  No need to restrict the repetition and FEC in D2R SNR evaluation.

Observation 5: For different BLF, the SNR@10% BLER is same if the reference data rate keeps unchanged.

Proposal 5: Consider the 0 dB SNR as the starting point for 15kHz D2R REFSENS.

Observation 6: The impact of phase noise will be more significant when the BLF is smaller.

Proposal 6: The impact of CW (e.g., phase noise) is related to the minimum small frequency shift which need to be pending on RAN1 progress.

- References

- R4-2502859 WF on requirements for A-IoT BS and CW