# 3GPP TSG-RAN WG4 Meeting #114	R4-2503820

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Further discussion on SRS antenna switching requirements

Agenda Item:	7.1.1.3.3

Document for:	Approval

Introduction

SRS antenna switching requirements are one of important aspects for 6Rx requirements. In the last meeting, we agreed to consider all available patterns for 3T6R SRS antenna switching and ΔTRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6. The WFs are captured in [1]. In this contribution, we will further discuss the remaining issues with the addition of t3r6.

Discussion

The WF [1] for SRS antenna switching and ΔTRxSRS of last meeting was captured as follows:

Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS Agreement: Consider all available patterns for 3T6R AS-SRS. Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Agreement: Use the values for ΔTRxSRS from the WF in R4-2420376 as shown in the table below. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t3r6 (dB); ΔTRxSRS t1r6-t3r6 (dB); ΔTRxSRS t2r6-t3r6 (dB); ΔTRxSRS t1r6-t2r6-t3r6 (dB). Band n41, n77, n78; 3.0; 4.5; 4.0; 5.5. Band n79; 4.5; 6.0; 5.5; 7.0. Band n104; 5.0; 7.0; 6.0; 8.0.

The SRS antenna switching requirements ΔTRxSRS are defined for all possible AS-ARS patterns for t3r6. For t1r6, t2r6, t1r6-t2r6, the numbers are already agreed in previous meeting as shown in the following table.

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4.0 | 3.5 | 4.5 |
| Band n79 | 5.5 | 5.0 | 6.0 |
| Band n104 | 6.5 | 5.5 | 7.0 |

Based on above agreements, we provide our draft CR to capture SRS antenna switching requirement in [3].

One issue is when ∆TRxSRS is applied for 6Rx including t1r6, t2r6, t3r6, the core spec didn’t support the indication for t3r6 for now. The part for t3r6 was in brackets in our draft CR.

We tried to give the wording for this:

h)	UE transmits SRS on the second, third, fourth, fifth, sixth SRS resources of the total 6 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r6', or

i)	UE transmits SRS from the SRS port pair on the second, third SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r6', or

[j) UE transmits SRS from the set of SRS ports on the second SRS resource in every configured SRS resource set consisting of three SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't3r6',]

The feature srs-AntennaSwitchingBeyond4RX-r17 leverages antenna switching techniques to support more than 4Rx antennas without a proportional increase in hardware complexity or cost. However, the UE capability srs-AntennaSwitchingBeyond4RX-r17, it can not support indication of t3r6 now, which is shown as following table. In order to complete the requirements for t3r6, we suggest to send an LS to RAN2 to capture t3r6 in the core spec.

srs-AntennaSwitchingBeyond4RX-r17 Indicates whether the UE supports SRS Antenna switching for more than 4 Rx. The capability signalling comprises the following parameters: - supportedSRS-TxPortSwitchBeyond4Rx-r17 indicates a combination of supported xTyRs. It includes 11-bit bitmap, where starting from the leading / leftmost bit (bit 0), each bit corresponds to {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}. For any indicated value, x shall be equal to or smaller than the one associated with the largest y. - entryNumberAffectBeyond4Rx-r17 indicates the lowest band entry number of the UL group (see entryNumberSwitchBeyond4Rx-r17) that impacts the DL of this band entry; - entryNumberSwitchBeyond4Rx-r17 indicates the lowest band entry of the UL group, which is defined as band entries with UL (see NOTE 1) that impact each other's UL (i.e. SRS TX port switching on any of the cells in the group will impact UL on all the cells in the group). This parameter is absent if an UL group contains only one band entry. The UE indicating support of this shall indicate support of srs-TxSwitch.

Observation 1: The UE capability srs-AntennaSwitchingBeyond4RX-r17 cannot support the indication of t3r6.

Proposal 1: Send LS to RAN2 to add t3r6 for the UE capability srs-AntennaSwitchingBeyond4RX-r17.

Conclusion

This contribution discusses ΔTRxSRS requirements for 6Rx considering SRS antenna switching configurations t3r6, t1r6-t3r6, t2r6-t3r6 and t1r6-t2r6-t3r6. The following proposals are made:

Observation 1: The UE capability srs-AntennaSwitchingBeyond4RX-r17 cannot support the indication of t3r6.

Proposal 1: Send LS to RAN2 to add t3r6 for the UE capability srs-AntennaSwitchingBeyond4RX-r17.

- References

[1] R4-2420376, WF on 6Rx requirements, AT&T, RAN4#113.

[2] R4-2503014, WF on 6Rx requirements, AT&T, RAN4#114.

[3] R4-2503821, Draft CR to 38.101-1 on SRS antenna switching requirements for 6Rx, vivo, RAN4#114bis.