# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503823

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the subband configurations and guardbands for gNB SBFD

Agenda Item:	7.23.1

Document for:	Approval

Introduction

In the last meeting, we further discussed system parameters for gNB SBFD. The WF for gNB SBFD was captured in [3]. In this contribution, we focus on the remaining issues for subband configurations and guard band issues for gNB SBFD.

Discussion

In the last meeting, we discussed FR2-1 CBW and SCS support for SBFD operation. There was one FFS to further study on 100MHz for DUD case to identify if any issue. The agreements can be found as follows:

Issue 1-1-1: FR2-1 CBW and SCS support for SBFD Agreement: - In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz. - RAN4 to further study on 100MHz for DUD case to identify if any issue.

From our perspective, there is no issue on 100MHz for DUD case for FR2-1. We used the exact 100MHz CBW for FR2-1 and DUD as the typical subband configuration in the feasibility study. There was no feasibility issue for this case. Therefore, we suggest to support 100MHz DUD case for FR2-1.

Proposal 1: BS channel bandwidth 100MHz for DUD case can be supported for FR2-1 gNB SBFD.

##### Issue 1-1-4: Declaration and flexibility for SBFD operation ##### Agreement: - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS

For clarification on declaration and flexibility, we introduced very strict principles for SBFD configurations. We suggest it only applied to FR1 WA BS, not for FR1 MR/LA BS and FR BS for SBFD operation. For FR1 WA BS, we see the difficulty to implement SBFD from the feasibility study, since it requires large RSIC to reduce the self-interference within the BS. There are many different kinds of isolation techniques used for RSIC to guaranteed the feasibility of FR1 WA SBFD operation. Restricting the subband configurations for FR1 WA BS may reduce the design difficulty for such SBFD capable BS product. However, for other types of BS, it is less challenging to support SBFD. Restricting SBFD configuration will not be too necessary.

Proposal 2: Only apply the restriction of SBFD configuration to FR1 WA BS, not for FR1 MR/LA and FR2-1 SBFD capable BS product.

- RAN4#113 Agreements Issue 1-3-6: Other restriction on DL subband size Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size). - FR2 minimum CBW to support SBFD is 100MHz. - RAN4#114 Agreements Issue 1-2-4: Other restriction on DL subband size - No new agreement achieved.

For other restriction on DL subband size, we understand the intention is to gurantee DL subband can cover at least one SSB. This comes from the sync raster design. However, we think this restriction is unnecessary. From our perspective, we think the channel bandwidth can cover at least one SSB is enough, we don’t need further restriction DL subband size to cover at least one SSB.

Proposal 3: It is not necessary to introduce the following restriction on DL subband size:

o	For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

o	FR2 minimum CBW to support SBFD is 100MHz.

Conclusion

This contribution discusses SBFD configurations and guardbands for gNB SBFD. The following proposals are made:

Proposal 1: BS channel bandwidth 100MHz for DUD case can be supported for FR2-1 gNB SBFD.

Proposal 2: Only apply the restriction of SBFD configuration to FR1 WA BS, not for FR1 MR/LA and FR2-1 SBFD capable BS product.

Proposal 3: It is not necessary to introduce the following restriction on DL subband size:

o	For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

o	FR2 minimum CBW to support SBFD is 100MHz.

- References

[1] R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung, RAN4#112.

[2] R4-2419907, Way Forward for [113][306] NR_duplex_evo_General, Samsung, RAN4#113.

[3] R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung, RAN4#114.