# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503824

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	vivo

Title: 	Discussion on simultaneous operation between GNSS and UL transmission in NR NTN

Agenda Item:	7.29.1

Document for:	Approval

Introduction

An LS [1] was sent to RAN2 on simultaneous operation between GNSS and NR NTN, asking to introduce improvements to existing IDC solutions to address this in-device coexistence issue. A received LS [2] was received from RAN2, requiring more information on requirements, e.g. the periodicity and duration of IDC free time to extend IDC TDM solution. In this contribution, we provide our view on the periodicity and duration of IDC free time.

Discussion

The content of the response LS [2] from RAN2 was captured as follows:

1. Overall Description: RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18. RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN. 2. Actions: To RAN4: ACTION: RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

IDC TDM solution is a mechanism defined in 3GPP standards to address in device interference between multiple radio technologies. In this discussion, it mainly focuses on the simultaneous operation between GNSS and UL transmission in NR NTN. In the LS, RAN2 asks more information on the periodicity and duration of time for UE read GNSS in NTN.

For GNSS pattern, it serves different purposes, e.g. power efficiency, interference mitigation, resource optimization, enhancing positioning performance. For some typical scenarios, they can be different as shown in the following table:

| Application Scenario | Period | Duration | Duty Cycle |
| --- | --- | --- | --- |
| High-Accuracy (e.g., Navigation) | 1 second | Continuous operation (no sleep) | 100% |
| Power-Sensitive Applications (e.g., IoT Devices) | 60 seconds | 5 seconds | ~8.3% |
| Balanced Power and Accuracy Scenarios (e.g., Vehicle Tracking) | 10 seconds | 2 seconds | 20% |

Besides above GNSS typical patterns, RAN2 may also consider the values defined in 3GPP. GNSS-PositionFixDuration-r18 is a feature introduced in 3GPP LTE Rel-18 to dynamically configure the duration of GNSS positioning fixes. It was introduced to:

- Support diverse application scenarios.

- Optimize device power consumption.

- Improve network resource utilization.

- Enhance positioning technology in Rel-18.

The duration of GNSS positioning fixes based on the configured value of GNSS-PositionFixDuration-r18 can be 1s~31s. short positioning duration is suitable for fast positioning, medium for general positioning needs and long for scenarios requiring high accuracy or continuous tracking. We think these GNSS duration values can still be used when consider IDC TDM solution.

#### GNSS-PositionFixDuration (From 36.331) The IE GNSS-PositionFixDuration indicates the time duration required for the UE to acquire a GNSS position. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. GNSS-PositionFixDuration information element -- ASN1START GNSS-PositionFixDuration-r18 ::= ENUMERATED{ s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31} -- ASN1STOP

Proposal 1: RAN2 can still follow 3GPP defined values for GNSS-PositionFixDuration in TS 36.331 for further enhancement of IDC TDM solution:

GNSS-PositionFixDuration-r18 ::= ENUMERATED{

s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31}

Conclusion

This contribution discusses GNSS pattern information to RAN2 for further enhancements for IDC TDD solution, allowing simultaneous operation between GNSS and NR NTN. The following proposal is made:

Proposal 1: RAN2 can still follow 3GPP defined values for GNSS-PositionFixDuration in TS 36.331 for further enhancement of IDC TDM solution:

GNSS-PositionFixDuration-r18 ::= ENUMERATED{

s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31}

- References

[1] R4-2419898, LS on simultaneous operation between GNSS and NR NTN, RAN4#113.

[2] R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN, RAN2#129.