3GPP TSG-RAN4 Meeting #114bis	R4-2503832

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	vivo

Title:	TP for TR 38.774 on LP-WUS UE RF considerations

Agenda item:			7.26.2.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.774 to add considerations and agreements for LP-WUS RF aspects.

2. Reference

[1] TR 38.774 v0.2.0

# 3. Text Proposal

<Start of Text Proposal>

7.1.2	Rx SNR evaluations

To derive SNR performance of LP-WUS, it was agreed to select OOK-4 M=4 under AWGN channel model with 1% MDR without repetition as worst case. Based on the simulation results, the SNR value for OOK-4 M=4 is considered as -6.5dB.

7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas.

For FR1, RAN4 agree two sets of IM+NF value:

	Set 1: 18dB

	Set 2: 13.5dB

•	assume IM is 2.5dB

7.1.4	REFSENS requirements

7.1.5	ASCS requirements

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent in-band NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR. The in-band LP-WUS and NR signal should be same PSD, and the NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs.

Based on the simulation results, the required number of guard RB is 0, and the impact of de-sense of LP-WUS SNR is negligible. To consider the implementation margin, the ASCS requirements can be specified as following:

The UE shall fulfil the minimum requirements specified in Table 7.1.5-1 for LP-WUR. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS shall be ≤ 1 % with the reference measurement channels as specified in Annexes TBD.

Table 7.1.5-1: ASCS parameters for LP-WUR

| RX parameter | Units | LP-WUS bandwidth (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth configuration | dBm | REFSENS+[0.5]dB |
| Pinterferer,NR | dBm | Same PSD as LP-WUS |
| Guard RB on each side of LP-WUS signal |  | 0 |
| NOTE: The NR signal occupies the remaining RB resources with the same PSD of LP-WUS within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs. |  |  |

<End of Text Proposal>